MEEHAN1.DOC

PLEASE NOTE THAT THE FOLLOWING TEXT IS ALWAYS BEING UPDATED SO

PLEASE READ THIS DOCUMENT FILE FULLY. SORRY FOR THE ERRRATIC

NATURE OF THIS FILE BUT HOPEFULLY WHEN I HAVE FINISHED THE

DEVELOPMENT OF THE PROGRAMS DESCRIBED BELOW A MORE COHERENT

DOCUMENT FILE WILL APPEAR.

GOOD LUCK!

--------------------------------------------------------------

DISCCOPY and ASCIICPY, how to use them.

---------------------------------------------------------------

Disccopy and Asciicpy are disk copying programs that will copy

the source disk to a blank target disk ie format as well as

copy. Disccopy will copy all files including the system tracks

that boot CP/M. Disccopy copies all files after the target disk

has been formatted first. Asciicpy will only copy ascii text as

the data from the source disk is sent out during format. As

.COM files can contain values from 00h to FFh Asciicpy cannot

copy these files reliably. This is because during format values

F5h to feh are reserved values used by the 2797 fdc chip ie

F7h generates two CRC bytes. As you can see if one byte in your

.COM file causes wrong bytes to be written and more bytes than

contained in the source file to be generated then serious

errors will occur. The reason for using Asciicpy is if you have

a disk containing large text files and you wish to make a back

up on a fresh disk then it will be quicker to use Asciicpy than

Disccopy. As Disccopy formats first then copies sector by

sector it takes longer, around 7 -10 mins. It is actually

faster to use format,pip and genldr to copy disks (depends on

how many files you have to copy). But if you want to copy a

disk while you are getting on with something else then Disccopy

comes in handy.

( P Meehan 21/1/89)

(Disccopy is now slightly faster and should  only take between

four and four and a half minutes.)

( P.Meehan 7/1/90).

The error checking carried out by these programs is

very strict. Every track,sector write and sector read is

checked. This means your disk drives and FDC1 card have to be

spot on as far as alignment goes. If you experience a lot of

problems using these programs you can try altering the source

code so that the error checking is not so intensive. My

experience whilst developing these programs is that if you

consistently encounter errors say for CRC, then by altering the

stat2 macro in the files disk.lib/cpy.lib, by placing a ';'

delimiter infront of the 'jp nz' instruction after the bit test

instruction for crc errors, your source files will be copied

across ok despite the errors. Do not be to enthusiastic in

taking out the error checking. Remember that asciicpy cannot

copy the system tracks properly so make sure your source disk

is not a bootable disk.

To use either program type in their respective names

and you will be prompted for source and target drives from a

window in the top left hand corner of the screen. If an error

does occur a message will appear before program termination.

With no errors the program will clear the screen before

termination. After the target drive letter has been entered the

copying/formatting will begin. Drives A - D are recognised by

both programs. These programs will only work on Interak

computers using the VDU-2K, FDC-1 card and CP/M+ operating

system.

----------------------------------------------------------------

......PLEASE NOTE......

----------------------------------------------------------------

****I will repeat again ASCIICPY can only copy ascii text***

****any files produced by wordprocessors such as Wordstar***

****will probably not copy correctly terminating ASCIICPY***

****before all files have been copied. This is because...***

****in Document mode most wordprocessors use hex codes...***

****above 80h for control characters/codes for things....***

****such as wordwrap, text justification, printer........***

****controls etc. Of course any files produced in........***

****Non-Document mode will be ok to copy. Also make sure ***

****the disk you are copying from has not had any .COM or***

****Document files on it even erased files. This is......***

****because all erasing does is to remove the directory  ***

****entry for a particular file, the data might still be ***

****within disk sectors. If you are in doubt use Disccopy***

****Both programs expect both source and target diskettes***

****not to be write protected. This is because of .......***

****my laziness. As I have outstanding hardware projects ***

****for my Interak and it has taken around 3 months to ..***

****develop these programs I did not wish to spend any ..***

****more time on them than necessary If you don't like ..***

****the idea take out the test for write protect or .....***

****copy stat2 macro as stat3 macro in both .LIB files...***

****Then for all read operations have them call stat3 ...***

****with the write protect test taken out. Also for the .***

****source drive take out stat1 macro as this is all it .***

****tests for............................................***

****There is also another flaw in these programs in that ***

****if you select a drive that does not exist then the ..***

****prog. will lock up. This happens because  the .......***

****2797 FDC actually works different to how I thought it***

****did. Consequently my program tests for the ready bit ***

****in the status register after a type 2 or 3 command is***

****given to make sure the disk drive is upto correct ...***

****operational speed. Thus if you select a non existent ***

****drive the program just sits there waiting for the ...***

****drive to get up to speed. In this situation a reboot  ***

****is needed unfortunately. Other than that the programs***

****seem to be reliable enough as long as the above is ..***

****remembered. A full 772KB of text  has  been .........***

****copied repeatedly during a 6 hour soak testing with .***

****only 2 minor failures. Disccopy in the same time had ***

****one failure which turned out to be a corrupted source***

****disk, after formatting this disk was copied to ok....***

______________________________________________________________

ADDENDUM

______________________________________________________________

THE ABOVE PROGRAMS HAVE BEEN RENAMED AS ACOPY FOR ASCIICPY AND

DCOPY FOR DISCCOPY. THE REASON FOR THIS IS THAT I HAVE ALTERED

ASCIICPY AND DISCCOPY SO THAT THEY RUN FROM A FRONT END MENU.

THE PROGRAMS TAKE ADVANTAGE OF CP/M+'s CHAIN PROGRAM BDOS

FUNCTION. THE FRONT END MENU IS CALLED TOOLS AND THE WHOLE

PACKAGE IS CALLED INTER-TOOLS. THESE UTILITIES HAVE BEEN

WRITTEN SO THAT WHEN A PROGRAM LIKE DISCCOPY FINISHES WITH NO

ERRORS IT RELOADS THE FRONT END MENU.THERE ARE OTHER UTILITIES

OTHER THAN ASCIICPY AND DISCCOPY,ie UNERASE, THAT WILL ALSO BE

RUN FROM THE MENU BUT AS OF YET THEY HAVE NOT BEEN WRITTEN.

ACOPY AND DCOPY ARE NON MENU VERSIONS OF ASCIICPY AND DISCCOPY.

MOST OF THE KNOWN BUGS THAT EXISTED IN THE EARLY

VERSIONS HAVE BEEN SORTED OUT. THE PROGRAMS NO LONGER LOCK UP

IF YOU CHOOSE A DRIVE THAT DOES NOT EXIST. REMEMBER THE NEW

VERSIONS THAT ARE NOT RUN FROM THE FRONT END MENU ARE CALLED

ACOPY FORMERLY ASCIICPY AND DCOPY FORMERLY DISCCOPY. THE MENU

DRIVEN PROGRAMS RETAIN THE ORIGINAL NAMES.

TOOLS THE FRONT END MENU NEEDS TO BE RUN FROM A

BOOTABLE DISK AS IT LOOKS FOR CCP.COM IN THE SYSTEM TRACKS.

TOOLS HAS ONLINE HELP WINDOW FACILITY DESCRIBING EACH PROGRAM

AND GENERAL USE. ONE MINOR FAILING WITH TOOLS IS THAT IT NEEDS

TO BE ON THE SOURCE DISK BECAUSE WHEN A UTILITY PROGRAM

FINISHES IT RELOADS TOOLS.COM. IF IT CANNOT FIND TOOLS THE

'TOOLS.COM?' ERROR MESSAGE WILL BE DISPLAYED. THUS THE WHOLE

UTILITY PACKAGE, WHICH IS CALLED 'INTER-TOOLS' RUNS BETTER

FROM A 3 DISK SYSTEM.

(The above statement about running from a 3 drive system is no

longer 100% true as the programs now give you a prompt to

reinsert the source disk containing Tools.com, thus making it

easier to use from a 2 drive system P.M. 5/9/89)

I believe that Asciicpy and Disccopy (also Acopy, Dcopy) now

run correctly without the need for both disks to be not write

protected and the programs no longer lock up if you select a

drive that does not exist.

P Meehan 2/8/89)

--------------------------------------------------------------

INTER TOOLS

--------------------------------------------------------------

Inter-tools is best run from a bootable disk that also has

Submit.Com copied onto it. Create a text file called

Profile.Sub that contains the word 'TOOLS'. Make sure you also

have the following on your bootable disc, Tools.Com,

Asciicpy.Com, Disccopy.Com, Unerase.Com. Once the above has

been completed Inter-Tools will be immediately run upon

boot up from this disk; if you prefer you can run Inter-Tools

by just typing 'Tools'. At present 3 of the 5 applications have

been written ie Asciicpy, Disccopy, Unerase. The 2 missing

programs are 'Verify' a disc sector test program that tests

your discs surface for defects and 'Disk-Bug' a disc sector

editor that will allow you to edit any data on your discs.

If you select any of these missing packages from the Inter-

Tools menu you will be returned to CP/M with an error similar

to this 'VERIFY.COM?'.

(P.Meehan.   7/1/90).

----------------------------------------------------------------

PLEASE NOTE:

-----------------------------------------------------------------

If you are using Zilogs new Z80280 cpu then do not

create a Profile.Sub file on your boot disc as the inialisation

carried out by DMON boot/monitor eprom may not be 100% correct.

This is due to the fact that the onboard cache memory is not

enabled and thus the Z80280 runs slower than a 4MHz Z80A. As

the version of CP/M+ that we use relies on a program loop to

decide when to read and write disks a small speed up program

needs to be run first before you can write safely to your

discs. Early versions of the Z80280 have a faulty cache,

weather this is put to right or not depends on Zilog. Basically

what happens that CP/M opens a temporary file when processing

'submit' files, as the Z80280 is not fast enough for the floppy

disc controller chip (WD2797) your discs directory gets

corrupted.

(P.Meehan.   1/12/90)

_______________________________________________________________

LATEST REPORT ON INTER-TOOLS.

_______________________________________________________________

The menu driven Unerase program has now been finished along

with improvements in the front end menu program called Tools.

Unerase programs the 2797 FDC chip directly, therefore Unerase

can only be used on standard 3.5" double sided double density

discs or other size discs that conform to the standard Interak

format as defined by Greenbank Electronics.

Novice users please note that if you know very little

about CP/M's disc directory file control block structure and or

anything about logical and physical extents then be careful how

you use Unerase. In time I will produce a document file

describing such matters but you should be warned that Unerase

is potentially lethal to your discs data if misused. If you

have accidentally erased a much needed file make a backup of

your disc using either Dcopy or Inter-tools' Disccopy program.

Although your file is erased Dcopy and Disccopy will still

backup the file onto another disc.

Some general help now follows:

If the file you need is 64KB or below in size

and there are two directory entries shown by Unerase then only

unerase one of them.

If on the other hand your file was larger than

64KB unerase all of the displayed directory entries for that

file.

example:- if  your file was 135KB in size there will be

three entries for that file. The number of entries depends on

how many times 64KB will divide into your files size, any

remainder will also have a directory entry as in the above

example (135/64=2.1093 this rounds up to 3 entries shown by

Unerase).

If you do not know the file size it will be left to

trial and error. Always use a backup disc for any

experimentation. Another tip is that if you have just erased a

file that you wanted to keep then use Unerase immediately and

do not copy any other files to this disc. This precaution stops

CP/M from allocating blocks (groups of sectors) of the erased

file to the newly copied file(s) thus losing part of the

desired file if not all of it.

( P Meehan 6/1/90)

----------------------------------------------------------------

UPDATE ON UNERASE.

----------------------------------------------------------------

I have found a bug in the Unerase program though nothing too

serious. What has happened is that I thought that the version

of CP/M+ that we use only reserved 1 block (4KB) for the

directory but actually reserves 2 blocks (8KB). What this means

is that  Unerase would only read half of the directory so would

only be able to unerase 128 files, 4KB/32=128. This is not as

serious as it sounds as I have many disks that are near full

but have only 50 files on them well under the 128 that can be

read. Also the maximum number of files that can be saved on

772KB discs is 193. This is because 193*4KB=772KB thus the full

256 directory entries cannot be used due to lack of disk space

(8KB/32=256). Nevertheless I have removed this bug as it is

possible to use part of the directory that was not accessible

to Unerase if a large number of files are used (193 maximum).

If you do not understand any of the above do not worry just

take it from me that Unerase now works correctly, unless you

know differently!

A second bug is likely to be unnoticed for some time. What

happens is when pressing carriage return to select a particular

file for unerasing the file is unerased and you are returned

to Inter-Tools' front end menu without giving you the option

of unerasing other files. This bug was highly erratic and

seldom occurred. Anyway the bug has now been corrected, let us

hope that is the last.

(P Meehan 23/5/90)

_______________________________________________________________

LATEST ADDITION TO INTER-TOOLS :- VERIFY.

_______________________________________________________________

Verify has now been added to the Inter-Tools disk utility suite

making four out of the five programs available for use.

Verify will test standard 3.5" Interak disks (772KB) or

any other size disk conforming to the Greenbank Electronics

format (please note that these programs do not support the new

high density disks (1.66 MB) that Greenbank are now starting

to use), for defective sectors. The tests performed are quite

stringent and will report such items as CRC errors wrong track

number etc. Verify will actually test a blank (unformatted)

disk all the way through to track 79 reporting the errors along

the way.

Note if the above test is performed it takes the 2797

an  extremely long time to complete the testing of an

unformatted disk. This program like the rest of the Inter-Tools

suite will only work on Interak's using the VDU-2K, FDC-1, and

CP/M+.

Once Verify has been started there is no way to end it

prematurely other than a reboot. Due to the limited amount of

time I can spend on these programs at any one moment this will

appear in a future update. Hopefully you should find most of

the source code released into the library thus allowing you to

add enhancements if you so wish. The code relies heavily on

macros and thus is usually longer than is necessary but is

easier to maintain.

The program reports the errors in a general way ie

it will not state on what track and sector the error occurred.

This information is fairly easy to obtain from the program

if it is studied during operation. When testing a sector a

high light bar indicates which sector is being read. In normal

operation this bar should whizz across the screen, if a sector

is troublesome then this bar will pause for a certain amount

of time over the sector  read indicator. The track being

read is shown in the top right hand corner. If a track ID error

occurrs ie expected track number does not equal track number

read from disk a window will pop up telling you so and will

wait for a key press before continuing.

If your disk has errors reported by this program then

format a new disk and use pip to copy what you can across to

the new disk.

The program expects the disk to be not write protected

the reason for this is that this is the best way that the 2797

can detect if a 3.5" disk is in the drive or not. Do not worry

no writing takes place within Verify. Verify is meant to tell

you if any soft errors are present on your disks but is not

intended to locate those errors as there is virtually nothing

you can do to repair them.

( P Meehan 17/3/90)

I hope that these programs will be of use to you. If you find any

please contact me.

P Meehan,

15 Pennine Road,

Chelmsford,

Essex,

CM1 2HG.

##################################################