8-BIT LOAD GROUP

LD A,A	7F	A=A

LD A,B	78	A=B

LD A,C	79	A=C

LD A,D	7A	A=D

LD A,E	7B	A=E

LD A,H	7C	A=H

LD A,L	7D	A=L

LD B,A	47	B=A

LD B,B	40	B=B

LD B,C	41	B=C

LD B,D	42	B=D

LD B,E	43	B=E

LD B,H	44	B=H

LD B,L	45	B=L

LD C,A	4F	C=A

LD C,B	48	C=B

LD C,C	49	C=C

LD C,D	4A	C=D

LD C,E	4B	C=E

LD C,H	4C	C=H

LD C,L	4D	C=L

LD D,A	57	D=A

LD D,B	50	D=B

LD D,C	51	D=C

LD D,D	52	D=D

LD D,E	53	D=E

LD D,H	54	D=H

LD D,L	55	D=L

LD E,A	5F	E=A

LD E,B	58	E=B

LD E,C	59	E=C

LD E,D	5A	E=D

LD E,E	5B	E=E

LD E,H	5C	E=H

LD E,L	5D	E=L

LD H,A	67	H=A

LD H,B	60	H=B

LD H,C	61	H=C

LD H,D	62	H=D

LD H,E	63	H=E

LD H,H	64	H=H

LD H,L	65	H=L

LD L,A	6F	L=A

LD L,B	68	L=B

LD L,C	69	L=C

LD L,D	6A	L=D

LD L,E	6B	L=E

LD L,H	6C	L=H

LD L,L	6D	L=L

LD A,n	3E n	A=n

LD B,n	06 n	B=n

LD C,n	0E n	C=n

LD D,n	16 n	D=n

LD E,n	1E n	E=n

LD H,n	26 n	H=n

LD L,n	2E n	L=n

LD A,(HL)	7E	A=(HL)

LD B,(HL)	46	B=(HL)

LD C,(HL)	4E	C=(HL)

LD D,(HL)	56	D=(HL)

LD E,(HL)	5E	E=(HL)

LD H,(HL)	66	H=(HL)

LD L,(HL)	6E	L=(HL)

LD A,(IX+d)	DD 7E d	A=(IX+d)

LD B,(IX+d)	DD 46 d	B=(IX+d)

LD C,(IX+d)	DD 4E d	C=(IX+d)

LD D,(IX+d)	DD 56 d	D=(IX+d)

LD E,(IX+d)	DD 5E d	E=(IX+d)

LD H,(IX+d)	DD 66 d	H=(IX+d)

LD L,(IX+d)	DD 6E d	L=(IX+d)

LD A,(IY+d)	FD 7E d	A=(IY+d)

LD B,(IY+d)	FD 46 d	B=(IY+d)

LD C,(IY+d)	FD 4E d	C=(IY+d)

LD D,(IY+d)	FD 56 d	D=(IY+d)

LD E,(IY+d)	FD 5E d	E=(IY+d)

LD H,(IY+d)	FD 66 d	H=(IY+d)

LD L,(IY+d)	FD 6E d	L=(IY+d)

LD (HL),A	77	(HL)=A

LD (HL),B	70	(HL)=B

LD (HL),C	71	(HL)=C

LD (HL),D	72	(HL)=D

LD (HL),E	73	(HL)=E

LD (HL),H	74	(HL)=H

LD (HL),L	75	(HL)=L

LD (IX+d),A	DD 77 d	(IX+d)=A

LD (IX+d),B	DD 70 d	(IX+d)=B

LD (IX+d),C	DD 71 d	(IX+d)=C

LD (IX+d),D	DD 72 d	(IX+d)=D

LD (IX+d),E	DD 73 d	(IX+d)=E

LD (IX+d),H	DD 74 d	(IX+d)=H

LD (IX+d),L	DD 75 d	(IX+d)=L

LD (IY+d),A	FD 77 d	(IY+d)=A

LD (IY+d),B	FD 70 d	(IY+d)=B

LD (IY+d),C	FD 71 d	(IY+d)=C

LD (IY+d),D	FD 72 d	(IY+d)=D

LD (IY+d),E	FD 73 d	(IY+d)=E

LD (IY+d),H	FD 74 d	(IY+d)=H

LD (IY+d),L	FD 75 d	(IY+d)=L

LD (HL),n	36 n	(HL)=n

LD (IX+d),n	DD 36 d n	(IX+d)=n

LD (IY+d),n	FD 36 d n	(IY+d)=n

LD A,(BC)	0A	A=(BC)

LD A,(DE)	1A	A=(DE)

LD A,(nn)	3A n n	A=(nn)

LD (BC),A	02	(BC)=A

LD (DE),A	12	(DE)=A

LD (nn),A	32 n n	(nn)=A

LD A,I	ED 57	A=I

LD A,R	ED 5F	A=R

LD I,A	ED 47	I=A

LD R,A	ED 4F	R=A

16-BIT LOAD GROUP

LD BC,nn	01 n n	BC=nn

LD DE,nn	11 n n	DE=nn

LD HL,nn	21 n n	HL=nn

LD SP,nn	31 n n	SP=nn

LD IX,nn	DD 21 n n	IX=nn

LD IY,nn	FD 21 n n	IY=nn

LD HL,(nn)	2A n n	L=(nn); H=(nn+1)

LD BC,(nn)	ED 4B n n	C=(nn); B=(nn+1)

LD DE,(nn)	ED 5B n n	E=(nn); D=(nn+1)

LD HL,(nn)	ED 6B n n	L=(nn); H=(nn+1)

LD SP,(nn)	ED 7B n n	SPL=(nn); SPH=(nn+1)

LD IX,(nn)	DD 2A n n	IXL=(nn); IXH=(nn+1)

LD IY,(nn)	FD 2A n n	IYL=(nn); IYH=(nn+1)

LD (nn),HL	22 n n	(nn)=L; (nn+1)=H

LD (nn),BC	ED 43 n n	(nn)=C; (nn+1)=B

LD (nn),DE	ED 53 n n	(nn)=E; (nn+1)=D

LD (nn),HL	ED 63 n n	(nn)=L; (nn+1)=H

LD (nn),SP	ED 73 n n	(nn)=SPL; (nn+1)=SPH

LD (nn),IX	DD 22 n n	(nn)=IXL; (nn+1)=IXH

LD (nn),IY	FD 22 n n	(nn)=IYL; (nn+1)=IYH

LD SP,HL	F9	SP=HL

LD SP,IX	DD F9	SP=IX

LD SP,IY	FD F9	SP=IY

PUSH BC	C5	(SP-1)=B; (SP-2)=C; SP=SP-2

PUSH DE	D5	(SP-1)=D; (SP-2)=E; SP=SP-2

PUSH HL	E5	(SP-1)=H; (SP-2)=L; SP=SP-2

PUSH AF	F5	(SP-1)=A; (SP-2)=F; SP=SP-2

PUSH IX	DD E5	(SP-1)=IXH; (SP-2)=IXL; SP=SP-2

PUSH IY	FD E5	(SP-1)=IYH; (SP-2)=IYL; SP=SP-2

POP BC	C1	C=(SP); B=(SP+1); SP=SP+2

POP DE	D1	E=(SP); D=(SP+1); SP=SP+2

POP HL	E1	L=(SP); H=(SP+1); SP=SP+2

POP AF	F1	F=(SP); A=(SP+1); SP=SP+2

POP IX	DD E1	IXL=(SP); IYH=(SP+1); SP=SP+2

POP IY	FD E1	IYL=(SP); IYH=(SP+1); SP=SP+2

EXCHANGE, BLOCK TRANSFER AND BLOCK SEARCH GROUPS

EX DE,HL	EB	DE<>HL

EX AF,AF'	08	AF<>AF'

EXX	D9	BC<>BC'; DE<>DE'; HL<>HL'

EX (SP),HL	E3	L<>(SP); H<>(SP+1)

EX (SP),IX	DD E3	IXL<>(SP); IXH<>(SP+1)

EX (SP),IY	FD E3	IYL<>(SP); IYH<>(SP+1)

LDI	ED A0	(DE)=(HL); DE=DE+1; HL=HL+1; BC=BC-1

LDIR	ED B0	(DE)=(HL); DE=DE+1; HL=HL+1; BC=BC-1; Repeat until BC=0

LDD	ED A8	(DE)=(HL); DE=DE-1; HL=HL-1; BC=BC-1

LDDR	ED B8	(DE)=(HL); DE=DE-1; HL=HL-1; BC=BC-1; Repeat until BC=0

CPI	ED A1	A-(HL); HL=HL+1; BC=BC-1

CPIR	ED B1	A-(HL); HL=HL+1; BC=BC-1; Repeat until A=(HL) or BC=0

CPD	ED A9	A-(HL); HL=HL-1; BC=BC-1

CPDR	ED B9	A-(HL); HL=HL-1; BC=BC-1; Repeat until A=(HL) or BC=0

8-BIT ARITHMETIC AND LOGICAL GROUP

ADD A,A	87	A=A+A

ADD A,B	80	A=A+B

ADD A,C	81	A=A+C

ADD A,D	82	A=A+D

ADD A,E	83	A=A+E

ADD A,H	84	A=A+H

ADD A,L	85	A=A+L

ADD A,n	C6 n	A=A+n

ADD A,(HL)	86	A=A+(HL)

ADD A,(IX+d)	DD 86 d	A=A+(IX+d)

ADD A,(IY+d)	FD 86 d	A=A+(IY+d)

ADC A,A	8F	A=A+A+CY

ADC A,B	88	A=A+B+CY

ADC A,C	89	A=A+C+CY

ADC A,D	8A	A=A+D+CY

ADC A,E	8B	A=A+E+CY

ADC A,H	8C	A=A+H+CY

ADC A,L	8D	A=A+L+CY

ADC A,n	CE n	A=A+n+CY

ADC A,(HL)	8E	A=A+(HL)+CY

ADC A,(IX+d)	DD 8E d	A=A+(IX+d)+CY

ADC A,(IY+d)	FD 8E d	A=A+(IY+d)+CY

SUB A	97	A=A-A

SUB B	90	A=A-B

SUB C	91	A=A-C

SUB D	92	A=A-D

SUB E	93	A=A-E

SUB H	94	A=A-H

SUB L	95	A=A-L

SUB n	D6 n	A=A-n

SUB (HL)	96	A=A-(HL)

SUB (IX+d)	DD 96 d	A=A-(IX+d)

SUB (IY+d)	FD 96 d	A=A-(IY+d)

SBC A,A	9F	A=A-A-CY

SBC A,B	98	A=A-B-CY

SBC A,C	99	A=A-C-CY

SBC A,D	9A	A=A-D-CY

SBC A,E	9B	A=A-E-CY

SBC A,H	9C	A=A-H-CY

SBC A,L	9D	A=A-L-CY

SBC A,n	DE n	A=A-n-CY

SBC A,(HL)	9E	A=A-(HL)-CY

SBC A,(IX+d)	DD 9E d	A=A-(IX+d)-CY

SBC A,(IY+d)	FD 9E d	A=A-(IY+d)-CY

AND A	A7	A=A AND A

AND B	A0	A=A AND B

AND C	A1	A=A AND C

AND D	A2	A=A AND D

AND E	A3	A=A AND E

AND H	A4	A=A AND H

AND L	A5	A=A AND L

AND n	E6 n	A=A AND n

AND (HL)	A6	A=A AND (HL)

AND (IX+d)	DD A6 d	A=A AND (IX+d)

AND (IY+d)	FD A6 d	A=A AND (IY+d)

OR A	B7	A=A OR A

OR B	B0	A=A OR B

OR C	B1	A=A OR C

OR D	B2	A=A OR D

OR E	B3	A=A OR E

OR H	B4	A=A OR H

OR L	B5	A=A OR L

OR n	F6 n	A=A OR n

OR (HL)	B6	A=A OR (HL)

OR (IX+d)	DD B6 d	A=A OR (IX+d)

OR (IY+d)	FD B6 d	A=A OR (IY+d)

XOR A	AF	A=A XOR A

XOR B	A8	A=A XOR B

XOR C	A9	A=A XOR C

XOR D	AA	A=A XOR D

XOR E	AB	A=A XOR E

XOR H	AC	A=A XOR H

XOR L	AD	A=A XOR L

XOR n	EE n	A=A XOR n

XOR (HL)	AE	A=A XOR (HL)

XOR (IX+d)	DD AE d	A=A XOR (IX+d)

XOR (IY+d)	FD AE d	A=A XOR (IY+d)

CP A	BF	A-A

CP B	B8	A-B

CP C	B9	A-C

CP D	BA	A-D

CP E	BB	A-E

CP H	BC	A-H

CP L	BD	A-L

CP n	FE n	A-n

CP (HL)	BE	A-(HL)

CP (IX+d)	DD BE d	A-(IX+d)

CP (IY+d)	FD BE d	A-(IY+d)

INC A	3C	A=A+1

INC B	04	B=B+1

INC C	0C	C=C+1

INC D	14	D=D+1

INC E	1C	E=E+1

INC H	24	H=H+1

INC L	2C	L=L+1

INC (HL)	34	(HL)=(HL)+1

INC (IX+d)	DD 34 d	(IX+d)=(IX+d)+1

INC (IY+d)	FD 34 d	(IY+d)=(IY+d)+1

DEC A	3D	A=A-1

DEC B	05	B=B-1

DEC C	0D	C=C-1

DEC D	15	D=D-1

DEC E	1D	E=E-1

DEC H	25	H=H-1

DEC L	2D	L=L-1

DEC (HL)	35	(HL)=(HL)-1

DEC (IX+d)	DD 35 d	(IX+d)=(IX+d)-1

DEC (IY+d)	FD 35 d	(IY+d)=(IY+d)-1

GENERAL PURPOSE ARITHMETIC AND CPU CONTROL GROUPS

DAA	27	Decimal adjust accumulator

CPL	2F	A=NOT A

NEG	ED 44	A=0-A

CCF	3F	CY=NOT CY

SCF	37	CY=1

NOP	00	No operation

HALT	76	CPU Halted

DI	F3	Disable maskable interrupts

EI	FB	Enable maskable interrupts

IM 0	ED 46	Set interrupt mode 0

IM 1	ED 56	Set interrupt mode 1

IM 2	ED 5E	Set interrupt mode 2

16-BIT ARITHMETIC GROUP

ADD HL,BC	09	HL=HL+BC

ADD HL,DE	19	HL=HL+DE

ADD HL,HL	29	HL=HL+HL

ADD HL,SP	39	HL=HL+SP

ADC HL,BC	ED 4A	HL=HL+BC+CY

ADC HL,DE	ED 5A	HL=HL+DE+CY

ADC HL,HL	ED 6A	HL=HL+HL+CY

ADC HL,SP	ED 7A	HL=HL+SP+CY

SBC HL,BC	ED 42	HL=HL-BC-CY

SBC HL,DE	ED 52	HL=HL-DE-CY

SBC HL,HL	ED 62	HL=HL-HL-CY

SBC HL,SP	ED 72	HL=HL-SP-CY

ADD IX,BC	DD 09	IX=IX+BC

ADD IX,DE	DD 19	IX=IX+DE

ADD IX,IX	DD 29	IX=IX+IX

ADD IX,SP	DD 39	IX=IX+SP

ADD IY,BC	FD 09	IY=IY+BC

ADD IY,DE	FD 19	IY=IY+DE

ADD IY,IY	FD 29	IY=IY+IY

ADD IY,SP	FD 39	IY=IY+SP

INC BC	03	BC=BC+1

INC DE	13	DE=DE+1

INC HL	23	HL=HL+1

INC SP	33	SP=SP+1

INC IX	DD 23	IX=IX+1

INC IY	FD 23	IY=IY+1

DEC BC	0B	BC=BC-1

DEC DE	1B	DE=DE-1

DEC HL	2B	HL=HL-1

DEC SP	3B	SP=SP-1

DEC IX	DD 2B	IX=IX-1

DEC IY	FD 2B	IY=IY-1

ROTATE AND SHIFT GROUP

RLCA	07	A rotated left, bit0=bit7, CY=bit7

RLA	17	A rotated left, bit0=CY, CY=bit7

RRCA	0F	A rotated right, bit7=bit0, CY=bit0

RRA	1F	A rotated right, bit7=CY, CY=bit0

RLC A	CB 07	A rotated left, bit0=bit7, CY=bit7

RLC B	CB 00	B rotated left, bit0=bit7, CY=bit7

RLC C	CB 01	C rotated left, bit0=bit7, CY=bit7

RLC D	CB 02	D rotated left, bit0=bit7, CY=bit7

RLC E	CB 03	E rotated left, bit0=bit7, CY=bit7

RLC H	CB 04	H rotated left, bit0=bit7, CY=bit7

RLC L	CB 05	L rotated left, bit0=bit7, CY=bit7

RLC (HL)	CB 06	(HL) rotated left, bit0=bit7, CY=bit7

RLC (IX+d)	DD CB d 06	(IX+d) rotated left, bit0=bit7, CY=bit7

RLC (IY+d)	FD CB d 06	(IY+d) rotated left, bit0=bit7, CY=bit7

RL A	CB 17	A rotated left, bit0=CY, CY=bit7

RL B	CB 10	B rotated left, bit0=CY, CY=bit7

RL C	CB 11	C rotated left, bit0=CY, CY=bit7

RL D	CB 12	D rotated left, bit0=CY, CY=bit7

RL E	CB 13	E rotated left, bit0=CY, CY=bit7

RL H	CB 14	H rotated left, bit0=CY, CY=bit7

RL L	CB 15	L rotated left, bit0=CY, CY=bit7

RL (HL)	CB 16	(HL) rotated left, bit0=CY, CY=bit7

RL (IX+d)	DD CB d 16	(IX+d) rotated left, bit0=CY, CY=bit7

RL (IY+d)	FD CB d 16	(IY+d) rotated left, bit0=CY, CY=bit7

RRC A	CB 0F	A rotated right, bit7=bit0, CY=bit0

RRC B	CB 08	B rotated right, bit7=bit0, CY=bit0

RRC C	CB 09	C rotated right, bit7=bit0, CY=bit0

RRC D	CB 0A	D rotated right, bit7=bit0, CY=bit0

RRC E	CB 0B	E rotated right, bit7=bit0, CY=bit0

RRC H	CB 0C	H rotated right, bit7=bit0, CY=bit0

RRC L	CB 0D	L rotated right, bit7=bit0, CY=bit0

RRC (HL)	CB 0E	(HL) rotated right, bit7=bit0, CY=bit0

RRC (IX+d)	DD CB d 0E	(IX+d) rotated right, bit7=bit0, CY=bit0

RRC (IY+d)	FD CB d 0E	(IY+d) rotated right, bit7=bit0, CY=bit0

RR A	CB 1F	A rotated right, bit7=CY, CY=bit0

RR B	CB 18	B rotated right, bit7=CY, CY=bit0

RR C	CB 19	C rotated right, bit7=CY, CY=bit0

RR D	CB 1A	D rotated right, bit7=CY, CY=bit0

RR E	CB 1B	E rotated right, bit7=CY, CY=bit0

RR H	CB 1C	H rotated right, bit7=CY, CY=bit0

RR L	CB 1D	L rotated right, bit7=CY, CY=bit0

RR (HL)	CB 1E	(HL) rotated right, bit7=CY, CY=bit0

RR (IX+d)	DD CB d 1E	(IX+d) rotated right, bit7=CY, CY=bit0

RR (IY+d)	FD CB d 1E	(IY+d) rotated right, bit7=CY, CY=bit0

SLA A	CB 27	A shifted left, bit0=0, CY=bit7

SLA B	CB 20	B shifted left, bit0=0, CY=bit7

SLA C	CB 21	C shifted left, bit0=0, CY=bit7

SLA D	CB 22	D shifted left, bit0=0, CY=bit7

SLA E	CB 23	E shifted left, bit0=0, CY=bit7

SLA H	CB 24	H shifted left, bit0=0, CY=bit7

SLA L	CB 25	L shifted left, bit0=0, CY=bit7

SLA (HL)	CB 26	(HL) shifted left, bit0=0, CY=bit7

SLA (IX+d)	DD CB d 26	(IX+d) shifted left, bit0=0, CY=bit7

SLA (IY+d)	FD CB d 26	(IY+d) shifted left, bit0=0, CY=bit7

SRA A	CB 2F	A shifted right, bit7 unchanged, CY=bit0

SRA B	CB 28	B shifted right, bit7 unchanged, CY=bit0

SRA C	CB 29	C shifted right, bit7 unchanged, CY=bit0

SRA D	CB 2A	D shifted right, bit7 unchanged, CY=bit0

SRA E	CB 2B	E shifted right, bit7 unchanged, CY=bit0

SRA H	CB 2C	H shifted right, bit7 unchanged, CY=bit0

SRA L	CB 2D	L shifted right, bit7 unchanged, CY=bit0

SRA (HL)	CB 2E	(HL) shifted right, bit7 unchanged, CY=bit0

SRA (IX+d)	DD CB d 2E	(IX+d) shifted right, bit7 unchanged, CY=bit0

SRA (IY+d)	FD CB d 2E	(IY+d) shifted right, bit7 unchanged, CY=bit0

SRL A	CB 3F	A shifted right, bit7=0, CY=bit0

SRL B	CB 38	B shifted right, bit7=0, CY=bit0

SRL C	CB 39	C shifted right, bit7=0, CY=bit0

SRL D	CB 3A	D shifted right, bit7=0, CY=bit0

SRL E	CB 3B	E shifted right, bit7=0, CY=bit0

SRL H	CB 3C	H shifted right, bit7=0, CY=bit0

SRL L	CB 3D	L shifted right, bit7=0, CY=bit0

SRL (HL)	CB 3E	(HL) shifted right, bit7=0, CY=bit0

SRL (IX+d)	DD CB d 3E	(IX+d) shifted right, bit7=0, CY=bit0

SRL (IY+d)	FD CB d 3E	(IY+d) shifted right, bit7=0, CY=bit0

RLD	ED 6F	too complicated

RRD	ED 67	too complicated

INPUT AND OUTPUT GROUP

IN A,(n)	DB n	A=(n)

IN A,(C)	ED 78	A=(C)

IN B,(C)	ED 40	B=(C)

IN C,(C)	ED 48	C=(C)

IN D,(C)	ED 50	D=(C)

IN E,(C)	ED 58	E=(C)

IN H,(C)	ED 60	H=(C)

IN L,(C)	ED 68	L=(C)

INI	ED A2	(HL)=(C); B=B-1; HL=HL+1

INIR	ED B2	(HL)=(C); B=B-1; HL=HL+1; Repeat until B=0

IND	ED AA	(HL)=(C); B=B-1; HL=HL-1

INDR	ED BA	(HL)=(C); B=B-1; HL=HL-1; Repeat until B=0

OUT (n),A	D3 n	(n)=A

OUT (C),A	ED 79	(C)=A

OUT (C),B	ED 41	(C)=B

OUT (C),C	ED 49	(C)=C

OUT (C),D	ED 51	(C)=D

OUT (C),E	ED 59	(C)=E

OUT (C),H	ED 61	(C)=H

OUT (C),L	ED 69	(C)=L

OUTI	ED A3	(C)=(HL); B=B-1; HL=HL+1

OTIR	ED B3	(C)=(HL); B=B-1; HL=HL+1; Repeat until B=0

OUTD	ED AB	(C)=(HL); B=B-1; HL=HL-1

OTDR	ED BB	(C)=(HL); B=B-1; HL=HL-1; Repeat until B=0

JUMP GROUP

JP nn	C3 n n	PC=nn

JP NZ,nn	C2 n n	if non zero then PC=nn

JP Z,nn	CA n n	if zero then PC=nn

JP NC,nn	D2 n n	if no carry then PC=nn

JP C,nn	DA n n	if carry then PC=nn

JP PO,nn	E2 n n	if parity odd then PC=nn

JP PE,nn	EA n n	if parity even then PC=nn

JP P,nn	F2 n n	if sign positive then PC=nn

JP M,nn	FA n n	if sign negative then PC=nn

JR d	18 d	PC=PC+d

JR C,d	38 d	if C=1 then PC=PC+d

JR NC,d	30 d	if C=0 then PC=PC+d

JR Z,d	28 d	if Z=1 then PC=PC+d

JR NZ,d	20 d	if Z=0 then PC=PC+d

JP (HL)	E9	PC=HL

JP (IX)	DD E9	PC=IX

JP (IY)	FD E9	PC=IY

DJNZ d	10 d	B=B-1; if B<>0 then PC=PC+d

CALL AND RETURN GROUP

CALL nn	CD n n	(SP-1)=PCH; (SP-2)=PCL; SP=SP-2; PC=nn

CALL NZ,nn	C4 n n	if NZ then (SP-1)=PCH; (SP-2)=PCL; SP=SP-2; PC=nn

CALL Z,nn	CC n n	if Z then (SP-1)=PCH; (SP-2)=PCL; SP=SP-2; PC=nn

CALL NC,nn	D4 n n	if NC then (SP-1)=PCH; (SP-2)=PCL; SP=SP-2; PC=nn

CALL C,nn	DC n n	if C then (SP-1)=PCH; (SP-2)=PCL; SP=SP-2; PC=nn

CALL PO,nn	E4 n n	if PO then (SP-1)=PCH; (SP-2)=PCL; SP=SP-2; PC=nn

CALL PE,nn	EC n n	if PE then (SP-1)=PCH; (SP-2)=PCL; SP=SP-2; PC=nn

CALL P,nn	F4 n n	if P then (SP-1)=PCH; (SP-2)=PCL; SP=SP-2; PC=nn

CALL M,nn	FC n n	if M then (SP-1)=PCH; (SP-2)=PCL; SP=SP-2; PC=nn

RET	C9	PCL=(SP); PCH=(SP+1); SP=SP+2

RET NZ	C0	if NZ then PCL=(SP); PCH=(SP+1); SP=SP+2

RET Z	C8	if Z then PCL=(SP); PCH=(SP+1); SP=SP+2

RET NC	D0	if NC then PCL=(SP); PCH=(SP+1); SP=SP+2

RET C	D8	if C then PCL=(SP); PCH=(SP+1); SP=SP+2

RET PO	E0	if PO then PCL=(SP); PCH=(SP+1); SP=SP+2

RET PE	E8	if PE then PCL=(SP); PCH=(SP+1); SP=SP+2

RET P	F0	if P then PCL=(SP); PCH=(SP+1); SP=SP+2

RET M	F8	if M then PCL=(SP); PCH=(SP+1); SP=SP+2

RETI	ED 4D	Return from interrupt

RETN	ED 45	Return from non maskable interrupt

RST 00H	C7	(SP-1)=PCH; (SP-2)=PCL; SP=SP-2; PC=0000H

RST 08H	CF	(SP-1)=PCH; (SP-2)=PCL; SP=SP-2; PC=0008H

RST 10H	D7	(SP-1)=PCH; (SP-2)=PCL; SP=SP-2; PC=0010H

RST 18H	DF	(SP-1)=PCH; (SP-2)=PCL; SP=SP-2; PC=0018H

RST 20H	E7	(SP-1)=PCH; (SP-2)=PCL; SP=SP-2; PC=0020H

RST 28H	EF	(SP-1)=PCH; (SP-2)=PCL; SP=SP-2; PC=0028H

RST 30H	F7	(SP-1)=PCH; (SP-2)=PCL; SP=SP-2; PC=0030H

RST 38H	FF	(SP-1)=PCH; (SP-2)=PCL; SP=SP-2; PC=0038H

BIT SET, RESET AND TEST GROUP

BIT 0,A	CB 47	Z=NOT bit0

BIT 0,B	CB 40	Z=NOT bit0

BIT 0,C	CB 41	Z=NOT bit0

BIT 0,D	CB 42	Z=NOT bit0

BIT 0,E	CB 43	Z=NOT bit0

BIT 0,H	CB 44	Z=NOT bit0

BIT 0,L	CB 45	Z=NOT bit0

BIT 1,A	CB 4F	Z=NOT bit1

BIT 1,B	CB 48	Z=NOT bit1

BIT 1,C	CB 49	Z=NOT bit1

BIT 1,D	CB 4A	Z=NOT bit1

BIT 1,E	CB 4B	Z=NOT bit1

BIT 1,H	CB 4C	Z=NOT bit1

BIT 1,L	CB 4D	Z=NOT bit1

BIT 2,A	CB 57	Z=NOT bit2

BIT 2,B	CB 50	Z=NOT bit2

BIT 2,C	CB 51	Z=NOT bit2

BIT 2,D	CB 52	Z=NOT bit2

BIT 2,E	CB 53	Z=NOT bit2

BIT 2,H	CB 54	Z=NOT bit2

BIT 2,L	CB 55	Z=NOT bit2

BIT 3,A	CB 5F	Z=NOT bit3

BIT 3,B	CB 58	Z=NOT bit3

BIT 3,C	CB 59	Z=NOT bit3

BIT 3,D	CB 5A	Z=NOT bit3

BIT 3,E	CB 5B	Z=NOT bit3

BIT 3,H	CB 5C	Z=NOT bit3

BIT 3,L	CB 5D	Z=NOT bit3

BIT 4,A	CB 67	Z=NOT bit4

BIT 4,B	CB 60	Z=NOT bit4

BIT 4,C	CB 61	Z=NOT bit4

BIT 4,D	CB 62	Z=NOT bit4

BIT 4,E	CB 63	Z=NOT bit4

BIT 4,H	CB 64	Z=NOT bit4

BIT 4,L	CB 65	Z=NOT bit4

BIT 5,A	CB 6F	Z=NOT bit5

BIT 5,B	CB 68	Z=NOT bit5

BIT 5,C	CB 69	Z=NOT bit5

BIT 5,D	CB 6A	Z=NOT bit5

BIT 5,E	CB 6B	Z=NOT bit5

BIT 5,H	CB 6C	Z=NOT bit5

BIT 5,L	CB 6D	Z=NOT bit5

BIT 6,A	CB 77	Z=NOT bit6

BIT 6,B	CB 70	Z=NOT bit6

BIT 6,C	CB 71	Z=NOT bit6

BIT 6,D	CB 72	Z=NOT bit6

BIT 6,E	CB 73	Z=NOT bit6

BIT 6,H	CB 74	Z=NOT bit6

BIT 6,L	CB 75	Z=NOT bit6

BIT 7,A	CB 7F	Z=NOT bit7

BIT 7,B	CB 78	Z=NOT bit7

BIT 7,C	CB 79	Z=NOT bit7

BIT 7,D	CB 7A	Z=NOT bit7

BIT 7,E	CB 7B	Z=NOT bit7

BIT 7,H	CB 7C	Z=NOT bit7

BIT 7,L	CB 7D	Z=NOT bit7

BIT 0,(HL)	CB 46	Z=NOT bit0

BIT 1,(HL)	CB 4E	Z=NOT bit1

BIT 2,(HL)	CB 56	Z=NOT bit2

BIT 3,(HL)	CB 5E	Z=NOT bit3

BIT 4,(HL)	CB 66	Z=NOT bit4

BIT 5,(HL)	CB 6E	Z=NOT bit5

BIT 6,(HL)	CB 76	Z=NOT bit6

BIT 7,(HL)	CB 7E	Z=NOT bit7

BIT 0,(IX+d)	DD CB d 46	Z=NOT bit0

BIT 1,(IX+d)	DD CB d 4E	Z=NOT bit1

BIT 2,(IX+d)	DD CB d 56	Z=NOT bit2

BIT 3,(IX+d)	DD CB d 5E	Z=NOT bit3

BIT 4,(IX+d)	DD CB d 66	Z=NOT bit4

BIT 5,(IX+d)	DD CB d 6E	Z=NOT bit5

BIT 6,(IX+d)	DD CB d 76	Z=NOT bit6

BIT 7,(IX+d)	DD CB d 7E	Z=NOT bit7

BIT 0,(IY+d)	FD CB d 46	Z=NOT bit0

BIT 1,(IY+d)	FD CB d 4E	Z=NOT bit1

BIT 2,(IY+d)	FD CB d 56	Z=NOT bit2

BIT 3,(IY+d)	FD CB d 5E	Z=NOT bit3

BIT 4,(IY+d)	FD CB d 66	Z=NOT bit4

BIT 5,(IY+d)	FD CB d 6E	Z=NOT bit5

BIT 6,(IY+d)	FD CB d 76	Z=NOT bit6

BIT 7,(IY+d)	FD CB d 7E	Z=NOT bit7

SET 0,A	CB C7	bit0=1

SET 0,B	CB C0	bit0=1

SET 0,C	CB C1	bit0=1

SET 0,D	CB C2	bit0=1

SET 0,E	CB C3	bit0=1

SET 0,H	CB C4	bit0=1

SET 0,L	CB C5	bit0=1

SET 1,A	CB CF	bit1=1

SET 1,B	CB C8	bit1=1

SET 1,C	CB C9	bit1=1

SET 1,D	CB CA	bit1=1

SET 1,E	CB CB	bit1=1

SET 1,H	CB CC	bit1=1

SET 1,L	CB CD	bit1=1

SET 2,A	CB D7	bit2=1

SET 2,B	CB D0	bit2=1

SET 2,C	CB D1	bit2=1

SET 2,D	CB D2	bit2=1

SET 2,E	CB D3	bit2=1

SET 2,H	CB D4	bit2=1

SET 2,L	CB D5	bit2=1

SET 3,A	CB DF	bit3=1

SET 3,B	CB D8	bit3=1

SET 3,C	CB D9	bit3=1

SET 3,D	CB DA	bit3=1

SET 3,E	CB DB	bit3=1

SET 3,H	CB DC	bit3=1

SET 3,L	CB DD	bit3=1

SET 4,A	CB E7	bit4=1

SET 4,B	CB E0	bit4=1

SET 4,C	CB E1	bit4=1

SET 4,D	CB E2	bit4=1

SET 4,E	CB E3	bit4=1

SET 4,H	CB E4	bit4=1

SET 4,L	CB E5	bit4=1

SET 5,A	CB EF	bit5=1

SET 5,B	CB E8	bit5=1

SET 5,C	CB E9	bit5=1

SET 5,D	CB EA	bit5=1

SET 5,E	CB EB	bit5=1

SET 5,H	CB EC	bit5=1

SET 5,L	CB ED	bit5=1

SET 6,A	CB F7	bit6=1

SET 6,B	CB F0	bit6=1

SET 6,C	CB F1	bit6=1

SET 6,D	CB F2	bit6=1

SET 6,E	CB F3	bit6=1

SET 6,H	CB F4	bit6=1

SET 6,L	CB F5	bit6=1

SET 7,A	CB FF	bit7=1

SET 7,B	CB F8	bit7=1

SET 7,C	CB F9	bit7=1

SET 7,D	CB FA	bit7=1

SET 7,E	CB FB	bit7=1

SET 7,H	CB FC	bit7=1

SET 7,L	CB FD	bit7=1

SET 0,(HL)	CB C6	bit0=1

SET 1,(HL)	CB CE	bit1=1

SET 2,(HL)	CB D6	bit2=1

SET 3,(HL)	CB DE	bit3=1

SET 4,(HL)	CB E6	bit4=1

SET 5,(HL)	CB EE	bit5=1

SET 6,(HL)	CB F6	bit6=1

SET 7,(HL)	CB FE	bit7=1

SET 0,(IX+d)	DD CB d C6	bit0=1

SET 1,(IX+d)	DD CB d CE	bit1=1

SET 2,(IX+d)	DD CB d D6	bit2=1

SET 3,(IX+d)	DD CB d DE	bit3=1

SET 4,(IX+d)	DD CB d E6	bit4=1

SET 5,(IX+d)	DD CB d EE	bit5=1

SET 6,(IX+d)	DD CB d F6	bit6=1

SET 7,(IX+d)	DD CB d FE	bit7=1

SET 0,(IY+d)	FD CB d C6	bit0=1

SET 1,(IY+d)	FD CB d CE	bit1=1

SET 2,(IY+d)	FD CB d D6	bit2=1

SET 3,(IY+d)	FD CB d DE	bit3=1

SET 4,(IY+d)	FD CB d E6	bit4=1

SET 5,(IY+d)	FD CB d EE	bit5=1

SET 6,(IY+d)	FD CB d F6	bit6=1

SET 7,(IY+d)	FD CB d FE	bit7=1

RES 0,A	CB 87	bit0=0

RES 0,B	CB 80	bit0=0

RES 0,C	CB 81	bit0=0

RES 0,D	CB 82	bit0=0

RES 0,E	CB 83	bit0=0

RES 0,H	CB 84	bit0=0

RES 0,L	CB 85	bit0=0

RES 1,A	CB 8F	bit1=0

RES 1,B	CB 88	bit1=0

RES 1,C	CB 89	bit1=0

RES 1,D	CB 8A	bit1=0

RES 1,E	CB 8B	bit1=0

RES 1,H	CB 8C	bit1=0

RES 1,L	CB 8D	bit1=0

RES 2,A	CB 97	bit2=0

RES 2,B	CB 90	bit2=0

RES 2,C	CB 91	bit2=0

RES 2,D	CB 92	bit2=0

RES 2,E	CB 93	bit2=0

RES 2,H	CB 94	bit2=0

RES 2,L	CB 95	bit2=0

RES 3,A	CB 9F	bit3=0

RES 3,B	CB 98	bit3=0

RES 3,C	CB 99	bit3=0

RES 3,D	CB 9A	bit3=0

RES 3,E	CB 9B	bit3=0

RES 3,H	CB 9C	bit3=0

RES 3,L	CB 9D	bit3=0

RES 4,A	CB A7	bit4=0

RES 4,B	CB A0	bit4=0

RES 4,C	CB A1	bit4=0

RES 4,D	CB A2	bit4=0

RES 4,E	CB A3	bit4=0

RES 4,H	CB A4	bit4=0

RES 4,L	CB A5	bit4=0

RES 5,A	CB AF	bit5=0

RES 5,B	CB A8	bit5=0

RES 5,C	CB A9	bit5=0

RES 5,D	CB AA	bit5=0

RES 5,E	CB AB	bit5=0

RES 5,H	CB AC	bit5=0

RES 5,L	CB AD	bit5=0

RES 6,A	CB B7	bit6=0

RES 6,B	CB B0	bit6=0

RES 6,C	CB B1	bit6=0

RES 6,D	CB B2	bit6=0

RES 6,E	CB B3	bit6=0

RES 6,H	CB B4	bit6=0

RES 6,L	CB B5	bit6=0

RES 7,A	CB BF	bit7=0

RES 7,B	CB B8	bit7=0

RES 7,C	CB B9	bit7=0

RES 7,D	CB BA	bit7=0

RES 7,E	CB BB	bit7=0

RES 7,H	CB BC	bit7=0

RES 7,L	CB BD	bit7=0

RES 0,(HL)	CB 86	bit0=0

RES 1,(HL)	CB 8E	bit1=0

RES 2,(HL)	CB 96	bit2=0

RES 3,(HL)	CB 9E	bit3=0

RES 4,(HL)	CB A6	bit4=0

RES 5,(HL)	CB AE	bit5=0

RES 6,(HL)	CB B6	bit6=0

RES 7,(HL)	CB BE	bit7=0

RES 0,(IX+d)	DD CB d 86	bit0=0

RES 1,(IX+d)	DD CB d 8E	bit1=0

RES 2,(IX+d)	DD CB d 96	bit2=0

RES 3,(IX+d)	DD CB d 9E	bit3=0

RES 4,(IX+d)	DD CB d A6	bit4=0

RES 5,(IX+d)	DD CB d AE	bit5=0

RES 6,(IX+d)	DD CB d B6	bit6=0

RES 7,(IX+d)	DD CB d BE	bit7=0

RES 0,(IY+d)	FD CB d 86	bit0=0

RES 1,(IY+d)	FD CB d 8E	bit1=0

RES 2,(IY+d)	FD CB d 96	bit2=0

RES 3,(IY+d)	FD CB d 9E	bit3=0

RES 4,(IY+d)	FD CB d A6	bit4=0

RES 5,(IY+d)	FD CB d AE	bit5=0

RES 6,(IY+d)	FD CB d B6	bit6=0

RES 7,(IY+d)	FD CB d BE	bit7=0