Originated - 7/8/82 by Tom Vyse, San Diego, Calif.

The FAST program is specific to the Osborne 1.

It also will only work on those Osbornes which have

MPI disk drives. Those can be identified by a plastic

lever hanging down in the right hand side of the

disk drive slot. This is part of the door interlock

mechanism. Osbornes are also shipped with other, and

frequently slower, drives from different manufac-

turers.

FAST is designed to increase the disk drive step

rate from 20 ms./step (the default) to 6ms./step.

It does this by altering the value in the operating

system parameter table which controls this rate.

The address of this parameter (in both versions

1.2 and 1.3) is EF13H.  The effect of various

parameters in this location are:

Value   Step Rate

-----   ---------

0      6 ms. (MPI only)

1     12 ms.

2     20 ms. (Seimens)

3     30 ms.

The code to drive this is simply:

MVI    A,0

STA    0EF13H

JMP    0

END

Not much of a program, but it sure makes programs

with a lot of disk access run nicer. Also, the MPI

drive will operate more quietly at the higher step

rate.

Fini.

##################################