Here are some things I found out about the Exec.

1)  DOCINDEX.DOC- I  wrote  this the first month I had  my  Exec,

since,  as you know,  the write-ups for all the various  CP/M+

utilities  are scattered throughout the six books we  received

with the Exec. (and people thought the Osborne 1 book was hard

to  use!)  If  you didn't get volumes 0-4  and  the  looseleaf

"Reference Guide", GET THEM! You can circumvent this scattered

approach  somewhat by buying the complete CP/M+  documentation

from Digital Research ($40. and maybe worth it), but even then

you don't get write-ups of the programs written by OCC:  COPY,

CHARGEN,  COPYSYS,  and SETUP.  Voila!  my one page index.  It

tells  where the write-ups are and also on which of the  three

system disks the program appears.

Also, at the bottom of the page is a quick translation guide

to let you converse with OsOne and other CP/M 2.2 users.

2) After you push the reset button, you have three choices:

A) press return- normal load process

B) press T- run the self-test until you press reset again

C) press Tab- this inverts the drives; your top drive is now B:

and your bottom drive is A: (where your CP/M+ system disk

needs to be)

X) The Exec will load any program you rename EXECST.COM as it

loads the system.

3)Alternate character fonts

A) first, load one; there are three on this disk. CHARGEN.COM

on  your first CP/M+ disk will let you "read a file into the

character  set." Use INVERTED.CHR from this  disk- go  ahead

and  "write  the character set to the system tracks" as  the

Alternate character set.   Unless you  have already modified

your character fonts, your main and alternate character sets

were identical. (you can un-do this later, if you want.)

B) now, Reset, Return. (You probably call this "Booting", but I

call it initial program load or "I.P.L.ing", so let's use

terms we all understand) This loads your original (Main) and

new (Alternate) character sets to the font RAM.

C) Now,  as you keystroke,  hitting Control Tab will toggle you

back  and forth from your Main to Alternate  fonts.  If  you

used my INVERTED.CHR, isn't this a lot easier than using

SETUP.COM each time you want inverted characters?

D)  The  other  fonts  I have  included  are  BOLDFACE.CHR  and

BOLDINVR.CHR,  which  I  find somewhat easier  to  read.  Be

creative! If you invent a nice font, please get it into this

FOG library.

4)  Hardware- The Zilog Z-80A SIO2 Serial Input/Output Controller

is a monster! Let me quote from Zilog's Data Book:

"Two independant full-duplex channels,  with separate control

and status lines for modems or other devices.

"Data  rates of 0 to 800k bits/second with a 4.0  MHz  clock.

"Asynchronous  protocols:  everything necessary for  complete

messages in 5,  6,  7, or 8 bits/character.

"Synchronous protocols: everything necessary for complete bit

or byte oriented messages in 5, 6, 7 or 8 bits/character, in-

cluding IBM Bisync,  SDLC, HDLC, CCITT-X.25 and others. Auto-

matic CRC generation/checking, sync character and zero inser-

tion/deletion, abort generation/detection and flag insertion"

Etc. I'm not even sure what all of that is, but     what I do

understand really verifies what Ron Brown said at the January

MtDOG  meeting;  that  is,  the Exec was intended  to      be

neither an improved OsOne nor an IBM-PC clone, but a communi#

cation machine.  (For comparison, the OsOne uses a 6850 Motor#

ola  asynchronous  communication interface  adapter,  with  a

speed  of  1200 bits/sec or less) Have you watched  a  Modem7

download at 9600 or 19200 bps? it's FAST!

So,  since  the Data Book I quoted from above was the 1981

edition, why hasn't the UTE Universal Terminal Emulator prog-

ram been released yet?  OCC isn't the only company using  the

Z80A SIO2,  is it?  I mean, the wheel must have been invented

already by somebody, and I want it.

5)Hardware- the  OsOne has a Western Digital 1793 floppy  disk

controller, while the OsEx has a 1797. The difference is side

select.  So come on,  aftermarket!  sell me some  half-height

double-sided drives with a modified BIOS or BDOS so I can use

them.  That's all that's needed.  That says 366K of formatted

storage per disk to me.

6)Hardware- Parallel  printer connection. The documentation is

a mess, so pay attention.

Look  at  your IEEE 488 connector closely.  The  top  row  of

contacts is labled 12-1 (left to right), while the bottom row

is labled 24-13.  This is just like the picture on page R-443

(R means Reference Guide), and this matches the listing on p.

R-442.  BUT,  the  signal descriptions on R-442 are IEEE 488,

and  not Centronics or other printer manufacturers.

The listing on R-445 matches the listing on 2-210, but the

only  picture that matches these lists is in the Osborne  One

Reference Guide!

Using the listings on R-445 and 2-210, use this picture:

23 21 19 17 15 13 11  9  7  5  3  1

24 22 20 18 16 14 12 10  8  6  4  2

_________________________________________________________________

OSBORNE EXECUTIVE                           PARALLEL PRINTER

pins 1-8           Data Bits >>>>>>>>>>>>>>>>

pin 11             Strobe >>>>>>>>>>>>>>>>>>>

pin 15  <<<<<<<<<< Busy

pin 19  <<<<<<<<<< Select, Ready, Demand

pins 10, 12 14 16 18 20 22 24  Ground.

Select,  Ready,  Demand, Service Request are different names

for  the  same signal.  My printer,  and maybe all  printers

alternate this signal with Busy.

That's all I know for now, and the more I know, the more I know I

don't know.  I'm Clark Wood, aka CWOOD, and I am a member of East

Bay Chapter of FOG, and a systems engineer for Kaiser Permanente

Medical Care Program, Oakland. I may be reached for talk or data

(at 300 bps) at (415)724-6668, evenings.     CWOOD       17FEB84######################