***********************************************************************

*                                                                      *

*                  D I S K  E X E R C I S O R  1 . 0                   *

*                                                                      *

************************************************************************

Copyright Bill Rink; 1984

(**Configured for Oxborne 1 by Mel Hulse**)

(DOES NOT WORK ON THE EXECUTIVE)

This software is made available to the public domain and may be freely

distributed. It is not to be sold or used for commercial use.

DISKX.COM is a comprehensive disk exercisor that can be used to certify

either the operation of your disk drive(s) or the quality of a floppy.

It is particularly useful if you are trying out a new brand of diskettes.

I wrote it initially for testing some new 96 tpi Shuggart drives and

to check the compatabilty of 48 tpi diskettes with them.

This program was written for a Western Digital (or compatable) 179X

Floppy Disk Controller and so the error conditions detected are set-up

for that device. (Note: the Osborne 1 uses a 1793 look alike. - MWH.) If

you have some other controller it will still work but you will not get

error counts for CRC, RNF (record not found), or Write Fault errors.

You will be prompted for the status port address of your 179X controller.

For the Osborne 1 the disk status port address is 2100. For computers not

using the 179x controller simply press return.

(note: 179X stands for 1791,1792,1793,1794,1795,1797 controller family.)

DISKX.COM can be initialized to test up to 2 drives. There are three

basic tests performed on each pass.

Verify Scan, Read

This test will scan the disk first to verify it is free of format and

CRC errors. This is a READ only test.

Sequential, R/W

A random data pattern is written and the read back on every track and

sector on the disk. A spiral pattern is used to start with the first

track/sector and then sprial to the inner most tracks, then repeating

this after incrementing the sector bias by one.

Random, R/W

Random data is written and read on randomly generated track and sector

numbers. This will give the disk a good workout and more closely resemble

real activity, though more intensive.

(I know that there are certain data patterns that are good for detecting

certain write problems, but I was in a hurry to get something together,

so I used random data patterns.)

Each test will loop for the number displayed in brackets next to the loop

counter. At any time, you may either SKIP the current test or HALT the

program by entering either 'S' or 'H'.

Play with it, you'll get the idea.

INSTALLATION

Provided in the library file is an istallation program called DISKXINS.COM

You must also have the DISKX.INS and DISKX.DTA files on you disk at the time

of installation. Simply execute the DISKXINS.COM file and follow the

instructions. Use the "None of above" selection if your terminal is not

listed.

#ute the DISKXINS.COM file and follow the

instructions. Use the "None of above" selection if your termina