This program times your Osborne 1 disk drives.

The form of the command is

TD<cr>  or  TD A<cr>   or  TD B<cr>

There must be a diskette in the target drive for

this program to work properly.  TD<cr> without

a specified disk drive will default to the "boot"

drive.

The displayed D range is for double density.

Optimum is from 200 to 203 ms/rev.  The farther

from this range that your drive varies, the greater

the chances of BDOS errors.  Single density drives

are more tolerant of the variation in speed, and

I've seen one work at 215 ms/rev.

To stop the program, press and hold any key.  To

test the opposite drive, press and hold the key

that corresponds to that drive.  If no mark appears

to indicate the timing, no index mark is being

detected and the program will wait until one is

detected or you press the RESET button.

-M.J.Guerra

######################################################################################################################