NAME

INSTALL  - integrate new device drivers into  CP/M,  Revision

1.3.

SYNOPSIS

INSTALL - string newbios address [ prlfile2 address2 ... ]

DESCRIPTION

INSTALL  integrates new device drivers into an existing  CP/M

2.x system.  "Newbios" is a skeletal CP/M BIOS module in .PRL

format  (see the PRL.DOC  file) containing the  new  drivers.

INSTALL  first loads "newbios" at the given address  (usually

the  top of CP/M),  then patches it into the existing BIOS by

exchanging  the addresses of the 17 jump vectors  that  begin

each  BIOS.  If  additional  .PRL files are given,  they  are

loaded at the requested addresses with no patching.  Finally,

INSTALL  prints

(top of newbios) + (newbios buffer size, if any) + 1

as  the  "new load address".   Additional drivers can now  be

installed at this address.

The  flagged  argument  (-string) is copied to 80H  and  left

there for examination by the newly installed "Newbios",  or a

NULL  if  no flagged argument is present.  The  BIOS  modules

provided  here  use this as an initial  drive  specification,

i.e.,  -c  specifies  that the newly  installed  BIOS  should

respond to drives C.

The  CP/M system must be standard:  its BIOS must begin  with

the jump table as defined in the "CP/M 2.0 Alteration Guide",

and  it must store the address of this table at location 1 in

memory.

A "skeletal" BIOS module begins with the same jump table, but

may leave some of the required functions unimplemented.   The

corresponding jump vectors are left as jump-to-selfs.

EXAMPLES

To install the 14 inch hard disk (M26) as drives C,  D, and E

in  a  48K  CP/M (assuming that you have memory  above  48K),

execute

INSTALL -C M26.prl C000

Here,  M26.prl is the "skeletal BIOS" (provided with INSTALL)

containing the 14 inch hard disk drivers,  and C000 (hex  for

48K)  is the top of CP/M.   If you only have 48K of memory in

.pa

your system,  you will first have to move your CP/M  downward

to make room.  At this writing, the 14 inch hard disk drivers

required approximately 3.5K bytes,  (see FILES below), so you

should construct a 44K CP/M, and type

INSTALL -C M26.prl B000

(B000 is 44K in hex.) If you don't know how to move your CP/M

downward, see  your system manual.  The required  program  is

usually called "MOVCPM".

To  install  the Disk Jockey as drives L and M in a 44K  CP/M

(with at least 48K of memory), type

INSTALL dj.prl  B000  djram.prl  E400

Here,  "dj.prl" contains the Disk Jockey 2D drivers,  B000 is

the  top  of  the  44K  CP/M  (the  drivers  require  3.25K),

"djram.prl"  is updated controller firmware to be loaded into

the controller RAM, and E400 is the address of the controller

RAM. (If you have a non-standard controller, use the control-

ler address +400 hex.)

To  install both the 14 inch hard disk and the  Disk  Jockey

drives, you could first install one, then the other on top of

it  (at the printed "new load address").  This would  require

6.75K of memory above CP/M. As a better alternative, there is

a combined driver package that requires only 5K of memory. To

use it,  configure a 43K CP/M (assuming 48K of memory, or use

available memory minus 5K), and type

INSTALL M26+dj.prl  AC00  djram.prl  E400

BUGS

Addresses must be specified in 4-digit hex (with no leading 0

and no trailing H).  INSTALL should also accept octal and  X-

notation.

When the Disk Jockey drivers are first accessed,  they locate

the  controller by searching memory for a certain pattern  in

the controller ROM.  The search is done carefully, but never-

theless, might clobber someone's memory-mapped I/O.

#################################################################################