DISC FILE ALLOCATION IN

CP/M

THIS DESCRIPTION IS FOR THE STANDARD IMPLEMENTATION OF

CP/M ON SINGLE DENSITY 8 INCH IBM FORMAT FLOPPY DISCS.  THESE

DISCS HAVE 77 TRACKS (NUMBERED: 0-76) AND 26 SECTORS PER TRACK

(NUMBERED: 1-26).  EACH SECTOR HOLDS 128 BYTES OF INFORMATION.

EACH SECTOR HAS A HEADER WHICH (EXCLUSIVE OF THE 128 DATA BYTES)

CONTAINS THE TRACK AND SECTOR IDENTIFICATION NUMBERS (IN THE IBM

FORMAT).  IT IS THESE NUMBERS (READABLE BY THE DISC CONTROLLER)

WHICH SPECIFY THE "PHYSICAL" TRACK AND SECTOR NUMBERING.  THIS

NUMBERING DOES NOT NECESSARILY CORRESPOND TO THE "GEOMETRIC"

NUMBERING (I.E. PHYSICAL SECTORS 1 AND 2 MAY NOT BE ADJACENT ON

THE DISC- DEPENDING ON HOW THE DISC WAS FORMATTED).  IN ANY CASE

, THIS POSSIBLE DIFFERENCE IN THE GEOMETRIC AND PHYSICAL

NUMBERING IS NOT USUALLY APPARENT TO THE CONTROLLER OR SOFTWARE

UNLESS THEY MAKE SOME SPECIFIC TIMING ASSUMPTIONS.  HOWEVER, YOU

CAN USUALLY SEE THE DIFFERENCE IN DISC OPERATIONS IN THESE CASES

.

THE BOOT AND RESIDENT PORTIONS OF THE OPERATING SYSTEM

ARE HANDLED IN A DIFFERENT WAY THAN THOSE FILES WHICH ARE LISTED

IN THE FILE DIRECTORY (AS IS COMMON FOR MANY DISC OPERATING

SYSTEMS).  THE BOOT AND RESIDENT PORTIONS ARE ON TRACKS 0 AND 1

IN A MEMORY IMAGE FORM AS SHOWN BELOW:

BOOT    : TRACK 0, SECTOR 1

RESIDENT: TRACK 0, SECTOR 2,3,...,26

TRACK 1, SECTOR 1,2,...,26.

THE SECTORS LISTED FOR THE RESIDENT ARE ORDERED IN THE WAY THEY

ARE TO BE LOADED BACK INTO MEMORY.  THE REMAINING TRACKS OF THE

DISC ARE ADDRESSED BY CP/M BY A "LOGICAL" SECTORING SCHEME.

BOTH THE DIRECTORY AND ALL FILES IN THE DIRECTORY ARE

ACCESSED IN 1024 BYTE BLOCKS (8- SECTORS) WHICH IN TURN ARE

SPECIFIED BY A LOGICAL SECTORING TABLE.  THE PHYSICAL TO LOGICAL

SECTOR MAP USES A 1:6 INTERLACE AS SHOWN IN THE TABLE BELOW.

LOGICAL     PHYSICAL

SECTOR #    SECTOR #

1           1

2           7

3          13

4          19

5          25

6           5

7          11

8          17

9          23

10           3

11           9

12          15

13          21

14           2

15           8

16          14

17          20

18          26

19           6

20          12

21          18

22          24

23           4

24          10

25          16

26          22

THE USE OF A 1:6 INTERLACE SEEMS TO BE ABOUT OPTIMUM FOR THE

STANDARD CP/M UTILITIES RUNNING WITH A 8080 AT 2 MHZ.  HOWEVER,

A Z-80 RUNNING AT 4 MHZ WITH A FAST DISC CONTROLLER CAN WORK

WITH A 1:3 INTERLACE.  THIS CAN BE FAKED BY THE FORMAT ROUTINE.

EACH DISC BLOCK (1024 BYTES=1K) IS ACCESSED BY ITS SEQUENTIAL

BLOCK # (0-242).  THIS SCHEME IS DEMONSTRATED IN THE TABLE BELOW

BLOCK #  TRACK#   LOGICAL SECTORS         PHYSICAL SECTORS

------I--------I------------------------I----------------------

0        2     1, 2, 3, 4, 5, 6, 7, 8   1,7,13,19,25,5,11,17

1        2     9,10,11,12,13,14,15,16   23,3,9,15,21,2,8,14

2        2    17,18,19,20,21,22,23,24   20,26,6,12,18,24,4,10

3 *      2    26,26                     16,22

3 *      3     1, 2, 3, 4, 5, 6         1,7,13,19,25,5

4        3     7, 8, 9,10,11,12,13,14   11,17,23,3,9,15,21,2

:        :     :                        :

--- DIRECTORY FORMAT ---

THE DIRECTORY OCCUPIES BLOCKS 0 AND 1.  EACH DIRECTORY

ENTRY OCCUPIES 32 BYTES AND IS THE MEMORY IMAGE OF THE FIRST

32 BYTES OF THE 33 BYTE FILE CONTROL BLOCK (FCB) AS DESCRIBED IN

THE CP/M INTERFACE GUIDE (FIGURE 2).  EACH BYTE OF THE 16 "DM"

(DISC ALLOCATION MAP) BYTES GIVES THE BLOCK ADDRESS OF THE DISC

STORAGE OF THE FILE (IN 1024 BYTE PIECES). THUS EACH DIRECTORY

ENTRY CAN DESCRIBE THE DISC ALLOCATION OF 16 BLOCKS (OR LESS).

FILES LARGER THAN 16K (16*1024 BYTES) USE 2 OR MORE DIRECTORY

ENTRYS.  IN FACT A FILE EXACTLY 16*1024 BYTES LONG USES 2 ENTRYS

, EVEN THOUGH THE SECOND ENTRY DESCRIBES THE DISC ALLOCATION OF

ZERO BLOCKS.  DIRECTORY ENTRYS FOR LARGE FILES (>= 128 SECTORS)

CAN BE RECOGNIZED BY EITHER:

1.  "EX" (EXTENT BYTE) >0

OR  2.  "RC" (RECORD COUNT=SECTOR COUNT) =80H (128 DECIMAL).

EACH SUCH DIRECTORY ENTRY DESCRIBES DISC ALLOCATION OF UP TO

128 SECTORS.  THE DIRECTORY ENTRY OF THE TAIL END OF SUCH A

LARGE FILE WILL HAVE "RC" (RECORD COUNT)<80H AMD THUS LESS THAN

16 VALID "DM" BYTES.  THESE MULTIPLE DIRECTORY ENTRYS NEED NOT

OCCURE IN LOGICALLY ADJACENT POSITIONS IN THE DIRECTORY.  WHEN

A FILE IS DELETED, ALL THE DIRECTORY ENTRYS DESCRIBING THE FILE

ARE MODIFIED BY CHANGING THE FIRST BYTE ( THE "ET" BYTE) OF EACH

ENTRY FROM A  00H  TO A  E5H (THE DIRECTORY OF A NEW IBM

FORMATTED DISC IS FILLED WITH  E5H).  A HEX AND ASCII DUMP OF

A TYPICAL DIRECTORY ENTRY IS SHOWN BELOW FOR THE CASE OF A DISC

WITH 2 FILES CREATED BY THE SAVE COMMANDS:

SAVE  10 TEST.COM

SAVE 130 BIG.COM

00: 00 54 45 53 54 20 20 20 20 43 4F 4D 00 00 00 14

.  T  E  S  T              C  O  M  .  .  .  .

10: 02 03 04 00 00 00 00 00 00 00 00 00 00 00 00 00

.  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

20: 00 42 49 47 20 20 20 20 20 43 4F 4D 00 00 00 80

.  B  I  G                 C  O  M  .  .  .  .

30: 05 06 07 08 09 0A 0B 0C 0D 0E 0F 10 11 12 13 14

.  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

40: 00 42 49 47 20 20 20 20 20 43 4F 4D 01 00 00 80

.  B  I  G                 C  O  M  .  .  .  .

50: 15 16 17 18 19 1A 1B 1C 1D 1E 1F 20 21 22 23 24

.  .  .  .  .  .  .  .  .  .  .     !  "  #  $

60: 00 42 49 47 20 20 20 20 20 43 4F 4D 02 00 00 04

.  B  I  G                 C  O  M  .  .  .  .

70: 25 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

%  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

NOTE, IN THIS CASE THE "DM" BYTES ARE IN NUMERICAL ORDER SINCE

THE DISC WAS INITIALLY EMPTY BEFORE THESE FILES WERE SAVED.  IF

THE FILES HAD BEEN SAVED ON A DISC ON WHICH SOME FILES HAD JUST

BEEN ERASED, THERE WOULD PROBABLY BE A MORE RANDOM LOOKING CHOICE

OF THE BLOCKS USED.  HOWEVER, IF THE FILES HAD BEEN SAVED (WITHOUT

ANY INTERVENING DISC READS OR WRITES TO OTHER FILES) THE BLOCKS

CHOSEN WOULD STILL BE IN ASCENDING ORDER SINCE CP/M WILL ALWAYS

SELECT THE NEXT AVAILABLE BLOCK # WITH THE LOWEST NUMERICAL

VALUE.

THERE ARE SEVERAL PROGRAMS ON THE VARIOUS CP/M USER'S

GROUP DISCS THAT CAN BE USED TO EXAMINE THE DISC DIRECTORY

AND FILE ALLOCATION.  THE ONE THAT I HAVE FOUND MOST USEFUL IS

DUMP.COM     (ON DISC # 24).

HOWEVER, THIS MAY NOT WORK FOR ALL DISC CONTROLLERS (IT WORKS

OK FOR LIFEBOATS CP/M ON NORTH STAR 5" DISC AND WITH TARBELL'S

CONTROLLER BUT NOT WITH THE DISCUS CONTROLLER (A BIOS PROBLEM?)).

SPAT1.ASM    (ON DISC #16 ) IS ALSO GOOD.

-- SOME COMMENTS ON LIFEBOATS CP/M IMPLEMENTATION ON --

THE 5" HARD SECTORED NORTH STAR SYSTEM

THE LIFEBOATS IMPLEMENTATION OF CP/M ON THE 5" NORTH STAR

SYSTEM DIFFERS MAINLY IN THEIR UN-SCRAMBLING OF THE 1:6

INTERLACE.  THEIR LARGE   CBIOS   INCLUDES ANOTHER SECTOR

MAPPING TABLE THAT MAPS ONTO THE 5" DISC .  THIS MAPPING

UNWINDS THE 1:6 INTERLACE AND MAPS THE USUAL 8" DISC ADDRESSING

ONTO THE 5" DISC WHICH HAS 35 TRACKS WITH 10 SECTORS (OF 256

BYTES EACH) PER TRACK.  THUS, SAVING THE FILES DESCRIBED ABOVE

UNDER THE LIFEBOATS CP/M (ON A BLANK DISC) WOULD PUT ALL OF THE

PARTS OF THE FILES TOGATHER (THE SAME CONVENTION USED BY THE

USUAL NORTH STAR DOS).  THE FORMAT OF THE BOOT AND RESIDENT

PORTIONS IS SOMEWHAT SCRAMBLED BECAUSE OF THE WAY THE NORTH STAR

BOOT UP ROUTINE WORKS (IT LOADS FROM SECTOR 4 INSTEAD OF 0).

------------------------

BY     BRUCE KENDALL

(415) 967-3613

----------------------------------------------------------------

################################################