SOME NOTES ON THE INSIDES OF CP/M (V1.4)

THIS FILE CONTAINS SECRET INFORMATION- IT SHOULD BE ERASED

BEFORE READING IT.  THIS INFORMATION WAS OBTAINED BY EITHER POKING AROUND

IN CP/M OR FROM EXAMINATION OF CLASSIFIED DOCUMENTS ( FBI & CIA BEWARE).

ALL LOCATIONS ARE GIVEN IN HEX (AN ARBITRARY BASE SHOULD HAVE BEEN USED

TO PROTECT THE INNOCENT ) AND THEY REFER TO A 16K CP/M VERSION OF CP/M

VERSION 1.4 ON A STANDARD 8 INCH SINGLE DENSITY IBM FORMATTED DISC.

I.  "AUTO LOAD" FEATURE

CP/M 1.4 HAS THE ABILITY TO AUTOMATICALLY EXECUTE A PRE-SELECTED

COMMAND AT THE END OF A COLD OR WARM BOOT STARTUP.  THIS FEATURE

IS NORMALLY USED TO COME UP WITH A SIGN-ON MESSAGE OR TO LOAD A SPECIFIC

PROGRAM (SUCH AS BASIC). IT COULD ALSO BE USED AS A DIRTY TRICK BY MAKING

IT LOAD A DISC FORMATTER OR ERASING PROGRAM.

THE BASIC IDEA IS VERY SIMPLE.  ON COLD BOOT, THE  GOCPM   ROUTINE

(LOOK AT A COPY OF THE BIOS) JUMPS INTO CP/M AT THE START OF  CCP

(AT 2900H).  THIS ENTRY POINT FORCES A 'LOG-ON' OF DRIVE  'A' AND

AN EXECUTION OF ANY SINGLE LINE COMMAND THAT IS IN THE LINE INPUT BUFFER.

THE STANDARD VERSIONS OF CP/M HAVE THIS INPUT BUFFER SET TO THE EMPTY

STATE, SO THE STANDARD PROMPT IS DISPLAYED (A>) AFTER LOG-ON.  SOME OF

THE RELEVANT CODE IS SHOWN BELOW:

2900H:	CCPBASE:	JMP START  ;USUAL GOCPM ENTRY

JMP START/CLEAR   ;ALTERNATE ENTRY TO CLEAR

;					   BUFFER FIRST

DB  7FH    ;MAXIMUM INPUT BUFFER SIZE

BUFLEN:		DB  00	   ;BUFFER CHARACTER COUNTER

DS  16	   ;SPACE FOR 16 LETTERS

DB  "COPYRIGHT ..."  ;COPYRIGHT NOTICE (CAN

;					     ; BE WRITTEN OVER)

FOR EXAMPLE:

TO MAKE THE COLD AND WARM BOOT UP SEQUENCES LOAD BASIC FROM

DRIVE B, YOU NEED ONLY PATCH THE ABOVE CODE TO READ:

BUFLEN:		DB  7   ;COMMAND LENGTH

DB "B:BASIC" ;7 LETTER COMMAND TO LOAD

;					FILE: BASIC.COM.

DB 00H,0DH   ;COMMAND TERMINATORS

YOU CAN EASILY DO THIS WITH  SYSGEN  AND  DDT.  IF YOU ONLY WANT THE

COMMAND TO EXECUTE ON A COLD START AND NOT ON A WARM BOOT (AS CAUSED

BY A CONTROL-C), YOU MUST ALSO MODIFY YOUR  BIOS  SO THAT IT JUMPS

TO CCPBASE ON A COLD-START-- BUT CCPBASE+3 ON A WARM START.  THIS

ENTRY POINT CLEARS THE LINE BUFFER BEFORE ENTERING THE COMMAND

PROCESSOR.

II.  CP/M SERIAL NUMBERS

I WAS GOING TO GIVE THIS INFORMATION IN CYPHER--BUT THE TRAPDOOR

CODE IS TOO COMPLICATED.  THUS I WILL GIVE YOU A HINT:  THE SERIAL

NUMBERS OCCURE IN TWO IDENTICAL BLOCKS OF 6 BYTES OF CODE IN CP/M

(IN BOTH THE CCP AND BDOS).  THESE SAME TWO BLOCKS OCCURE IN THE

RELOCATED IMAGE OF CP/M CONTAINED IN THE PROGRAM: MOVCPM.COM

(THE CP/M RELOCATOR).  IF YOU CAN'T FIND THEM KNOWING THIS, YOU

DON'T DESERVE TO KNOW ABOUT THEM.

III.  TABLEIZED CONTROL DATA FOR CP/M

THE AUTHORS OF CP/M 1.4 DECIDED TO MAKE IT EASIER FOR  OEM'S

TO PERSONALIZE CP/M (FOR VARIOUS TYPES OF HARDWARE) BY PUTTING

A NUMBER OF PIECES OF CONTROL DATA IN TABLES OR SINGLE ROUTINES.

THE BULK OF THESE ARE CONTAINED IN BDOS.  THESE INCLUDE THE SECTOR

INTERLACE TABLE AND PROCESSING ROUTINE, THE SYSTEM ERROR ROUTINE

ADDRESS TABLE, AND THE DISC SIZE SPECIFICATION BYTES.

THIS IS SHOWN BELOW:

3100H:		DS 6   ;SERIAL # DATA.

3106H:	BDOS:	JMP  ENTRY  ; THIS IS WHERE YOU ENTER WHEN YOU

;		      CALL THE VARIOUS CP/M FUNCTIONS

;		      THRU A CALL TO ADDRESS 05H.

; --- TRANSFER TABLE FOR DISC ERRORS

3109H:		DW  PERSUB ;PERMANENT DISC ERROR ROUTINE

DW  SELSUB ;SELECT ERROR ROUTINE

DW  ROERR  ;RO (READ ONLY) ERROR ROUTINE

;

; --- ROUTINE TO TRANSLATE LOGICAL TO PHYSICAL SECTOR #

;     BY TABLE LOOKUP.  ENTER WITH REQUESTED LOGICAL SECTOR

;     # IN (C), EXIT WITH PHYSICAL SECTOR # IN (C).

;

310FH:	TRANS:	LXI H,TABLE	;POINT TO TABLE

MVI B,0

DAD B

MOV C,M		;CONVERT

JMP SELSEC

NOP

;  TRANSLATION TABLE

311AH:	TABLE:	DB 01H,07H,0DH,13H,19H,05H,0BH,11H

DB 17H,03H,09H,0FH,15H,02H,08H,0EH

DB 14H,1AH,06H,0CH,12H,18H,04H,0AH,10H,16H

;--- 6 EXTRA BYTES

DB 00H,00H,00H,00H,00H,00H

;

; --- DISC SIZE PARAMETER BLOCK

;

313AH:	SECSIZ:	DB 26  ;NUMBER OF SECTORS/TRACK

DIRMAX: DB 63  ;MAX. NUMBER OF DIRECTORY ENTRYS

BLKSHF: DB 3   ;2^3=8=NUMBER OF SECTORS/BLOCK

BLKMSK: DB 7   ;2^BLKSHF-1

MAXBLK: DB 242 ;# OF ALLOCATED BLOCKS ON DISC

DIRALL: DB 0C0H; FIRST BYTE OF ALLOCATION MAP TO RESERVE

;		 TWO BLOCKS FOR DIRECTORY.

OPTRKS: DB 2   ;NUMBER OF TRACKS USED TO SAVE BOOT AND

;		OPERATION SYSTEM.

;------------------------------------------------------

NOTE: IF YOU TRY CHANGING THE DISC LAYOUT BY ALTERING THE

ABOVE TABLE, YOU MAY HAVE TO MAKE THE PATCH SHOWN BELOW.  THIS PATCH

ACTUALLY FIXES A BUG IN CP/M 1.4 WHICH YOU WOULD NOT SEE UNTIL YOU

TRIED TO CONFIGURE IT FOR A DIFFERENT DISC LAYOUT (THEY FORGOT TO MAKE

ONE ROUTINE USE THE TABLE DATA FOR THE DISC LAYOUT).

3972:	CALL	33F4H	;USE ROUTINE THAT DOES IT RIGHT

NOP

NOP		;PATCH OUT UNUSED CODE

NOP

3978:	MOV	A,C	;GET BLOCK # POSITION

3979:	LXI	H,3DF8H

IV.  ALLOCATION MAP AND DIRECTORY CHECKSUM TABLE

WHEN YOU DO A WARM BOOT OR SELECT A NEW DRIVE FOR THE FIRST TIME

SINCE THE LAST WARM BOOT, THERE IS A NOTICABLE DELAY WHILE CP/M DOES

SOMETHING ON THE DISC (BEFORE IT ISSUES THE PROMPT).  THIS DELAY

IS CAUSED BY 3 BASIC OPERATIONS BEING DONE BY CP/M:

1.  RE-BOOTING IN CP/M FROM THE DISC.

2.  CONSTRUCTING AN ALLOCATION MAP FOR THE DISC (BY

READING THE DIRECTORY).

3.  CHECKSUMMING THE DIRECTORY.

CP/M 1.4 MAINTAINS SEPARATE ALLOCATION MAPS (31 BYTES EACH) AND

CHECKSUM TABLES (16 BYTES EACH) FOR EACH OF THE FOUR POSSIBLE

DISC DRIVES.

WHAT IS AN ALLOCATION MAP I HERE YOU ASK?  IT IS ESSENTIALLY

A DIRECTORY OF ALL OF THE USED (ALLOCATED) AND UNUSED 1K BLOCKS ON THE

GIVEN DISC.  CP/M USES THIS TABLE TO FIND OUT WHICH BLOCKS ARE AVAILABLE

WHEN YOU TRY TO SAVE A NEW FILE ON THE DISC.  IT CONSTRUCTS THIS TABLE

BY LOOKING AT THE ALL OF THE ALLOCATION BYTES IN THE VARIOUS FILE CONTROL

BLOCKS SAVED IN THE DISC DIRECTORY.  THIS IS DONE THE FIRST TIME YOU

SELECT A NEW DRIVE (I CALL THIS A LOGON OPERATION).  IT DOES NOT DO THIS

EACH TIME YOU SELECT THE DRIVE (TO SAVE TIME), BUT ONLY THE FIRST TIME

AFTER EACH WARM OR COLD BOOT (OR BDOS RESET).  THUS THERE IS A POSSIBILITY

OF YOU REALLY MAKING A BAD MISTAKE IF YOU SWITCH DISCETTES ON A DRIVE

THAT HAS ALREADY BEEN LOGGED IN BECAUSE THE ALLOCATION MAP NO LONGER

IS CORRECT FOR THIS DRIVE.  FOR INSTANCE, IF YOU TRIED TO SAVE A FILE

ON THIS DRIVE, IT MIGHT (AND PROBABLY WOULD) USE A 1K BLOCK THAT WAS

AVAILABLE ON THE PREVIOUS DISCETTE- BUT NOT SO ON THE PRESENT ONE (THUS

OVERWRITING PART OF ANOTHER FILE).  THIS ERROR COULD ACTUALLY OCCURE

UNDER CP/M 1.3.  TO FIX THIS IN CP/M 1.4, THE AUTHORS CREATED CHECKSUM

TABLES WHICH PROVIDE A 1 BYTE CHECKSUM FOR EACH OF THE SECTORS OF THE

DIRECTORY- FOR EACH OF THE FOUR DRIVES.  THE INTERNAL MACHINERY OF CP/M

1.4 TESTS THESE CHECKSUMS EACH TIME A SECTOR OF THE DIRECTORY IS ACCESSED

AND WILL SET THE DRIVE TO READ-ONLY STATUS IF THE CHECKSUMS CHANGE

UNEXPECTEDLY.

THE ALLOCATION MAPS CAN BE FOUND (IN A 16K CP/M 1.4) STARTING AT:

3CFA, 3D1A, 3D3A, AND 3D5A

THE FORMAT IS TO USE ONE BIT FOR THE ALLOCATION OF EACH 1K BLOCK OF

THE DISC THAT IS ACCESSABLE FOR FILE OPERATIONS (TRACKS: 2-76).

THE BLOCK IS ALLOCATED (IN USE) IF ITS CORRESPONDING MAPPING BIT IS

A ONE AND AVAILABLE FOR USE IF IT IS A ZERO.  A NEW DISC (OR AN EMPTY

ONE) HAS ONLY TWO BLOCKS ALLOCATED ( 0,1) .  THIS PREVENTS THE

DIRECTORY (THE FIRST TWO BLOCKS) FROM BEING OVERLAYED.  SUCH A DISC

WOULD HAVE AN ALLOCATION MAP LIKE:

3CFA:	DB 0C0H	;ALLOCATE TWO BLOCKS FOR DIRECTORY

DB 00

DB 00

.  .

.  .

.  .

DB 00

NOTE: BDOS FUNCTION CODE # 27 IS USED TO ACCESS THIS MAP AND SOME

PROGRAMS IN THE CP/M USER GROUP DISCS (SUCH AS DUMP.COM ON DISC 24)

WILL GIVE A NICE PRINT OUT OF IT ON THE CONSOLE.

V.  A PATCH TO FIX THE R/O ERROR SEQUENCE IN CP/M 1.4

CP/M 1.4 HAS THE ANNOYING HABIT OF AUTOMATICALLY MAKING A PREVIOUSLY

LOGGED ON DISC DRIVE READ ONLY (R/O) IF YOU SWITCH DISCS IN THAT DRIVE

(WITHOUT LOGGING ON THE DRIVE AGAIN).  THIS CAN BE PARTICULARLY ANNOYING

IF YOU ONLY HAVE A SINGLE DRIVE.  THIS IS NOT REALLY A BUG IN CP/M.  THIS

IS A WAY OF AVOIDING A POSSIBLY DISASTEROUS CONDITION (AS DESCRIBED ABOVE).

HOWEVER, I DO NOT COMPLETELY AGREE WITH THEIR WAY OF HANDLING THIS ERROR

CONDITION.  I THINK THAT IN MOST SITUATIONS, A BETTER CHOICE IS TO

AUTOMATICALLY RE-LOGON THE NEW DISCETTE WHEN THE ERROR IS DETECTED (BEFORE

ANY DISC WRITE OPERATIONS).  THE PATCH SHOWN BELOW IMPLEMENTS THIS CHOICE.

WARNING: IF ANY DISC FILES ARE STILL OPEN WHEN THE DISCS ARE SWITCHED YOU

CAN STILL GET INTO TROUBLE- SO WATCH OUT!!

OLD CODE			NEW CODE

34C6:	CALL	34AE		CALL	34AE

RAR			RAR

JNC	34D5		JNC	34D5

LXI	H,34D5	   --   JMP	35E0	;LOGON DRIVE

PUSH	H		PUSH	H

LHLD	310D		LHLD	310D

PCHL			PCHL

34D5:	RET			RET

VI.  THE ET BYTE (FIRST BYTE OF THE FCB)

THE FIRST BYTE OF A FCB (WHEN USED FOR DISC FILE OPERATIONS)

IS USUALLY SPECIFIED IN THE CP/M MANUALS AS A ZERO.  HOWEVER IT IS

ACTUALLY USED BY CP/M TO PERFORM AUTOMATIC DRIVE SELECTS DURING

FILE ACCESS.  IT SPECIFIES WHICH DRIVE THE SPECIFIED FILE IS ON:

ET BYTE=0	FILE ON CURRENTLY SELECTED DRIVE

1	FILE IS ON DRIVE A

2	FILE IS ON DRIVE B

3	FILE IS ON DRIVE C

4	FILE IS ON DRIVE D

THIS PARTICULARLY USEFUL WHEN USING   DDT.   THIS IS BECAUSE

THE INPUT OPERATION IN DDT WILL NOT ACCEPT THE USUAL CP/M SYNTAX:

-IB:FILE.HEX		(IT DOESN'T LIKE THE   B:  PART)

HOWEVER YOU CAN NOW DO THE SAME THING BY FIRST USING THE INPUT

COMMAND AND THEN SETTING THE ET BYTE OF THE FCB (AT 05CH) WITH

THE DDT SET COMMAND:

-IFILE.HEX

-S5C

005C 00 02

005D 46 .

-

VII.  RE-ENTRY POINTS TO CP/M AND DDT

OCCASIONALLY I FIND IT USEFUL (WHEN POKING AROUND IN MEMORY)

TO BE ABLE TO RE-ENTER CP/M OR DDT (WHEN LOADED) WITHOUT DOING A

WARM BOOT (JUMP 00H) .  AFTER LOOKING AROUND IN CP/M 1.4 AND  DDT

I FOUND SOME POINTS THAT SEEM TO WORK OK.  I FOUND THEM BY LOOKING

AT HOW THE PROGRAMS GET INITIALIZED AND WHERE THEY RESET THE STACK

POINTER.

16K CP/M 1.4 RE-ENTRY POINT: 3037H

3037:	LXI	SP,3085

LDA	30C9	;GET DRIVE #

CALL	29B8	;LOG IN THIS DRIVE #

JMP	2C71	;PRINT PROMPT, START CP/M.

NOTE:  WHEN DDT IS LOADED, CP/M OVERLAYS PART OF THIS CODE AND

PUTS A  RST 00  AT  3037H.

RE-ENTRY POINTS FOR DDT

I FOUND TWO POINTS THAT SEEM TO WORK OK IN  DDT.  HOWEVER, SINCE

DDT SITS AT A MEMORY LOCATION THAT IS DEPENDENT ON YOUR CP/M SIZE, I

WILL HAVE TO GIVE THE LOCATIONS ON A RELATIVE BASIS.  IN YOU SYSTEM,

LOAD  DDT  AND DISASSEMBLE THE JUMP AT LOCATION  05H:

A>DDT

DDT VER 1.4

-L5

0005  JMP 2100       --- CALL THIS JUMP ADDRESS : ADDX

THE ADDRESS REFERENCED BY THE JUMP AT  0005H WILL BE USED AS THE BASIS

FOR THE TWO RE-ENTRY POINTS:

ADDX+06FEH

ADDX+06D2H

---------------------------------------------------------------------

BY  BRUCE KENDALL

##############################################################