TASM USER'S MANUAL

TASM - A Table Driven Cross Assembler for the MSDOS* Environment

Thomas N. Anderson

Speech Technology Incorporated

837 Front Street South, Issaquah, WA   98027

March, 1992

Version 2.9

[Speech Technology Incorporated  manufactures electronic devices  to aid  the

visually impaired employing digital speech synthesis technology.]

TASM software is Copyright (C)  1985-1992 by Speech Technology  Incorporated.

All rights reserved.

This document is Copyright (C)  1985-1992 by Speech Technology  Incorporated.

All rights reserved.  Permission is granted to copy this document and related

software except  for  the source  code.    The source  code,  distributed  to

registered users, may be copied for the sole use of the registered user.

* MSDOS is a trademark of Microsoft Corporation.

TASM - Table Driven Assembler Version 2.9                              Page 2

TABLE OF CONTENTS

SECTION                                             PAGE

______________________________________________________________________

INTRODUCTION                                           4

INVOCATION                                             5

File Names                                            5

Option: a - Assembly Control                          6

Option: b - Binary Object Format                      7

Option: c - Contiguous Block Output                   7

Option: d - Define a Macro                            7

Option: e - Expand Source                             7

Option: f - Fill Memory                               7

Option: g - Object File Format                        7

Option: h - Hex Object Code Table                     7

Option: i - Ignore Case in Labels                     8

Option: l - Label Table                               8

Option: m - MOS Technology Object Format              8

Option: o - Set Number of Bytes per Object Record     8

Option: p - Page Listing File                         8

Option: q - Disable Listing File                      8

Option: r - Set Read Buffer Size                      8

Option: s - Enable Symbol File Generation             9

Option: t - Table Name                                9

Option: x - Enable Extended Instruction Set          10

Option: y - Enable Assembly Timing                   10

ENVIRONMENT VARIABLES                                 11

TASMTABS                                             11

TASMOPTS                                             11

EXIT CODES                                            11

SOURCE FILE FORMAT                                    12

EXPRESSIONS                                           13

Labels                                               13

Numeric Constants                                    13

Character Constants                                  14

String Constants                                     14

Location Counter Symbol                              14

Operators                                            15

ASSEMBLER DIRECTIVES                                  17

ADDINSTR                                             17

AVSYM                                                17

BLOCK                                                17

BSEG/CSEG/DSEG/NSEG/XSEG                             17

BYTE                                                 17

CHK                                                  18

CODES/NOCODES                                        18

DB                                                   18

DW                                                   18

DEFINE                                               18

DEFCONT                                              19

EJECT                                                19

ELSE                                                 20

END                                                  20

ENDIF                                                20

EQU                                                  20

EXPORT                                               21

TASM - Table Driven Assembler Version 2.9                              Page 3

IFDEF                                                21

IFNDEF                                               21

IF                                                   22

INCLUDE                                              22

LIST/NOLIST                                          22

LOCALLABELCHAR                                       22

LSFIRST/MSFIRST                                      22

ORG                                                  22

PAGE/NOPAGE                                          23

SET                                                  23

SYM/AVSYM                                            23

TEXT                                                 24

TITLE                                                25

WORD                                                 26

OBJECT FILE FORMATS                                   27

Intel Hex Object Format                              27

MOS Technology Hex Object Format                     28

Motorola Hex Object Format                           28

Binary Object Format                                 28

LISTING FILE FORMAT                                   30

PROM PROGRAMMING                                      31

ERROR MESSAGES                                        32

BUGS AND LIMITATIONS                                  35

APPENDIX A - ORDERING INFORMATION                     36

TASM - Table Driven Assembler Version 2.9                              Page 4

INTRODUCTION

TASM is a table driven cross assembler for the MS-DOS environment.   Assembly

source code, written in the appropriate dialect (generally very close to  the

manufacturers assembly  language),  can  be  assembled  with  TASM,  and  the

resulting object code  transferred to  the target  microprocessor system  via

PROM or other mechanisms.

The current microprocessor families supported by TASM are:

6502                       8048

6800/6801/68HC11           8051

6805                       8080/8085

TMS32010/TMS320C25         Z80

TMS7000

The user  so  inclined  may  build  tables  for  other  microprocessors.  The

descriptions of the various existing tables and instructions on building  new

tables are not in this document but can be found in the TASMTABS.DOC file  on

the TASM distribution disk.

TASM characteristics include:

1.  Powerful expression parsing (17 operators).

2.  Supports a subset of the 'C' preprocessor commands.

3.  Macro capability (through use of DEFINE directive).

4.  Multiple statements per line.

5.  Four object file formats:

Intel hex, MOS Technology hex, Motorola hex, binary.

6.  Absolute code generation only.

7.  Source code available (in C).

8.  Uniform syntax  across  versions  for  different  target machines.

9.  Features in support of PROM programming (preset memory,

contiguous block).

10. Supports extended instructions for many of the supported

microprocessor families.

11. Tables read at run time; single TASM executable for all table versions.

12. Symbol table export for inclusion in subsequent assemblies.

13. Symbol table export file compatible with the Avocet 8051

simulator (.SYM format).

SHAREWARE

TASM is distributed  as shareware.    TASM is  not public domain.   The  TASM

distribution files may be freely copied (excluding the source code files) and

freely used for the purpose of evaluating the suitability of TASM for a given

purpose.   Use  of  TASM  beyond  a  reasonable  evaluation  period  requires

registration.   Prolonged use without registration is unethical.

TASM - Table Driven Assembler Version 2.9                              Page 5

INVOCATION

TASM can be invoked as follows  (optional fields shown in brackets,  symbolic

fields in italics):

tasm -pn [-option_flag ...] src_file

[obj_file [lst_file [exp_file [sym_file]]]]

Where  option_flag can be one or more of the following:

-table        Specify version (table = table designation)

-ttable       Table (alternate form of above)

-aamask       Assembly control (optional error checking)

-b            Produce object in binary (.COM) format

-c            Object file written as a contiguous block

-dmacro       Define a macro (or just a macro label)

-e            Show source lines after macro expansion

-ffillbyte    Fill entire memory space with fillbyte (hex)

-gobjtype     Object file (0=Intel, 1=MOS Tech, 2=Motorola, 3=binary)

-h            Produce hex table of the assembled code (in list file)

-i            Ignore case for labels

-l[al]        Produce a label table in the listing

-m            Produce object in MOS Technology format

-oobytes      Bytes per object record (for hex obj formats)

-p[lines]     Page the listing file (lines per page. default=60)

-q            Quiet, disable the listing file

-rkb          Set read buffer size in Kbytes (default 2 Kbytes)

-s            Write a symbol table file

-x[xmask]     Enable extended instruction set (if any)

-y            Time the assembly

The file parameters are defined as follows:

src_file  Source file name

obj_file  Object code file name

lst_file  Listing file name

exp_file  Symbol export file (only if the EXPORT directive is used).

sym_file  Symbol table file

(only if the "-s" option or the SYM/AVSYM directives are used).

The source  file  must be  specified.   If  not,  some usage  information  is

displayed.  Default file names for all the other files are generated if  they

are not explicitly  provided.  The  filename is formed  by taking the  source

filename and changing the extension to one of the following:

Extension           File type

________________________________________________________

.OBJ                Object file

.LST                Listing file

.EXP                Symbol export file

.SYM                Symbol table  file

TASM has  no built-in  instruction set  tables.   Instruction set  definition

files are read at run time.  TASM determines which table to use based on  the

'-table' field shown  above.  For  example, to  assemble the code  in a  file

called source.asm, one would enter:

TASM - Table Driven Assembler Version 2.9                              Page 6

tasm -48   source.asm            for an 8048 assembly

tasm -65   source.asm            for a  6502 assembly

tasm -51   source.asm            for an 8051 assembly.

tasm -85   source.asm            for an 8085 assembly.

tasm -80   source.asm            for a  Z80  assembly.

tasm -05   source.asm            for a  6805 assembly.

tasm -68   source.asm            for a  6800/6801/68HC11 assembly.

tasm -70   source.asm            for a  TMS7000  assembly.

tasm -3210 source.asm            for a  TMS32010 assembly.

tasm -3225 source.asm            for a  TMS320C25 assembly.

The file name that the  tables are read from is  formed by taking the  digits

specified after the '-' and appending it to 'TASM' then appending the  '.TAB'

extension.  Thus, the '-48' flag would cause  the tables to be read from  the

file 'TASM48.TAB'.

It is possible to designate  tables by non numeric  part numbers if the  '-t'

flag is used.  For  example, if a user built  a table called TASMF8.TAB  then

TASM could be invoked as follows:

tasm -tf8 source.asm

Each option flag must be  preceded by a dash.   Options need not precede  the

file names, however.  The various options are described below:

a  -  Assembly  Control.  TASM  can  provide  additional  error  checking  by

specifying the '-a' option at the time of execution.  If the '-a' is provided

without a digit following, then all the available error checking is done.  If

a digit follows, then it is used as  a mask to determine the error checks  to

be made.  The bits of the mask are defined as follows:

Bit    Option Flag   Description

___________________________________________________________________________

0      -a1           Check for apparent illegal use of indirection

1      -a2           Check for unused data in the arguments

2      -a4           Check for duplicate labels

3      -a8           Check for non-unary operators at start of expression.

4      -a10          Check for 8051 AJMP/ACALL branches off current 2K page.

Combinations of the above bits  can also be used.   For example, '-a5'  would

enable the checking for illegal indirection and duplicate labels.

Illegal indirection applies to micros that use parenthesis around an argument

to indicate indirection.  Since  it is always legal to  put an extra pair  of

parenthesis around  any  expression  (as  far as  the  expression  parser  is

concerned), the user may think that  he/she is indicating indirection for  an

instruction that has no  indirection and TASM would  not complain.   Enabling

this checking will  result in an  error message (warning)  whenever an  outer

pair of parenthesis is used and the instruction set definition table does not

explicitly indicate that to be a valid form of addressing.

Unused data in  arguments applies to  cases where  a single byte  of data  is

needed from an  argument, but  the argument contains  more than  one byte  of

data.  If  a full  sixteen bit address  is used  in a  'Load Immediate'  type

instruction that needs  only a  single byte,  for example,  an error  message

would be generated.  Here is an example (6502 code):

TASM - Table Driven Assembler Version 2.9                              Page 7

0001   1234                    .org $1234

tasm: Unused data in MS byte of argument.  Line 0002 in test.asm

0002   1234 A9 34       start  lda  #start

To make the above checks occur whenever  you do an assembly, add a line  like

this to your AUTOEXEC.BAT file:

SET TASMOPTS=-a

b  -  Binary Object Format. This option causes the object file to be  written

in binary  - one  byte for  each byte  of code/data.   Note  that no  address

information is included in  the object file in  this format.  The  contiguous

block (-c) output mode is forced when  this option is invoked.  This flag  is

equivalent to '-g3'.

c - Contiguous Block Output. If this  option is specified, then all bytes  in

the range from the  lowest used byte  to the highest will  be defined in  the

object file.  Normally, with the default Intel Hex object format enabled,  if

the Program Counter  (PC) jumps  forward because  of an  .ORG directive,  the

bytes skipped over will not have any value assigned them in the object  file.

With this option enabled, no output to  the object file occurs until the  end

of the assembly at  which time the whole  block is written.   This is  useful

when using TASM to  generate code that will  be put into a  PROM so that  all

locations will have a known value.  This option is often used in  conjunction

with the -f option to ensure all unused bytes will have a known value.

d - Define  a Macro.  Macros are  defined on  the command  line generally  to

control the assembly of various IFDEF's that are  in the source  file.   This

is a convenient way to generate various versions of object code from a single

source file.

e - Expand Source. Normally TASM shows lines in the listing file just as they

are in the source file.  If macros  are in use (via the DEFINE directive)  it

is sometimes desirable to see the source lines after expansion.  Use the '-e'

flag to accomplish this.

f   -   Fill   Memory.   This   option   causes the  memory image  that  TASM

maintains to be initialized to the value specified by the two hex  characters

immediately following the 'f'.  TASM maintains a memory image that is a  full

64K bytes in size (even if the  target processor cannot  utilize that  memory

space).  Invocation of this option introduces a delay at start up of up to  2

seconds (time required to initialize all 64K bytes).

g -  Object File  Format. TASM  can generate  object code  in four  different

formats as indicated below:

-g0    for Intel hex (default)

-g1    for MOS Technology hex (same as -m)

-g2    for Motorola hex

-g3    for binary (same as -b)

The '-m' and '-b' flags may also be used, as indicated above.

See the section on OBJECT FILE FORMATS for descriptions of each of the above.

TASM - Table Driven Assembler Version 2.9                              Page 8

h - Hex Object  Code Table. This option  causes a hex  table of the  produced

object code to  appear in the  listing file.   Each line of  the table  shows

sixteen bytes of code.

i - Ignore Case in Labels.  TASM is normally case sensitive when dealing with

labels.  For  those that  prefer case  insensitivity, the  '-i' command  line

option can be employed.

l - Label Table.  This option causes  a label table to appear in the  listing

file.  Each  label is shown  with its corresponding  value. Macro labels  (as

established via the DEFINE directives) do not appear.

Two optional suffixes may follow the '-l' option:

Suffix          Description

_______________________________________________

l               Use long form listing

a               Show all labels (including local labels)

The suffix should immediately follow the '-l'.  Here are some examples:

-l      to show non-local labels in the short form

-la     to show all labels in the short form

-ll     to show non-local labels in the long form

-lal    to show all labels in the long form

m - MOS Technology Object  Format. This option causes  the object file to  be

written in  MOS Technology  hex  format rather  than  the default  Intel  hex

format.  See section on OBJECT FILE FORMATS for a description of the format.

o - Set Number of Bytes per Object Record.  When  generating  object code  in

either the MOS Technology format  or the Intel hex format,   a default of  24

(decimal) bytes of object are defined on each  record. This can be altered by

invoking the '-o' option  immediately followed by   two  hex digits  defining

the number of bytes per record desired. For example,  if 32 bytes per  record

are desired,  one might  invoke TASM as:

TASM -48 -o20 source.asm

p - Page Listing  File. This option  causes the listing file  to have top  of

page headers and form  feeds inserted at  appropriate intervals (every  sixty

lines of output).  To override the default of sixty lines per page,  indicate

the desired number of lines per page as a decimal number immediatly following

the '-p'.  Here is an example:

TASM -48 -p56 source.asm

q - Disable Listing File.  This option causes all output to the listing  file

to be  suppressed, unless a .LIST directive is encountered in the source file

(see LIST/NOLIST directives).

r - Set Read Buffer Size. This option overrides the default read buffer  size

of 2 Kbytes.  The first hexadecimal digit immediately after the 'r' is  taken

as the  number of  K  bytes to  allocate for  the  read buffer  (.e.g.  '-r8'

TASM - Table Driven Assembler Version 2.9                              Page 9

indicates an 8K byte buffer, '-rf' indicates  a 15K byte buffer).  Note  that

that read buffers are  taken from the  same memory pool  as labels and  macro

storage, and that additional read buffers are needed if "includes" are  used.

Thus, using 8K byte buffers may be suitable for most assemblies, but programs

with large numbers of symbols may not allow such a value.  Also, reducing the

buffer size  to 1  Kbyte can  increase the  memory pool  available for  label

storage, if such is needed.

s - Enable Symbol File Generation.    If this flag  is set, a symbol file  is

generated at the end of the assembly.  The format of the file is one line per

label, each label starts in the first  column and is followed by white  space

and then four hexadecimal digits representing the  value of the label.    The

following illustrates the format:

label1         FFFE

label2         FFFF

label3         1000

The symbol  file name  can be  provided as  the fifth  file name  on the  the

command line, or the name will be generated from the source file name with  a

'.SYM' extension.  The symbol table file  can also be  generated by  invoking

the SYM directive.  The AVSYM directive also generates the symbol file but in

a different format (see section on ASSEMBLER DIRECTIVES).

t - Table Name.  As an alternative to specifying the instruction set table as

two decimal digits, the table indication may be preceeded by the '-t' option.

This is useful if the desired table name starts with a non-numeric.  Thus,  a

table for an F8 might be selected as:

TASM -tf8 source.asm

TASM would expect to read the  instruction set definition tables from a  file

named TASMF8.TAB.

TASM - Table Driven Assembler Version 2.9                             Page 10

x - Enable Extended Instruction Set.  If a processor family has  instructions

that are valid for only  certain members, this option  can be used to  enable

those beyond the basic standard instruction set.  A hex digit may follow  the

'x' to indicate a  mask value used in  selecting the appropriate  instruction

set.  Bit 0 of the mask selects the basic instruction set, thus a '-x1' would

have  no  effect.    A  '-x3'  would  enable  the  basic  set  plus  whatever

instructions have bit  1 set in  their class mask.   A '-x'  without a  digit

following is equivalent to a '-xf' which sets all four of the mask bits.  The

following table indicates the current extended instruction sets available  in

the TASM tables:

Base   Base           Extension 1   Extension 2  Extension 3    Extension 4

Table  Family         (-x3)         (-x7)        (-x5)          (-x9)

_____________________________________________________________________________

48     8048           8041A                      8022           8021

65     6502           R65C02                     R65C00/21

05     6805           M146805 CMOS               HC05C4

80     Z80            HD64180

68     6800           6801/6803     68HC11

51     8051

85     8080

3210   TMS32010

3225   TMS320C25                                 TMS320C26

70     TMS7000

The above  table does  not attempt  to show  the many  microprocessor  family

members that may apply under a given column.

See the TASMTABS.DOC on-line document for details on each specific table.

y - Enable Assembly Timing.  If  this option is enabled TASM will generate  a

statement of elapsed time and  assembled lines per second  at the end of  the

assembly.

TASM - Table Driven Assembler Version 2.9                             Page 11

ENVIRONMENT VARIABLES

The TASM  environment can  be customized  by using  the enviroment  variables

listed below:

TASMTABS.  This  variable  specifies  the  path  to  be  searched  for   TASM

instruction set definition tables.   If it is  not defined then the  table(s)

must exist in the current  working directory.  If it  was desired to put  the

instruction set definition  tables in a  directory called 'TASM'  on a  drive

called 'C:', the following statement would be appropriate in the AUTOEXEC.BAT

file:

set TASMTABS=C:\TASM

TASMOPTS. This variable specifies  TASM command line options  that are to  be

invoked every time TASM is executed.  For example, if TASM is being used  for

8048 assemblies  with  binary  object  file  output  desired,  the  following

statement would be appropriate in the AUTOEXEC.BAT file:

set TASMOPTS=-48 -b

EXIT CODES

When TASM terminates, it will return to DOS the following exit codes:

Exit Code      Meaning

___________________________________________________________

0              Normal completion, no assembly errors

1              Normal completion, with assembly errors

2              Abnormal completion, insufficient memory

3              Abnormal completion, file access error

4              Abnormal completion, general error

Exit codes 2 and above  will also be accompanied  by messages to the  console

concerning the error.

TASM - Table Driven Assembler Version 2.9                             Page 12

SOURCE FILE FORMAT

Statements in the  source file must  conform to a  format as follows  (except

for assembler  directive  statements  which are  described  in  a  subsequent

section):

label  operation    operand       comment

All of  the  fields  are  optional,  under  appropriate  circumstances.    An

arbitrary amount of white space (space and tabs) can separate each field  (as

long as the maximum line length of 255 characters is not exceeded).  Each  of

the fields are described below:

Label Field. If the first character of the line is alphabetic, it is  assumed

to be the start of  a label.  Subsequent characters  are accepted as part  of

that label until a space, tab, or ':' is encountered.  The assembler  assigns

a value to the label corresponding  to the current location counter.   Labels

can be a maximum of 32 characters  long.  Labels can contain upper and  lower

case letters, digits, underscores, and  periods (the first character must  be

alphabetic).  Labels are  case sensitive - the  label 'START' is a  different

label from 'start' - unless the '-i' (ignore case) option is enabled.

Operation Field. The operation field  contains an instruction mnemonic  which

specifies the action  to be  carried out by  the target  processor when  this

instruction is executed.  The interpretation of each mnemonic is dependent on

the target microprocessor  (as indicated by  the selected TASM  table).   The

operation field may  begin in  any column except  the first.   The  operation

field is case insensitive.

Operand Field. The operand field specifies the data to be operated on by  the

instruction.  It  may include expressions  and/or special symbols  describing

the addressing mode  to be  used.  The  actual format  and interpretation  is

dependent on  the target  processor.   For a  description of  the format  for

currently supported  processors,  see  the  TASMTABS.DOC  file  on  the  TASM

distribution disk.

Comment Field.  The comment field always  begins with a semicolon.  The  rest

of the line from the semicolon to the end of the line is ignored by TASM, but

passed on to  the listing file  for annotation purposes.   The comment  field

must be the last field on a line,  but it may be the only field, starting  in

column one, if desired.

Multiple  Statement Lines.   If the backslash  character is encountered on  a

source line,   it  is  treated as  a  newline.   The  remainder of  the  line

following  the  backslash  will be processed as an independent line of source

code.  This allows one to put multiple statements  on a line.  This  facility

is not so useful of itself,   but when coupled with  the capability  of   the

DEFINE   directive,    powerful    multiple  statement    macros    can    be

constructed  (see  section  on ASSEMBLER DIRECTIVES).  Note that when   using

the  statement  separator,  the character  immediately  following  it  should

be considered the first character of a new  line,  and thus must either be  a

start of a label or  white space (not an instruction).  As the examples show,

a space is put between the backslash and the start of the next instruction.

Some  examples  of  valid  source  statements follow (6502 mnemonics shown):

TASM - Table Driven Assembler Version 2.9                             Page 13

label1   lda      byte1   ;get the first byte

dec      byte1

jne      label1

;

label2

sta      byte2,X

;        a multiple statement line follows

lda      byte1\ sta byte1+4\ lda byte2\ sta byte2+4

EXPRESSIONS

Expressions are  made  up of  various  syntactic elements  (tokens)  combined

according to  a set  of syntactical  rules.   The  tokens are  summarized  as

follows:

1.  Labels

2.  Constants

3.  Location Counter Symbol

4.  Operators

5.  Parenthesis

Labels. Labels are strings of characters that have a numeric value associated

with them, generally representing an address.   Labels can contain upper  and

lower case letters, digits,  underscores, and periods.   The first  character

must be a  letter or the  local label prefix  (default '_'). The  value of  a

label is  limited  to  32  bit  precision.   Labels  can  contain  up  to  32

characters, all of which are significant (none are ignored when looking at  a

label's value, as in some assemblers).   Case is significant unless the  '-i'

command line option is invoked.

Local labels must  only be  unique within the  scope of  the current  module.

Modules are defined with the MODULE directive.  Here is an example:

.MODULE xxx

lda regx

jne _skip

dec

_skip  rts

.MODULE yyy

lda regy

jne _skip

dec

_skip  rts

In the above example, the _skip label is reused without harm.  As a  default,

local labels are not shown in the label table linting (resulting from the  '-

l' command line  option).   See also  sections on  MODULE and  LOCALLABELCHAR

directives.

Numeric Constants. Numeric constants must  always begin with a decimal  digit

TASM - Table Driven Assembler Version 2.9                             Page 14

(thus hexadecimal constants that  start with a letter  must be prefixed by  a

'0' unless the  '$' prefix is  used).  The  radix is determined  by a  letter

immediately following the digit string according to the following table:

Radix          Suffix                   Prefix

______________________________________________________

2              B or b                   %

8              O or o                   @

10             D or d (or nothing)

16             H or h                   $

Decimal is the default radix, so decimal constants need no suffix or prefix.

The following representations are equivalent:

1234H          or     $1234

100d           or     100

177400O        or     @177400

01011000b      or     %01011000

The prefixes  are provided  for  compatibility with  some other  source  code

formats but  introduce  a  problem of  ambiguity.    Both '%'  and  '$'  have

alternate uses  ('%' for  modulo,  '$' for  location  counter symbol).    The

ambiguity is  resolved  by examining  the  context.   The  '%'  character  is

interpreted as the modulo operator only if it is in a postion suitable for  a

binary operator. Similarly, if the first character following a '$' is a valid

hexadecimal digit, it is assumed to be a radix specifier and not the location

counter.

Character Constants. Character constants are single characters surrounded  by

single quotes  (following  quote  is  optional).   The  ASCII  value  of  the

character in the quotes is returned.  No escape provision exists to represent

non-printable characters within the quotes,  but this is not necessary  since

these can be just  as easily represented as  numeric constants (or using  the

TEXT directive which does allow escapes).

String Constants. String constants are  one or more characters surrounded  by

double quotes.  Note  that string constants are  not allowed in  expressions.

They are  only  allowable  following  the  TITLE,  BYTE  and  TEXT  assembler

directives. The quoted strings  may also contain escape  sequences to put  in

unprintable values.  The following escape sequences are supported:

Escape

Sequence       Description

___________________________________________________________

\n             Line Feed

\r             Carriage return

\b             Backspace

\t             Tab

\f             Formfeed

\\             Backslash

\"             Quote

\000           Octal value of character

Location  Counter  Symbol.   The  current value of the location counter  (PC)

can be  used in  expressions by  placing a  '$' in  the desired  place.   The

TASM - Table Driven Assembler Version 2.9                             Page 15

Location Counter Symbol is allowable anywhere  a numeric constant is.   (Note

that if the '$' is followed  by a decimal digit then it  is  taken to be  the

hexadecimal radix  indicator  instead  of the  Location  Counter  symbol,  as

mentioned above).    The '*'  may  also be  used  to represent  the  location

counter,  but  is  less   preferred  because  of   its  ambiguity  with   the

multiplicative operator.

Operators. Expressions  can  optionally  contain operators  to  perform  some

alterations or  calculations  on  particular  values.     The  operators  are

summarized as follows:

Operator    Type            Description

__________________________________________

+           Additive        addition

-                           subtraction

*           Multiplicative  multiplication

/                           division

%                           modulo

<<                          logical shift left

>>                          logical shift right

~           Unary           bit inversion (one's complement)

-                           unary negation

=           Relational      equal

==                          equal

!=                          not equal

<                           less than

>                           greater than

<=                          less than or equal

>=                          greater than or equal

&           Binary          binary 'and'

|                           binary 'or'

^                           binary 'exclusive or'

The syntax is much the same as in 'C' with the following notes:

1.  No operator precedence  is in effect.  Evaluation  is from left to  right

unless grouped by  parenthesis (see example below).

2.  All evaluations are done with 32 bit signed precision.

3.  Both '=' and '==' are allowable equality checkers.  This is allowed since

the syntax  does not  provide assignment  capability (as  '=' would  normally

imply).

The relational operators return a value of 1 if the relation is true and 0 if

it is false.  Thirtytwo bit signed arithmetic is used.

It is  always  a  good idea  to  explicitly  indicate the  desired  order  of

evaluation with  parenthesis,   especially  to   maintain  portability  since

TASM does  not  evaluate  expressions  in  the  same  manner  as  many  other

assemblers.   To  understand  how it  does  arrive    at   the    values  for

expressions, consider the following example:

TASM - Table Driven Assembler Version 2.9                             Page 16

1 + 2*3 + 4

TASM would evaluate this as:

(((1 + 2) * 3) + 4) = 13

Typical rules of precedence would cause the (2*3) to be evaluated first, such

as:

1 + (2*3) + 4      = 11

To make  sure  you get  the  desired  order of  evaluation,  use  parenthesis

liberally.

Note that  TASM versions  earlier than  2.7.8 employed  a somewhat  different

method of evaluating expressions  (also without precedence) that  essentially

resulted in a right to left evaluation.

Here are some examples of valid expressions:

(0f800H + tab)

(label_2 >> 8)

(label_3 << 8) & $f000

$ + 4

010010000100100b + 'a'

(base + ((label_4 >> 5) & (mask << 2))

TASM - Table Driven Assembler Version 2.9                             Page 17

ASSEMBLER DIRECTIVES

Most of  the  assembler directives  have  a  format similar  to  the  machine

instruction format.    However,  instead of  specifying  operations  for  the

processor to carry out,  the directives cause the  assembler to perform  some

function related to  the assembly process.  TASM has two  types of  assembler

directives - those that mimic the 'C' preprocessor functions, and those  that

resemble the more traditional assembler  directive functions.  Each of  these

will be discussed.

The 'C' preprocessor  style directives are  invoked with a  '#' as the  first

character of the line followed by the appropriate directive (just as in 'C').

Thus, these directives cannot have a label preceding them (on the same line).

Note that in the examples directives are shown in upper case, however, either

upper or lower case is acceptable.

ADDINSTR. This directive can  be used to  define additional instructions  for

TASM to use in this assembly.  The format is:

[label]  .ADDINSTR inst args opcode nbytes modop class shift binor

The fields are  separated by  white space  just as  they would  appear in  an

instruction  definition  file.  See  the   TASMTABS.DOC  file  on  the   TASM

distribution disk for more detail.

AVSYM.  See SYM/AVSYM.

BLOCK. This directive causes the Instruction Pointer to advance the specified

number of bytes without assigning values to the skipped over locations.   The

format is:

[label] .BLOCK        expr

Some valid examples are:

word1   .BLOCK     2

byte1   .block     1

buffer  .block     80

BSEG/CSEG/DSEG/NSEG/XSEG. These  directives can  be invoked  to indicate  the

appropriate address space for  symbols and labels  defined in the  subsequent

code.   The  invocation  of these  directives  in  no way  affects  the  code

generated, only provides  more information in  the symbol table  file if  the

AVSYM directive is employed.   Segment control directives  such as these  are

generally supported  by assemblers  that  generate relocatable  object  code.

TASM does not generate  relocatable object code and  does not support a  link

phase, so these  directives have  no direct  effect on  the resulting  object

code. The segments are defined as follows:

Directive       Segment Description

_________________________________________________________

BSEG            Bit  address

CSEG            Code address

DSEG            Data address (internal RAM)

NSEG            Number or constant (EQU)

XSEG            External data address (external RAM)

TASM - Table Driven Assembler Version 2.9                             Page 18

BYTE.  This directive allows a value assignment to the byte pointed to by the

current Instruction Pointer.  The format is:

[label] .BYTE   expr [, expr ...]

Onlythe lower eight bits of expr are used.  Multiple bytes may be assigned by

separating them with  commas or  (for printable strings)  enclosed in  double

quotes. Here are some examples:

label1   .BYTE     10010110B

.byte     'a'

.byte     0

.byte     100010110b,'a',0

.byte     "Hello", 10, 13, "World"

CHK.  This directive causes  a checksum to be  computed and deposited at  the

current location.    The  starting  point  of  the  checksum  calculation  is

indicated as an argument.  Here is the format:

.CHK    starting_addr

Here is an example:

start:  NOP

LDA #1

.CHK start

The checksum is calculated as the simple arithmetic sum of all bytes starting

at the start_add up to  but not including the  address of the CHK  directive.

The least significant byte is all that is used.

CODES/NOCODES. These directives can be used to alternately turn on or off the

generation of formatted listing output with line numbers, opcodes, data, etc.

With NOCODES  in  effect, the  source  lines are  sent  to the  listing  file

untouched.  This  is useful around  blocks of  comments that need  a full  80

columns of width for clarity.

DB.  This is alternate form of the BYTE directive.

DW.  This is alternate form of the WORD directive.

DEFINE. The DEFINE directive  is one of the  most powerful of the  directives

and allows string substitution with optional arguments (macros).  The  format

is as follows:

#DEFINE  macro_label[(arg_list)]  [macro_definition]

macro_label := string to be expanded when found in the source file.

arg_list    := optional argument list for variable substitution

macro_def   := string to replace the occurrences of macro_label in

TASM - Table Driven Assembler Version 2.9                             Page 19

the source file.

The simplest form of the DEFINE directive might look like this:

#DEFINE         MLABEL

Notice that  no  substitutionary string  is  specified.   The  purpose  of  a

statement like this would typically be to  define a label for the purpose  of

controlling some subsequent conditional assembly (IFDEF or IFNDEF).

A more complicated example, performing  simple substitution, might look  like

this:

#DEFINE         VAR1_LO         (VAR1 & 255)

This statement would cause all occurrences of  the  string  'VAR1_LO' in  the

source to be substituted with '(VAR1 & 255)'.

As   a   more   complicated   example,    using   the   argument    expansion

capability, consider this:

#DEFINE         ADD(xx,yy)      clc\ lda xx\ adc yy\ sta xx

If the source file then contained a line like this:

ADD(VARX,VARY)

It would be expanded to:

clc\ lda VARX\ adc VARY\ sta VARX

The above  example shows  the use  of the  backslash ('\')  character   as  a

multiple   instruction    statement  delimiter.   This  approach  allows  the

definition of fairly  powerful,   multiple statement   macros.   The  example

shown generates 6502 instructions to add one memory location to another.

Some rules associated with the argument list:

1.  Use a maximum of 10 arguments.

2.  Each argument should be a maximum of 15 characters.

Note that macros can be defined on the TASM command line, also, with the '-d'

option flag.

DEFCONT. This directive can be used to  add to the last macro started with  a

DEFINE directive.   This  provides a  convenient way  to define  long  macros

without running off the edge of the page. The ADD macro shown above could  be

defined as follows:

#DEFINE         ADD(xx,yy)     clc

#DEFCONT                     \ lda xx

#DEFCONT                     \ adc yy

#DEFCONT                     \ sta xx

EJECT.   This   directive   can   be used  to force  a top  of form  and  the

TASM - Table Driven Assembler Version 2.9                             Page 20

generation of a page  header on the  list file.  It  has no   effect  if  the

paging mode is off (see PAGE/NOPAGE).  The format is:

.EJECT

ELSE.  This directive  can optionally be  used with IFDEF,  IFNDEF and IF  to

delineate an alternate block of code to be assembled if the block immediately

following the IFDEF, IFNDEF or IF is not assembled.

Here are some examples of the use of IFDEF, IFNDEF,  IF,  ELSE,  and ENDIF:

#IFDEF   label1

lda      byte1

sta      byte2

#ENDIF

#ifdef   label1

lda      byte1

#else

lda      byte2

#endif

#ifndef  label1

lda      byte2

#else

lda      byte1

#endif

#if ($ >= 1000h)

; generate an invalid statement to cause an error

;  when we go over the 4K boundary.

!!! PROM bounds exceeded.

#endif

END.   This  directive should follow all  code/data generating statements  in

the source file.  It forces the last record to be written to the object file.

The format is:

[label]       .END

ENDIF. This directive must  always follow an IFDEF,  IFNDEF, or IF  directive

and signifies the end of the conditional block.

EQU. This directive can be used to  assign values to labels.  The labels  can

then be used in expressions in place of the literal constant.  The format is:

label   .EQU    expr

Here is an example:

MASK   .EQU   0F0H

;

lda    IN_BYTE

and    MASK

sta    OUT_BYTE

TASM - Table Driven Assembler Version 2.9                             Page 21

An alternate form of 'EQU' is  '='.  The previous example is equivalent to:

MASK    =   0F0H

or

MASK    =0FOH

MASK    =$FO

White space must exist after the label,  but none is required after the '='.

EXPORT. This directive can be used to define labels (symbols) that are to  be

written to the export symbol file.  The symbols are written as equates (using

the .EQU  directive)  so  that  the  resulting file  can  be  included  in  a

subsequent assembly.  This feature can help overcome some of the deficiencies

of TASM due to its lack of a relocating linker. The format is:

[label]  .EXPORT      label

The following example  illustrates the use  of the EXPORT  directive and  the

format of the resulting export file:

Source file:

.EXPORT        read_byte

.EXPORT        write_byte

.EXPORT        open_file

Resulting export file:

read_byte      .EQU   $1243

write_byte     .EQU   $12AF

open_file      .EQU   $1301

IFDEF. This directive can be used to optionally assemble a block of code.  It

has the following form:

#IFDEF  macro_label

When invoked, the list of macro labels (established via DEFINE directives) is

searched.  If the label is found, the following lines of code are  assembled.

If not found, the input file is skipped  until an ENDIF or ELSE directive  is

found.

Lines that are skipped over still appear in the listing file, but a '~'  will

appear immediately after the current PC and no object code will be  generated

(this is applicable to IFDEF, IFNDEF, and IF).

IFNDEF. This directive is the opposite of the IFDEF directive.  The block  of

code following is assembled only  if the specified macro_label is  undefined.

It has the following form:

#IFNDEF  macro_label

When invoked, the list of macro labels (established via DEFINE directives) is

TASM - Table Driven Assembler Version 2.9                             Page 22

searched.   If the  label  is not  found, the  following  lines of  code  are

assembled.  If it is found, the input file is skipped until an ENDIF or  ELSE

directive is found.

IF.   This directive  can be  used to  optionally assemble  a block  of  code

dependent on the value of a given expression.  The format is as follows:

#IF     expr

If the expression expr evaluates to non-zero, the following block of code  is

assembled (until an ENDIF or ELSE is encountered).

INCLUDE.  The INCLUDE directive reads  in and assembles the indicated  source

file.  INCLUDEs can  be nested up  to six levels.   This allows a  convenient

means to keep common definitions, declarations, or subroutines in files to be

included as needed.  The format is as follows:

#INCLUDE        filename

The filename must be enclosed in double quotes.  Here are some examples:

#INCLUDE       "macros.h"

#include       "equates"

#include       "subs.asm"

LIST/NOLIST.  These directives can be used to alternately turn the output  to

the list file on (LIST) or off (NOLIST).  The format is:

.LIST

.NOLIST

LOCALLABELCHAR. This directive can be used to override the default "_" as the

label prefix indicating a local label.  For example, to change the prefix  to

"?" do this:

.LOCALLABELCHAR "?"

Be carefull to  use only  characters that  are not  operators for  expression

evaluation.  To do  so causes ambiguity for  the expression evaluator.   Some

safe characters are "?", "{", and "}".

LSFIRST/MSFIRST. These  directives  determine  the  byte  order  rule  to  be

employed for the WORD  directive.  The default  (whether correct or not)  for

all TASM  versions  is the  least  significant  byte first  (LSFIRST).    The

following illustrates its effect:

0000  34 12    .word $1234

0002           .msfirst

0002  12 34    .word $1234

0004           .lsfirst

0004  34 12    .word $1234

ORG. This directive provides the means to set the Instruction Pointer (a.k.a.

Program Counter) to the desired value.  The format is:

TASM - Table Driven Assembler Version 2.9                             Page 23

[label] .ORG    expr

The label is optional.  The Instruction pointer is assigned the value of  the

expression, expr.  For example, to  generate code starting at address  1000H,

the following could be done:

start   .ORG    1000H

The expression  (expr)  may contain  references  to the  current  Instruction

Pointer, thus  allowing various  manipulations to  be done.  For example,  to

align the Instruction Pointer  on the next 256  byte boundary, the  following

could be done:

.ORG  (($ + 0FFH) & 0FF00H)

ORG can also be used to reserve space without assigning values:

.ORG    $+8

An alternate form of ORG  is '*=' or '$='.   Thus the following two  examples

are exactly equivalent to the previous example:

*=*+8

$=$+8

PAGE/NOPAGE.  These  directives can be  used to alternately  turn the  paging

mode on (PAGE) or  off (NOPAGE).  If  paging is in  effect, then every  sixty

lines of output will be  followed by a Top of  Form character and a two  line

header containing page number, filename, and the title.  The format is:

.PAGE

.NOPAGE

The number of lines per page can be set with the '-p' command line option.

SET. This directive allows the value of an existing label to be changed.  The

format is:

label   .SET    expr

The use of the SET directive should be avoided since changing the value of  a

label can sometimes  cause phase  errors between  pass 1  and pass  2 of  the

assembly.

SYM/AVSYM. These directives can be  used to cause a  symbol table file to  be

generated.  The format is:

.SYM    ["symbol_filename"]

.AVSYM  ["symbol_filename"]

TASM - Table Driven Assembler Version 2.9                             Page 24

For example:

.SYM       "symbol.map"

.SYM

.AVSYM     "prog.sym"

.AVSYM

The two  directives are  similar, but  result in  a different  format of  the

symbol table file.  The format of the  SYM file is one line per symbol,  each

symbol starts in the  first column and  is followed by  white space and  then

four hexadecimal digits representing the value of the symbol.  The  following

illustrates the format:

label1         FFFE

label2         FFFF

label3         1000

The AVSYM is provided  to generate symbol tables  compatible with the  Avocet

8051 simulator.  The format is similar, but each line is prefixed by an  'AS'

and each symbol value is prefixed by a segment indicator:

AS     start          C:1000

AS     read_byte      C:1243

AS     write_byte     C:1280

AS     low_nib_mask   N:000F

AS     buffer         X:0080

The segment  prefixes are  determined by  the most  recent segment  directive

invoked (see BSEG/CSEG/DSEG/NSEG/XSEG directives).

TEXT. This directive allows an ASCII string to be used to assign values to  a

sequence of  locations starting  at  the current  Instruction Pointer.    The

format is:

[label] .TEXT   "string"

The ASCII value of each character in string is taken and assigned to the next

sequential location.  Some escape sequences are supported as follows:

Escape

Sequence       Description

___________________________________________________

\n             Line Feed

\r             Carriage return

\b             Backspace

\t             Tab

\f             Formfeed

\\             Backslash

\"             Quote

\000           Octal value of character

TASM - Table Driven Assembler Version 2.9                             Page 25

Here are some examples:

message1   .TEXT   "Disk I/O error"

message2   .text   "Enter file name "

.text   "abcdefg\n\r"

.text   "I said \"NO\""

TITLE.  This directive allows the user to define a title string that  appears

at the top of each page of the list file (assuming the PAGE mode is on).  The

format is:

.TITLE  "string"

The string should not exceed 80 characters.  Here are some examples:

.TITLE  "Controller version 1.1"

.title  "This is the title of the assembly"

.title  ""

TASM - Table Driven Assembler Version 2.9                             Page 26

WORD. This directive allows a value assignment to the next two bytes  pointed

to by the current Instruction Pointer.  The format is:

[label] .WORD  expr

The least significant byte of expr is put at the current Instruction  Pointer

with the most significant  byte at the next  sequential location (unless  the

MSFIRST directive has been invoked).  Here are some examples:

data_table     .WORD   (data_table + 1)

.word   $1234

.Word   (('x' - 'a')  << 2)

.Word   12, 55, 32

TASM - Table Driven Assembler Version 2.9                             Page 27

OBJECT FILE FORMATS

TASM supports four object file formats:

1.  Intel Hex (default).

2.  MOS Technology Hex.

3.  Motorola Hex.

4.  Binary

Each are described below:

Intel Hex Object Format.  This is the  default format.   This format is  line

oriented and uses  only printable  ASCII characters except  for the  carriage

return/line feed at the end of each line.  Each line in the file assumes  the

following format:

:NNAAAARRHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHCCTT

Where:

All fields marked 'hex' consist of two or four ASCII hexadecimal digits (0-9,

A-F).  A maximum of 24 data bytes will be represented on each line  (override

the 24 byte default with the '-o' command line option).

:      = Record Start Character

NN     = Byte Count (hex)

AAAA   = Address of first byte (hex)

RR     = Record Type (hex, 00 except for last record which is 01)

HH     = Data Bytes (hex)

CC     = Check Sum (hex)

TT     = Line Terminator (carriage return, line feed)

The last line of  the file will be  a record conforming  to the above  format

with a byte count of zero:

:00000001FF

The checksum is defined as:

sum = byte_count + addr_hi + addr_lo + record_type + (sum of data bytes)

checksum =  ((-sum) & ffh)

TASM - Table Driven Assembler Version 2.9                             Page 28

MOS Technology Hex Object Format. This format is line oriented and uses  only

printable ASCII characters except  for the carriage  return/line feed at  the

end of each line.  Each line in the file assumes the following format:

;NNAAAAHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHCCCCTT

All fields marked 'hex' consist of two or four ASCII hexadecimal digits (0-9,

A-F).  A maximum of 24 data bytes will be represented on each line  (override

the 24 byte default with the '-o' command line option).

;      = Record Start Character

NN     = Byte Count (hex)

AAAA   = Address of first byte (hex)

HH     = Data Bytes (hex)

CCCC   = Check Sum (hex)

TT     = Line Terminator (carriage return, line feed)

The last line of the file will be a record with a byte count of zero (';00').

The checksum is defined as:

sum  =  byte_count + address_hi + address_lo + record_type +

(sum of all data bytes)

checksum =  (sum & ffffh)

Motorola Hex  Object Format.  This  format is  line  oriented and  uses  only

printable ASCII characters except  for the carriage  return/line feed at  the

end of each line.  Each line in the file assumes the following format:

S1NNAAAAHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHCCCCTT

All fields marked 'hex' consist of two or four ASCII hexadecimal digits (0-9,

A-F).  A maximum of 24 data bytes will be represented on each line  (override

the 24 byte default with the '-o' command line option).

S1     = Record Start Prefix

NN     = Byte Count (hex) (data byte count + 3)

AAAA   = Address of first byte (hex)

HH     = Data Bytes (hex)

CC     = Check Sum (hex)

TT     = Line Terminator (carriage return, line feed)

The last  line of  the  file will  be a  record  with a  byte count  of  zero

('S903AAAACCTT'). The checksum is defined as:

sum  =  byte_count + address_hi + address_lo + record_type +

(sum of all data bytes)

checksum =  (~sum & ffh)

Binary Object Format. This  file format has only  a binary representation  of

each data byte with no  address, checksum or format description,  whatsoever.

It is often a convenient format to use to pass the data to other programs  on

your PC (like a PROM programmer package) but because of the  non-printability

and lack of address information, it is not often used to transmit the code to

TASM - Table Driven Assembler Version 2.9                             Page 29

other systems.

Note that when this object format is  selected (-b option), the -c option  is

forced.  This is done so that no ambiguity arises as a result of the lack  of

address information in the file.  Without the -c option, discontinuous blocks

of object code would appear contiguous.

TASM - Table Driven Assembler Version 2.9                             Page 30

LISTING FILE FORMAT

Each line of  source code  generates one  (or more)  lines of  output in  the

listing file.  The fields of the output line are as follows:

1.  Current  source  file  line  number  (4 decimal digits).

2.  An optional '+'  appears if this is   an  'INCLUDE'   file. (One '+'  for

each level of INCLUDE invoked).

3.  Current Instruction Pointer (4 hex digits).  An optional '~' follows  the

Instruction Pointer if the line of source code is not being assembled because

of an IFDEF, IFNDEF, or IF directive.

4.  Resulting code/data generated from  this source line (two hex digits  per

byte, each byte separated by a space, up to six bytes per line).

5.  The source line exactly as it appears in the source file.

If   paging   is   enabled  (by either  the  '-p' option  flag or  the  .PAGE

directive) some additional  fields will be  inserted into the   listing  file

every 60 lines.  These fields are:

1.  Top of Form (form feed).

2.  Assembler identifier (e.g. "TASM 6502 Assembler").

3.  Initial source file name.

4.  Page number.

5.  Title.

TASM - Table Driven Assembler Version 2.9                             Page 31

PROM PROGRAMMING

A wide variety of PROM programming equipment is available that can use object

code in  one or  more of  the formats  TASM supports.   Here  are some  notes

concerning the generation of code to be put in PROMs:

1.  PRESET  MEMORY.   It is often  desirable to  have all bytes  in the  PROM

programmed even if not explicitly assigned  a value in the source code  (e.g.

the bytes are skipped over with a .ORG statement).  This can be  accomplished

by using the  -c (contiguous  block) and the  -f (fill)  command line  option

flags.  The -c will  ensure that every byte from  the lowest byte assigned  a

value to the the  highest byte assigned  a value will be  in the object  file

with no gaps.   The  -f flag will  assign the  specified value  to all  bytes

before the assembly begins so that when the object file is written, all bytes

not assigned a  value in  the source code  will have  a known value.   As  an

example, the following command line will generate object code in the  default

Intel Hex format with all bytes not assigned a value in the source set to  EA

(hex, 6502 NOP):

tasm -65 -c -fEA test.asm

2.  CONTIGUOUS BLOCKS.   To ensure that TASM  generates object code to  cover

the full address range of the target PROM, put a .ORG statement at the end of

the source file set to  the last address desired.   For example, to  generate

code to be put in a 2716 EPROM (2 Kbytes) from hex address $1000 to $17ff, do

something like this in the source file:

;start of the file

.ORG    $1000

;rest of the source code follows

<source code>

;end of the source code

.ORG    $17ff

.BYTE   0

.END

Now, to  invoke TASM  to generate  the code  in the  binary format  with  all

unassigned bytes set to 00 (6502 BRK), do the following:

tasm -65 -b -f00 test.asm

Note that -b forces the -c option.

TASM - Table Driven Assembler Version 2.9                             Page 32

ERROR MESSAGES

Error Message                             Description

______________________________________________________________________________

Binary operator where value expected.     Two  binary  operators  in  a   row

indicate a missing value.

Cannot malloc for label storage           Insufficient memory  to store  more

labels. See LIMITATIONS.

Duplicate label.                          Duplicate    label    checks    are

optionally  enabled  by  the   '-a'

option.

File name too short                       A file name on the command line  is

fewer than  3 characters.  This  is

done to  prevent a  garbled  option

flag from being  taken as a  source

file, which in  turn can result  in

the source file taken as an  object

file (which are truncated at start-

up time).

Heap overflow on label definition.        TASM was unable to allocate  memory

to store the label.

Invalid operand.  No indirection for this instruction.

The first character  of an  operand

was  a  left  parenthesis  for   an

instruction    that    does     not

explicitly  specify  that  as   the

format.    Some   micros  use   the

parenthesis  as  an  indicator   of

indirection, but putting a layer of

parenthesis around an expression is

always a valid thing to do (as  far

as  the  expression  evaluator   is

concerned).  The test for this case

is only done if  the -a4 option  is

selected. See  section on  ASSEMBLY

CONTROL.

Invalid token where value expected.       Two binary operators  in a row  are

not allowed.

Label too long                            Labels are limited to 31 characters.

Label value misaligned                    The value  of  a label  appears  to

have  a  different  value  on   the

second pass then it was computed to

have on the  first pass.   This  is

generally   due   to   Zero    Page

Addressing mode  problems with  the

6502 version of TASM.  Labels  that

are used in operands for statements

TASM - Table Driven Assembler Version 2.9                             Page 33

that  could   utilize   Zero   Page

addressing mode  should  always  be

defined before used as an operand.

Label not found                           A label used  in an expression  was

not  found  in  the  current  label

table.

Label must pre-exist for SET.             The  SET  directive  can  only   be

applied to an existing label.

Label table overflow                      To   many    labels    have    been

encountered.

List file open error                      TASM  was  not  able  to  open  the

specified list file.

Macro expansion too long.                 The expansion of  a macro  resulted

in a line that exceeded the maximum

length.

Maximum number of macros exceeded         To many macros (DEFINEs) have  been

encountered.

No END directive before EOF               The source file did not have an END

directive in it. This is not fatal,

but may cause the last object  file

record to be lost.

No  files  specified                      TASM was  invoked  with  no  source

file specified.

No such label yet defined.                A SET directive was encountered for

a label not yet defined.  The value

of labels that are modified by  the

SET directive must already exist.

No indirection for this instruction.      A parenthesis was found around  the

operand  expression.     This   may

indicate   an   attempt   to    use

indirection     where     it     is

inappropriate.

Non-unary operator at start of expression A binary operator (such as '*') was

found  at  the   beginning  of   an

expression.  Some micros use '*' as

an indirection operator.  Since  it

is also a  legititmate operator  in

an expression,  some ambiguity  can

arise.        If    a    particular

instruction/addressing  mode   does

not allow indirection, and a '*' is

placed in front  of the  associated

expression,  the   assembler   will

assume this error.     See the  -a8

option of ASSEMBLY CONTROL.

TASM - Table Driven Assembler Version 2.9                             Page 34

Object file open error                    TASM  was  not  able  to  open  the

specified object file.

Range of argument exceeded                The value  of an  argument  exceeds

the valid  range  for  the  current

instruction and addressing mode.

Range of relative branch exceeded         A branch  instruction  exceeds  the

maximum range.

Source file open error                    TASM  was  not  able  to  open  the

specified source file.

Unrecognized directive                    A statement starting with a '.'  or

'#' has  a  mnemonic  that  is  not

defined as a directive.

Unrecognized  instruction                 A statement has an opcode  mnemonic

that is not defined.

Unrecognized argument                     A statement has  an  operand format

that is not defined.

Unknown token                             Unexpected  characters  were  found

while parsing an expression.

Unused data in MS byte of argument.       An instruction  or  directive  used

the least  significant byte  of  an

argurment   and   left   the   most

significant byte unused, but it was

non-zero.

Unkown option Flag.                       Invalid  option   flag   has   been

specified  on  the  command   line.

invoke TASM  with  nothing  on  the

command line to see a list of valid

options.

TASM - Table Driven Assembler Version 2.9                             Page 35

BUGS AND LIMITATIONS

Limitations and Specifications

TASM                  TASMB

_____________________________________________________________________________

Maximum number of labels                   2000                  10000

Maximum length of labels                   32 characters

Maximum address space                      64 Kbytes (65536 bytes)

Maximum number of nested INCLUDES          4

Maximum length of TITLE string             79 characters

Maximum source line length                 255 characters

Maximum length after macro expansion       255 characters

Maximum length of expressions              255 characters

Maximum length of pathnames                79 characters

Maximum length of command line             127 characters

Maximum number of instructions (per table) 600                   1000

Maximum number of macros                   1000

Maximum number of macro arguments          10

Maximum length of macro argument           16 characters

Heap size (for labels, macros, & buffers)  20000 bytes           60000

Memory requirements                        160K                  256K

Bugs

1.  The  8048 version of  TASM does not  check for use  of memory beyond  any

reasonable bounds (e.g.  an 8048 has a maximum address space of 4 Kbytes  but

TASM will let you pretend that you have 64 Kbytes).

2.  Expression  evaluation has  no operator  precedence in  effect which  can

make  for  unexpected results if not explicitly grouped with parenthesis.

3.  First page of  listing file will not show  a user defined title  (defined

via TITLE directive).

4.  TASM sometimes does not generate  error messages  for  improperly  formed

expressions.

TASM - Table Driven Assembler Version 2.9                             Page 36

APPENDIX A - ORDERING INFORMATION

TASM is distributed as shareware.   The shareware portion of the product  may

be freely copied and  used for evaluation  purposes.  Use  of TASM beyond  an

evaluation period of 90 days requires registration. Registered users  receive

the following benefits:

1.  The recent version of TASM.

2.  TASM source code (in C).

3.  Bound TASM manual.

4.  Telephone support.

5.  Knowledge that they are supporting the development of useful

but inexpensive software.

DESCRIPTION                                  UNIT PRICE    PRICE

___________________________________________________________________________

TASM Registration (TASM disk, manual, & source)  $40.00    _______

TASM Site Registration (for sites with multiple   90.00    _______

users.  Includes same materials as above.)

TASM User's Manual (included above)               10.00    _______

TASM update for registered users                  10.00    _______

(latest disk (with source), and manual)

Subtotal                                                   _______

Tax (Washington state residents add 8.2%)                  _______

Billing fee (for orders not accompanied by check) 10.00    _______

Foreign postage (outside North America) add $10.00         _______

(Foreign orders must be in US funds drawn on a US bank)

TOTAL (post paid)                                          _______

Which processors are of primary interest to you?  __________________

(This is for our information only.  You will receive all current TASM tables).

Shipping Address:                                Send check or money order to:

Speech Technology Incorporated

______________________________________________   Software Division

837 Front Street South

______________________________________________   Issaquah, WA  98027   USA

______________________________________________