JEFF RICHARDS

25 Bowline St

Jamboree Heights

Qld    4074

Australia

25th November, 1982

XMON is an  extended monitor, designed for those who want to

modify the monitor to include just the features they need. Its

big advantage is that it can be run as an executable disk file,

so if you want to take advantage of the enhanced screen control

commands, for instance for WordStar, then you can load the

extended monitor before invoking the program. Resetting will

return you to the standard monitor.

XCOM.MAC is the source code for Microsoft's MACRO-80 assembler.

XMON.COM is an executable file containing the extended monitor

and its loader.

XMONMAC.DOC is a brief description of the extended monitor, with

instructions on how to compile it, execute it, and blow it into

an EPROM.

EXTENDED MONITOR - CONTROL CODES AND COMMANDS

=============================================

1. SCREEN CONTROL CODES

ASCII    HEX    KEY   NAME   EFFECT

0      00     ^@    NUL    NONE

1      01     ^A    SOH    NONE

2      02     ^B    STX    NONE

3      03     ^C    ETX    NONE

4      04     ^D    EOT    NONE

5      05     ^E    ENQ    ROTATE SCREEN LEFT

6      06     ^F    ACK    ROTATE SCREEN RIGHT

7      07     ^G    BEL    AUDIBLE BELL

8      08     ^H    BS     BACKSPACE OR CURSOR LEFT

9      09     ^I    HT     HORIZONTAL TAB

10      0A     ^J    LF     LINEFEED OR CURSOR DOWN

11      0B     ^K    VT     CURSOR UP

12      0C     ^L    FF     CURSOR RIGHT

13      0D     ^M    CR     CARRIAGE RETURN

14      0E     ^N    SO     NONE

15      0F     ^O    SI     NONE

16      10     ^P    DLE    NONE

17      11     ^Q    DC1    CLEAR TO END-OF-SCREEN

18      12     ^R    DC2    NONE

19      13     ^S    DC3    NONE

20      14     ^T    DC4    ROTATE SCREEN UP

21      15     ^U    NAK    ROTATE SCREEN DOWN

22      16     ^V    SYN    ROTATE LINE LEFT

23      17     ^W    ETB    ROTATE LINE RIGHT

24      18     ^X    CAN    CLEAR TO END-OF-LINE

25      19     ^Y    EM     NONE

26      1A     ^Z    SUB    CLEAR SCREEN

27      1B     ^[    ESC    INITIATE CURSOR CONTROL SEQUENCE

28      1C     ^\    FS     INSERT LINE

29      1D     ^]    GS     DELETE LINE

30      1E     ^^    RS     HOME CURSOR

31      1F     ^_    VS     DISPLAY CONTROL CHARACTER

2. MONITOR COMMANDS

K  CLOCK DISPLAY ON/OFF

R  DISK RECORD READ

O  OUTPUT DATA TO PORT

I  INPUT DATA FROM PORT

G  GOTO ADDRESS

T  SET CLOCK TIME AND TOGGLE DISPLAY ON

F  FILL MEMORY WITH CHARACTER

M  MEMORY DISPLAY AND ALTER

C  COPY BLOCKS OF MEMORY

B  BOOT THE DISK OPERATING SYSTEM

D  DISPLAY MEMORY ON SCREEN

S  SET AND ALTER CONSOLE DEVICE

1  SET BAUD RATE - PORT A

2  SET BAUD RATE - PORT B

X  ALTER CURSOR CHARACTER

Y  ALTER DISK TIME-OUT DELAY#########################################################################