June 12, 1985

To:       All Sanyo 1200/1250 owners

From:     E. Mark Kothe

RE:       Sanyo inverse video

If  you are one of the proud owners of a Sanyo 8-bit machine

(probably  ALL Sanyo's) I really don't have to tell you that  the

documentation  for these machines leaves a little to be  desired.

Hopefully this document will help,  maybe it is common knowledge?

The  Sanyo  1200/1250 (maybe others?) has four possibilities  for

video character display:

Standard

Inverse video

Underlined

Overlined

These  character  types may also be combined  for  other  effects

(except for inverse and standard)

Here  is the description on how to use this feature in the  Sanyo

MBC-1200 series users guide escape code section (incidentally MBC

stands for.....ya ready ?... My Business Computer)

If you have manual 9376411909401 on page R-17 you are greeted by:

ESC   t        1B.74     Attribute Set   (Inverse,   underline

Upperline)

Now come on this is as CLEAR as possible, right?

If  you happened to be blessed with manual 9376411909401A on page

R-18 they have expanded on the description to this:

ESC t          1B.74     Attribute On/Off(only 33-line mode)

(BIOS ver. 1.4

or after)

D7 ........... Fixed to 0

D6 ........... Fixed to 0

D5 ........... Fixed to 0

D4 ........... Fixed to 0

D3 ........... Overline

D2 ........... Inverse

D1 ........... Underline

D0 ........... Fixed to 0

What could be clearer,  EH?

Ok,  enough spouting off,  now I'll try for a translation on this

What  I find you must do is this.   Say you wanted to  print  the

string  `I  love  my Sanyo' underlined.   You have  to  send  the

following sequence to be printed.

1BH,'t',2,'I love my Sanyo',1BH,'t',0

or in BASIC it would be

PRINT CHR$(27);"t";CHR$(2);"I Love my Sanyo";CHR$(27);"t";CHR$(0)

What this does is prepares for Underline video, prints the string

then  turns the underline feature OFF!   Otherwise the  following

characters would also be underlined.

The  key  to  this whole escape sequence is  the  character  that

follows  the 1Bh,'t'.   This is how it works with  the  character

broken down into 8 bits.

n    n    n    n    o    i    u    n

o    o    o    o    v    n    n    o

t    t    t    t    e    v    d    t

r    e    e

u    u    u    u    l    r    r    u

s    s    s    s    i    s    l    s

e    e    e    e    l    e    i    e

d    d    d    d    e         n    d

e

|    |    |    |    |    |    |    |

v    v    v    v    v    v    v    v

0    0   0     0    0    0    0    0     <- Binary value

If you set the underline bit you get underline

If you set the overline bit you get overline.

If you set both you get both.

But..

If you set JUST the inverse bit you get inverse,  underline,  and

overline.

Any   bits   set  with  the  inverse  bit  set   subtracts   that

feature  (actually  it provides you with a non inverse  under  or

overline)

OK?  I included a short (and primative) program for you to run to

demonstrate.

If  there are any questions or comments you can try leaving me  a

message on my system 313-465-9615   300/1200 baud  24 hours

Thanks,  E. Mark Kothe

VIDEOBIT.DOC

10 PRINT CHR$(26):      'clear screen

20 FOR X=0 TO 16 STEP 2

30 PRINT CHR$(27);"t";CHR$(X);:'set the bits

40 PRINT "  I Love my Sanyo   "

50 PRINT CHR$(27);"t";CHR$(0): 'undo what we did

60 NEXT X

70 PRINT CHR$(27);"t";CHR$(4);

80 PRINT "                            "

90 PRINT "                            "

100 PRINT " >> ";

101 PRINT CHR$(27);"t";CHR$(0);

102 PRINT " WATCH OUT WORDSTAR ";

103 PRINT CHR$(27);"t";CHR$(4);

104 PRINT " << "

110 PRINT "                            "

120 PRINT "                            "

125 PRINT CHR$(27);"t";CHR$(0):    'all off