#FIRMWARE SPECIFICATION                               1#

#The_Firmware_(PROM)_Primatives#

All Prom routines are entered via a Far Call (CALLF)

instruction to the required entry point in the prom

jump table. The jump table is located at the start of

the prom at segment address FC00h, offset 3000h.

The disc related routines usually require large

amounts of stack space.

All registers not used for returning results, have

their entry value preserved.

The only re-entrant routines are the get switch

routine and the SIO input/output/status routines (when

called with different port numbers).

.PA�

#FIRMWARE SPECIFICATION                               2#

The following is a list of all the PROM calls and

their entry point within the jump table:

#Function      Entry      Action_Performed#

BOOT          FC00:3000  Bootstrap loader

SIO_IN        FC00:3005  Serial Input

SIO_OUT       FC00:300A  Serial Output

SIO_IN_STAT   FC00:300F  Serial Input status

SIO_OUT_STAT  FC00:3014  Serial Output status

PRNTR_ST      FC00:3019  Printer status

PRNTR_OUT     FC00:301E  Printer output

FLOP_SET      FC00:3023  Floppy initialisation

FLOP_ALL      FC00:3028  Floppy perform activity

FLOP_START    FC00:302D  Floppy start activity

FLOP_CONT     FC00:3032  Floppy continue activity

FLOP_ABORT    FC00:3037  Floppy abort activity

HDSK_ALL      FC00:303C  Hard disc perform activity

HDSK_START    FC00:3041  Hard disc start activity

HDSK_CONT     FC00:3046  Hard disc continue activity

HDSK_ABORT    FC00:304B  Hard disc abort activity

HDSK_SIZE     FC00:3050  Return size of hard disc

GET_SWITCH    FC00:3055  Get system byte

.PA�

#FIRMWARE SPECIFICATION - INITIALISE                  3#

#BOOT#

#Bootstrap_Loader#

Entry Values :      None

Exit Values  :      None

This is the Prom restart.

It performs a self test and initialises all the

hardware.

All interrupt vectors are set to a dummy routine which

clears the trap-flag and returns.

Sector zero is loaded from floppy or winchester into

memory at address FC00:BC00. If the six bytes from

offset BC02 match 'Lsi_M4' or 'Cal_PC' then a jump is

made to FC00:BC00. Failing that, a boot error message

is displayed, and once a key has been pressed the

process is repeated.

This routine does NOT return.

.PA�

#FIRMWARE SPECIFICATION - SERIAL I/O                  4#

#Serial_I/O_Routines#

SIO_IN          -    Serial Input

SIO_OUT         -    Serial Output

SIO_IN_STAT     -    Serial Input status

SIO_OUT_STAT    -    Serial Output status

#Definition_of_M-Four_Logical_Ports#

0 = Keyboard and Monitor

1 = RS232 port 1 - DART Channel B

2 = RS232 port 2 - DART Channel A

3 = RS232 port 3 - SIO Channel B

4 = RS422 port   - SIO Channel A

#Definition_of_Cal-PC_Logical_Ports#

0 = Keyboard and Monitor

1 = Keyboard and Monitor

2 = RS232 port 1 - DART Channel A

3 = RS232 port 2 - SIO Channel B

4 = RS422 port   - SIO Channel A

.PA�

#FIRMWARE SPECIFICATION - SERIAL I/O                  5#

#SIO_IN#

#Serial_Input#

Entry Values:       DL = logical port number

Exit Values:        AL = character

All registers preserved.

This routine waits until a character is available.

.PA�

#FIRMWARE SPECIFICATION - SERIAL I/O                  6#

#SIO_OUT#

#Serial_Output#

Entry Values:       AL = character

DL = logical port number

Exit Values:        Flags affected

Direction-flag reset if monitor

Escape sequences are handled if monitor port.

If not monitor port, the routine loops until

SIO_OUT_STAT returns ready status, before outputting

the character.

.PA�

#FIRMWARE SPECIFICATION - SERIAL I/O                  7#

#SIO_IN_STAT#

#Serial_Input_Status#

Entry Values:       DL = logical port number

Exit Values:        NZ - character available

Z - character not available

All registers preserved.

.PA�

#FIRMWARE SPECIFICATION - SERIAL I/O                  8#

#SIO_OUT_STAT#

#Serial_Output_Status#

Entry Values:       DL = logical port number

Exit Values:        NZ - ready

Z - not ready

All registers preserved.

Port 0 (the monitor) is always ready.

SIO ports are ready if the transmit buffer is not full

and the DSR and CTS inputs are active.

.PA�

#FIRMWARE SPECIFICATION - CENTRONICS PRINTER          9#

#PRNTR_ST#

#Printer_Status#

Entry Values:       None

Exit Values:        NZ - centronics port ready

Z - centronics port not ready

All registers preserved.

.PA�

#FIRMWARE SPECIFICATION - CENTRONICS PRINTER         10#

#PRNTR_OUT#

#Printer_Output#

Entry Values:       AL = character to print

Exit Values:        None

All registers preserved.

.PA�

#FIRMWARE SPECIFICATION - FLOPPY ROUTINES            11#

#Floppy_Disc_Routines#

FLOP_SET        -    Floppy initialisation

FLOP_ALL        -    Floppy perform activity

FLOP_START      -    Floppy start activity

FLOP_CONT       -    Floppy continue activity

FLOP_ABORT      -    Floppy abort activity

#Definition_of_Floppy_Disc_Parameters#

#Floppy_Flag_Byte#

#Bit       0_-_reset           1_-_set#

0        SINGLE SIDED        DOUBLE SIDED

1        unused              unused

2        unused              unused

3        DOUBLE DENSITY      SINGLE DENSITY

4        unused              DD DISC IN QD DRIVE

5        unused              unused

6        unused              unused

7        unused              unused

.PA�

#FIRMWARE SPECIFICATION - FLOPPY ROUTINES            12#

#Definition_of_Floppy_Disc_Parameters_-_continued#

#Floppy_Command_Byte#

#Bit       Action#

7         RECAL

6         WRITE

5         READ

4         VERIFY

3         unused

2         unused

1         unused

0         DRIVE

#Valid_Command_Bit_Combinations#

#Bit  7    6    5    4#

0    0    0    0    Status only

1    1    1    1    Uncommitted entry point

1    0    0    0    Recal only

x    0    0    1    Verify

x    0    1    0    Read

x    0    1    1    Unused

x    1    0    0    Write

x    1    0    1    Write + verify

x    1    1    0    Seek

x    1    1    1    NEVER to be used due to

Confusion with seek + recal

.PA�

#FIRMWARE SPECIFICATION - FLOPPY ROUTINES            13#

#Definition_of_Floppy_Disc_Parameters_-_continued#

#Prom_error_codes_returned_in_AL#

10        Seek error

20        CRC error in ID

30        Write protected (from FDC)

38        No track zero

40        Record not found

50        Lost data (from FDC)

60        CRC error in data

70        DMA not reached terminal count

80        Prom busy

90        Sector out of range

A0        Bank crossing

B0        Write protected disc

Retry count returned in bits 0, 1, and 2

.PA�

#FIRMWARE SPECIFICATION - FLOPPY ROUTINES            14#

#Definition_of_Floppy_Disc_Parameters_-_continued#

#FD-1793_error_codes_returned_in_AH#

Return codes for:

RESTORE, SEEK, STEP IN/OUT

BIT 7 - Drive is not ready

BIT 6 - Drive is write protected

BIT 5 - Head is loaded

BIT 4 - Seek error on desired track

BIT 3 - CRC error in sector ID field

BIT 2 - Read/write head is at track zero

BIT 1 - Index mark has been detected

BIT 0 - Command is in progress

Return codes for:

READ/WRITE SECTOR, READ ADDRESS, READ/WRITE TRACK

BIT 7 - Drive is not ready

BIT 6 - Drive is write protected

BIT 5 - On read record 1 = deleted data mark

0 = data mark

On any write   1 = write fault

BIT 4 - Desired track, sector or side not found

BIT 3 - If bit 4 = 1 - CRC error in ID field

If bit 4 = 0 - CRC error in data field

BIT 2 - Computer not taking data fast enough

BIT 1 - Date available on read operation or

- Data required during write operation

BIT 0 - Command is being executed

.PA�

#FIRMWARE SPECIFICATION - FLOPPY ROUTINES            15#

#FLOP_SET#

#Floppy_initialisation#

Entry Values:       AL = floppy flag byte

CL = retries (usually 10)

CH = sectors per track

SI = total sectors per disc

B� � byte� pe� sector

Exit Values:        None

All registers preserved.

.PA�

#FIRMWARE SPECIFICATION - FLOPPY ROUTINES            16#

#FLOP_ALL#

#Floppy_complete_activity#

Entry Values:       AL = floppy command byte

CX = number of sectors

DX = logical sector number

BX = buffer address

DS = buffer segment

Exit Values:        NZ - activity failed

Z - activity successful

AL = prom error code

AH = FD-1793 status

Perform a complete disc activity.

Register AX plus flags affected.

.PA�

#FIRMWARE SPECIFICATION - FLOPPY ROUTINES            17#

#FLOP_START#

#Floppy_start_activity#

Entry Values:       AL = floppy command byte

CX = number of sectors

DX = logical sector number

BX = buffer address

DS = buffer segment

Exit Values:        NZ - activity finished

Z - activity not finished

AL = prom error code

AH = FD-1793 status

Initiate floppy disc activity.

Register AX plus flags affected.

.PA�

#FIRMWARE SPECIFICATION - FLOPPY ROUTINES            18#

#FLOP_CONT#

#Floppy_continue_activity#

Entry Values:       None

Exit Values:        NZ - activity finished

Z - activity not finished

AL = prom error code

AH = FD-1793 status

Continue activity initiated by FLOP_START.

This routine may be called at any time after a call to

FLOP_START or FLOP_CONT returned NZ. However the call

is only useful if the controller is ready (i.e

interrupt pending)

Register AX plus flags affected.

.PA�

#FIRMWARE SPECIFICATION - FLOPPY ROUTINES            19#

#FLOP_ABORT#

#Floppy_abort_activity#

Entry Values:       None

Exit Values:        None

Aborts any pending floppy disc activity.

All registers preserved.

.PA�

#FIRMWARE SPECIFICATION - HARD DISC ROUTINES         20#

#Hard_Disc_Routines#

HDSK_ALL        -    Hard disc perform activity

HDSK_START      -    Hard disc start activity

HDSK_CONT       -    Hard disc continue activity

HDSK_ABORT      -    Hard disc abort activity

HDSK_SIZE       -    Return size of hard disc

#Definition_of_Hard_Disc_Parameters#

#Hard_Disc_Command_Byte#

#Bit       Action#

7         RECAL

6         WRITE

5         READ

4         VERIFY

3         CONTROLLER NUMBER

2         CONTROLLER NUMBER

1         UNIT NUMBER

0         UNIT NUMBER

.PA�

#FIRMWARE SPECIFICATION - HARD DISC ROUTINES         21#

#Definition_of_Hard_Disc_Parameters_-_continued#

#Prom_error_codes_returned_in_AL#

00-07   No error (or correctable if AH=98H)

08-0F   Select error

10-17   Timeout sending command data block

18-1F   Timeout waiting for Acknowledge

20-27   Wrong status after end-of-command int

28-2F   No message byte

48-4F   Error reading Xebec status

68-6F   Command or RAM error

70-77   DMA not reached terminal count

78-7F   Error successfully read

84      Prom controller busy

88      Controller bit busy

A0      Bank crossing error

Retry count returned in bits 0, 1, and 2.

.PA�

#FIRMWARE SPECIFICATION - HARD DISC ROUTINES         22#

#Definition_of_Hard_Disc_Parameters_-_continued#

#Xebec_error_codes_returned_in_AH#

00      No error detected

01      No index detected from disc drive

02      No seek complete from disc drive

03      Write fault from disc drive

04      Drive not ready after select

06      Track zero not found

10      ID field read error

11      Uncorrectable data error

12      Address Mark not found

14      Target sector not found

15      Seek error

18      Correctable data error

19      Bad track flag detected

1A      Format error

1C      Illegal access to alternate track

1D      On format alternate track command, the

requested alternate track has already

been assigned as an alternate, or is

flagged a bad track

1E      When controller attempted to access an

alternate track from the spared track,

the alternate track was not flagged as an

alternate

1F      On a format alternate track command, the

bad track equalled the alternate track

20      Invalid command

21      Illegal disc address

30      Ram diagnostic failure

31      Program memory checksum error

32      ECC diagnostic failure

The above codes may be output with bit 7 set,

indicating the previous command used a logical block

address.

.PA�

#FIRMWARE SPECIFICATION - HARD DISC ROUTINES         23#

#HDSK_ALL#

#Hard_disc_complete_activity#

Entry Values:       AL = command byte

CH:DX = start sector (absolute)

CL = number of sectors (1-256)

DS:BX = buffer start address

CH = bits 5,6,7 are control field

(normally 0, 111 for diag)

AH = user command if bits 7-4 of

AL are clear

DI = supplementary info if bits

7-4 of AL are clear.

bits 0-13  = length of DMA-1

bit 14 set = read cntrlr

bit 15 set = write cntrlr

bites 14/15 clear = no DMA

Exit Values:         Z = Ok

NZ = error

AL = error code + retry count

AH = Xebec S1410 status

if error:

CH:DX = bad sector address

Perform a complete disc activity.

.PA�

#FIRMWARE SPECIFICATION - HARD DISC ROUTINES         24#

#HDSK_START#

#Hard_disc_start_activity#

Entry Values:       AL = command byte

CH:DX = start sector (absolute)

CL = number of sectors (1-256)

DS:BX = buffer start address

CH = bits 5,6,7 are control field

(normally 0, 111 for diag)

AH = user command if bits 7-4 of

AL are clear

DI = supplementary info if bits

7-4 of AL are clear.

bits 0-13  = length of DMA-1

bit 14 set = read cntrlr

bit 15 set = write cntrlr

bites 14/15 clear = no DMA

Exit Values:         Z = activity not finished

NZ = activity finished

AL = error code + retry count

AH = Xebec S1410 status

if error:

CL:DX = bad sector address

Initiate a hard disc activity.

.PA�

#FIRMWARE SPECIFICATION - HARD DISC ROUTINES         25#

#HDSK_CONT#

#Hard_disc_continue_activity#

Entry Values:       None

Exit Values:         Z = activity not finished

NZ = activity finished

AL = error code + retry count

AH = Xebec S1410 status

if error:

CL:DX = bad sector address

Continue hard disc activity.

This may be called any time after the preceding

HDSK_START or HDSK_CONT call gave a zero return.

However the call is only useful if a hard disc

interrupt is pending.

.PA�

#FIRMWARE SPECIFICATION - HARD DISC ROUTINES         26#

#HDSK_ABORT#

#Hard_disc_continue_activity#

Entry Values:       None

Exit Values:        None

Aborts any pending hard disc activity.

All registers preserved.

.PA�

#FIRMWARE SPECIFICATION - HARD DISC ROUTINES         27#

#HDSK_SIZE#

#Hard_disc:_return_size_of#

Entry Values:       None

Exit Values:        BX = number of heads

CX = sectors per track

DX = number of cylinders

AX = total sectors on disc

.PA�

#FIRMWARE SPECIFICATION - SYSTEM SWITCH BYTE         28#

#Definition_of_System_Switch_Byte#

BIT 7 - Half height 5" floppy drives fitted

BITS 5 & 6 - Winchester drive type

0,0 = 5mb  ST-412 (M4), RO201 (Cal)

0,1 = 10mb RO202

1,0 = 15mb RO203

1,1 = 20mb RO204

BIT 4 - 2 floppies present (else only 1)

BIT 3 - Winchester present

BIT 2 - 8 inch drives

BIT 1 - Quad density drives

BIT 0 - Double sided drives

.PA�

#FIRMWARE SPECIFICATION - SYSTEM SWITCH BYTE         29#

#GET_SWITCH#

#Get_system_switch_byte#

Entry Values:       None

Exit Values:        AL = system dip switch setting

Returns the system dip switch setting.

All other registers plus flags preserved.

.PA�##################################################################################################################