Connecting IDE Drives

by

Tilmann Reh

Foreword

This text has been published in TCJ several years ago, in

three parts. For your convenience, I concatenated them into

one file again. Sometimes the text refers to other articles

in TCJ, especially to a description of my ECB-bus based

IDE interface. However, I think the information in this text

contains all necessary information about IDE interfacing in

general.

29. October 2001, Tilmann Reh

Remembering the Basics

Let us first have a short look at the drives we want to use.

When discussing the different hard disk interfaces in my

last article, I already pointed out that IDE drives of the

AT-type, thus often called AT-bus drives (Bill Kibler calls

them ATA drives, but this abbreviation is not the usual one,

at least in Europe), are the ones with the best

price/performance ratio one can get. This is even more the

case now. So IDE drives still are the very first choice if

you are looking for a good and cheap hard disk for your

computer.

But what's special with those drives?  I already mentioned

that the IDE drive contains the complete hard disk

controller. It is accessed with a system-bus interface

compatible with the PC/AT (ISA) bus and offers control and

data registers still compatible with the very first PC/AT

hard disk controller (based on the WD 1010 controller chip).

But even if those specifications come from something I don't

like at all, why not use the low-price components for real

computing (i.e., with a CPU280)?

Bringing the hard disk controller into the drive electronics

offers some advantages. One of the main features is that

you don't have a serial data stream with fixed bit rate

between controller and drive. Thus, there's no need for

conditioning the signals for the interface, and you can use

any bit rate. As a result, the hard disk performance is

limited by the drive technology, not by the interface's bit

rate. This is one reason why today's drives are so much

faster than the older ones. And technologies like Seagate's

ZBR (Zone Bit Recording) are possible with hardware-

independent interfaces.

There is another main feature of bringing the controller

into the disk drive. Today's drives have very extensive

checks for data security. They store error correction codes

(ECC) together with the sector data and automatically

correct single-bit errors, so the sector need not be re-read

in those cases. Additionally, if a sector is found to be

too unreliable, it is internally marked as bad and the data

is mapped to a spare sector (usually there is one spare

sector per track). All this is absolutely transparent to

the user. So you now know the reason why today's

intelligent drives don't have "defect lists" any more.

Since the PC's have such bad software (and hardware, too),

there is another thing the integrated controller can do:

translate virtual addressing information into physical.

That means that the IDE drive is able to emulate another

drive with different parameters (cylinder count, number of

heads, and sectors per track). For the PC this is necessary

because many PCs don't support drives with other than the

historical 17 sectors per track, and many do not support

free configuration of the drive parameters (only selection

from a table is allowed). Also, some PCs mask off some bits

of the cylinder number, since the first controller only had

a 10-bit cylinder register -- so nearly every IDE drive

still supports an emulation mode with less than 1024

cylinders and 17 sectors per track.

The IDE Interface

As mentioned above, the IDE interface is almost completely

identical with a subset of the PC/AT expansion bus, so the

drive can be connected (almost) directly to that. The only

things required externally are two select signals (I/O

address decoding). This gives us some information about how

the interface works. In a PC the drive is accessed directly

by the CPU via I/O accesses to registers internal to the

drive. The disk data is transferred via the 16-bit data

bus, but for compatibility to the older systems (again!)

only 8 bits are used for command and status information.

Besides the data bus, there are the standard Intel-type data

strobe signals (/IORD and /IOWR), a few address lines, and

some special signals. The connector is a 40-pin header, not

to be confused with the XT-type IDE interface connector,

which is also a 40-pin header but needs somewhat different

hardware and totally different software!

The IDE interface allows connection of two drives with one

cable. The second drive (slave) is then chained to the first

one (master). However, I heard about problems when trying

to connect different drives from different manufacturers.

And the capacities of today's drive are so high that a

single drive will always be enough for an 8-bit personal

computer system!  So, I never tried this option.

To understand the interface in detail, let's have a closer

look at the IDE interface connector and its signals:

1   /RES            2   GND

3   D7              4   D8

5   D6              6   D9

7   D5              8   D10

9   D4             10   D11

11   D3             12   D12

13   D2             14   D13

15   D1             16   D14

17   D0             18   D15

19   GND            20   No Pin

21   /IOCHRDY       22   GND

23   /IOWR          24   GND

25   /IORD          26   GND

27   /IOCHRDY       28   ALE

29   No Connection  30   GND

31   IRQ            32   /IO16

33   A1             34   /PDIAG

35   A0             36   A2

37   /CS0           38   /CS1

39   /ACT           40   GND

The signals of the IDE interface can be collected in several

groups: The general control signals are /RES (Reset) and

/PDIAG (Passed Diagnostics). The data bus consists of 16

data lines (D0..D15). The access control lines are three

address lines (A0..A2), the select signals /CS0 and /CS1

(Chip Select 0/1), and the strobe signals /IORD and /IOWR

(and eventually ALE, the address strobe). The remaining

signals (IOCHRDY, IRQ, /ACT, /IO16) are status signals.

Now Let's Go Into Details.

The reset signal normally is active-low. However, I heard

about drives with an active-high reset signal, but I never

saw one (or read such specifications). The /PDIAG pin

carries a bidirectional signal used for chaining two IDE

drives (master/slave). It normally can also be left open.

The data bus carries the 16-bit data words to and from the

host. However, when accessing the control and status

registers of the IDE drives, only data bits 0 through 7 are

used (8-bit transfer). The data bus lines are tri-state

lines that may be connected directly to the host's data bus.

However, to meet the host bus specs and to avoid noise

problems caused by the interface cable, a bus driver IC

should be used to decouple the IDE bus and the host bus.

The drive is accessed using the selection signals /CS0 and

/CS1. This also has historical (compatibility) reasons.

Together with the three address lines, there could be two-

times-eight addresses being occupied by an IDE drive.

However, while the main register set really has eight

registers and is accessed with /CS0 active, the other set

(with /CS1) has only two valid addresses. We will have a

deeper look at all the registers later. The data transfer

is always strobed by the timing signals /IORD and /IOWR, for

reading and writing, respectively. The address strobe (ALE)

is often unused in the drive; it should be pulled high for

static address lines (non-multiplexed busses).

The status signals are not absolutely needed to use IDE

drives. Some of these signals are not commonly delivered at

all (for example, /IOCHRDY (I/O Channel Ready), which is a

WAIT signal for the host when the drive is much slower than

the host processor in terms of interface access times). The

/IO16 line informs the host of 16-bit transfers. Since we

already know that data transfers are always 16-bit and

everything else is always 8-bit, this is redundant (however,

needed in the PC/ATs for their ISA bus). Line /ACT (Active)

is an output which can be used for driving a drive-busy LED.

Line IRQ is an interrupt request line that goes active on

some internal events (if enabled by software).

Most IDE drives contain some jumpers that allow some options

to be selected. This normally includes at least

master/slave selection. Sometimes the /ACT signal may also

be jumpered as an output signalling the presence of a second

(slave) drive. The default state of the jumpers normally

need not be changed (single drive, no special situation).

All interface lines carry CMOS-TTL-compatible signal levels.

However, some signals (IRQ, /PDIAG, /IO16, /ACT) are able to

drive higher currents. Those details should be looked up in

each drive's specifications (for example, the /ACT output

sinks 20 mA on my Conner drive, more than enough for an

LED).

Accessing the drive is done with the following sequence of

operations:  First, the address lines and the chip selects

must be set according to the desired register address. After

some time (a minimum of 25 ns), /IORD or /IOWR is activated.

This causes the data to appear on the data lines (when

reading) or to be written to the drive (with the trailing

edge of /IOWR, but there are setup and hold times to take

care of). After a minimum of 80 ns, the strobe signal has to

be removed. There are some more timing requirements, but

these are the main ones.

The above timing details might differ from drive to drive.

Always keep in mind that the IDE definition follows the

PC/AT system expansion bus and that official standards were

not specified until two years ago, when the IEEE finally

defined some specifictions (which many PC manufacturers are

not following).

Unfortunately, I found that the drives do not match their

own specs in every detail. For example, I found that the

address lines of my Conner drive (a CP-3044 with 42 MB) must

be kept stable for much more than the specified setup time.

In addition, the drive is very sensitive to spike noise on

the address lines, even if the noise appears long before an

access is initiated. I spent a great deal of time

struggling with such unlucky details (fixing other people's

bugs).

IDE Interface Registers

Now that we've covered the interface signals and their

meaning and usage, let's look at the registers of the

interface. We saw that there are eight addresses being

accessed through /CS0 and two addresses through /CS1. The

following is a list of all the internal registers of an IDE

drive:

/CS0  /CS1  A2 A1 A0  Addr.   Read Function       Write Function

----------------------------------------------------------------------

0     1    0  0  0   1F0    Data Register       Data Register

0     1    0  0  1   1F1    Error Register      (Write Precomp Reg.)

0     1    0  1  0   1F2    Sector Count        Sector Count

0     1    0  1  1   1F3    Sector Number       Sector Number

0     1    1  0  0   1F4    Cylinder Low        Cylinder Low

0     1    1  0  1   1F5    Cylinder High       Cylinder High

0     1    1  1  0   1F6    SDH Register        SDH Register

0     1    1  1  1   1F7    Status Register     Command Register

1     0    1  1  0   3F6    Alternate Status    Digital Output

1     0    1  1  1   3F7    Drive Address       Not Used

The above addresses are those used in the PC/AT. Of course

they are dependent on the decoding of the chip-select

signals. The registers accessed via /CS1 might differ

depending on the manufacturer of the drive. As far as I

know, they don't always follow the compatibility principle

with the first hard disk controller of the PC/AT.

The registers being accessed with /CS0 are also called the

"Task File", so sometimes the IDE is also referenced to as

"Task File Interface".

The error register can only be read. It contains valid

information only if the error bit in the status register is

set. Only five of the eight bits are used. They have the

following meaning:

Bit 7:    Bad block. This bit is set when the requested

sector's ID contained a bad block mark (can be set

when formatting the disk).

Bit 6:    Uncorrectable data error. Set when the sector

data can't be recreated (even with ECC).

Bit 4:    Requested sector ID not found (wrong sector

number).

Bit 2:    Command was aborted due to drive status error or

invalid command.

Bit 1:    Track 0 has not been found when recalibrating.

The unused bits are always read as zero. However, I guess

it's best not to rely on that!

The write precompensation register was previously used to

set the starting cylinder for write precompensation (a

slight shift of the serial data stream pulses to compensate

for some magnetic effects on the disk surface). Since IDE

drives handle all that internally, this function is not

needed any more. Today, this register is often used as a

parameter register for enabling or disabling look-ahead

reading. We'll have a deeper look at that when talking

about the various commands of IDE drives.

The sector-count register defines the number of sectors to

be read or written with the next read/write command. A zero

value causes 256 sectors to be processed, so the count

varies from 1 to 256. This register is also used during

drive initialization to specify the number of sectors per

track (remember the emulation capability).

The sector-number register contains the starting sector

number for any disk access. After a sector is processed,

and after the command is completed, this register is

updated. When an error occurs, this register contains the

ID number of the erroneous sector. Normally, the sector

numbers start with 1 and increase with each sector.

However, by reformatting the disk, this order and the values

may be changed.

The cylinder-low and cylinder-high registers contain the

10-bit cylinder number to be accessed. Since many drives

have more than 1024 cylinders today, the cylinder-high

register is often expanded to more than two bits. Like the

sector-number register, these registers are updated after

command completion and after errors. They are also used

during drive initialization as the number-of-cylinders

parameter.

The SDH register is a special register serving several

functions. SDH is an abbreviation for "Sector size, Drive

and Head". The bits of this register are arranged as

follows:

Bit 7:    Historical: Extension Bit. When zero, CRC data is

appended to the sector's data fields. When set to

one, no CRC data is appended. Since today's

drives always use ECC error correction, this bit

must always be set (no CRC).

Bit 6-5:  Sector Size. Since today's drives always have 512-

byte sectors (unchangeable by the user) because

PCs are not able to support other sizes, these

bits must always be 0-1.

Bit 4:    Drive. This bit distinguishes between the two

connected drives when using the master-slave

chain. Single drives are always accessed with the

drive bit set to zero.

Bit 3-0:  Head number. These four bits contain the head

number (that is, the disk surface number) for all

following accesses. Similar to the cylinder and

sector number, these bits are updated by the

drive. The head number field is also used for

drive initialization to specify the number of

heads.

The read-only status register contains eight single-bit

flags. It is updated at the completion of each command. If

the busy bit is active, no other bits are valid. The index

bit is valid independent of the applied command. The bit

flags are:

Bit 7:    Busy flag. When this flag is set, the task file

registers must not be accessed due to internal

operations.

Bit 6:    Drive ready. This bit is set when the drive is up

to speed and ready to accept a command. When

there is an error, this bit is not updated until

the next read of the status register, so it can be

used to determine the cause of the error.

Bit 5:    Drive write fault. Similar to "drive ready", this

bit is not updated after an error.

Bit 4:    Drive seek complete. This bit is set when the

actuator of the drive's head is on track. This

bit also is updated similarly to "drive ready".

Bit 3:    Data request. This bit indicates that the drive

is ready for a data transfer.

Bit 2:    Corrected data flag. Set when there was a

correctable data error and the data has been

corrected.

Bit 1:    Index. This bit is active once per disk

revolution. May be used to determine rotational

speed.

Bit 0:    Error flag. This bit is set whenever an error

occurs. The other bits in the status register and

the bits in the error register will then contain

further information about the cause of the error.

The command register is used to pass commands to the drive.

There are many commands, not always using all parameters in

the task file. Command execution begins immediately after

the command is written to this register. Since this article

is already quite long, I will cover the commands, their

parameters, and their usage in another article, probably in

the next TCJ issue.

The alternate status register contains the same information

as the status register in the task file. The only

difference is that reading this register does not imply

interrupt acknowledge to reset a pending interrupt (as the

main status register does).

The digital output register contains only two valid data

bits. Bit 2 is the software reset bit, which causes a drive

reset when being set, and bit 1 is the interrupt enable

flag.

The drive-address register simply loops back the drive

select bit and head select bits of the currently selected

drive. This information normally is of no use for the

programmer or user.

Last Words

Now that we had a look at the IDE interface, we also see the

physical limits of this interface definition. With a fully

expanded cylinder-high register, we are able to address up

to 65536 cylinders, with up to 16 heads and up to 256

sectors per track. This results in a maximum addressable

drive capacity of 128 gigabytes. I think this should be

enough for microcomputing!!  However, even if the PC/AT BIOS

limitations are encountered, we could address 1024 cylinders

with 16 heads and 64 sectors per tracks, giving 512

megabytes maximum capacity. This is also not bad, at least

for small (8-bit) computer systems, where complete

application software packages require only about 100

kilobytes of disk space.

Next time I would like to talk about the applicable commands

of IDE drives and give examples of how to write software

that accesses those drives.

List of Abbreviations:

AT    Advanced Technology               Class of PC's

BIOS  Basic I/O System                  Hardware-dependent part of OS

CMOS  Complementary Metal-Oxid-Silicon  Semiconductor technology

CRC   Cyclic Redundancy Check           Error detection code, see also ECC

ECB   ???                               European standard 8-bit system bus

ECC   Error Correction Code             Additional data for security

IDE   Integrated Drive Electronics      Intelligent hard disk interface

IEEE  Institute of Electrical and Electronics Engineers

I/O   Input/Output                      (self-explanatory)

ISA   Industry Standard Architecture    PC/AT expansion bus

LED   Light Emmitting Diode             Optoelectronical component

OS    Operating System                  Software which makes computers usable

PC    Personal Computer                 Synonym for the worst computer architecture

TTL   Transistor-Transistor-Logic       Digital component standard (74xx series)

XT    eXtended Technology               Class of PCs, previous to AT

ZBR   Zone Bit Recording                Variable Density Recording Method

Connecting IDE Drives, Part II

by

Tilmann Reh

In Part I (printed in the previous issue of TCJ) we covered the

basics of the IDE interface in terms of history, concept,

hardware, and register structure. This time we want to dig deeper

into the software side of those drives.

Terminology

Using common terminology, I often simply refer to the "drive"

when, in fact, I am thinking of the integrated controller of an

IDE drive. However, when explicitly talking of an external

controller like the WD1010, I always refer to the "controller".

Register Accessing

Let us first recall the Task File. It consists of the data

register, a set of six parameter registers, and the

command/status register. For those who don't have Part II lying

nearby, here is a shortform:

Relative Address    Register                           Abbr.

------------------------------------------------------------

0           Data Register                      D

1           Error Reg. / Write Precomp. Reg.   E / WP

2           Sector Count                       SC

3           Sector Number                      SN

4           Cylinder Low                       C

5           Cylinder High                      C

6           SDH (Sector Size, Drive, Head)     D,H

7           Status Reg. / Command Reg.

Also remember that the data register is the only 16-bit register!

Every parameter register of the task file is freely accessible as

long as there is no active command. Before loading the command

register, all related parameter registers must contain the

appropriate values. They may be loaded in any order. After the

command register is loaded, the issued command is immediately

started. The original WD1010 hard disk controller chip had a flag

(bit 1 of the status register) which was set during execution.

With IDE drives, the BUSY flag of the status register is simply

set until the command execution is completed.

The WD1010 controller chip knew only 6 commands. However, some of

the commands have option flags within them. To support additional

features, today's drives have many more commands. The following

is a list of common commands, options, and needed parameters,

with the WD1010 commands marked by an asterisk and the

manufacturer-dependent expansions marked with a plus sign:

Command             Type  7 6 5 4 3 2 1 0    Hex       Parameters

--------------------------------------------------------------------

Recalibrate          *    0 0 0 1  (Rate)    10-1F     D

Read Sector          *    0 0 1 0 0 M L T    20-27     SC,SN,C,D,H

Write Sector         *    0 0 1 1 0 M L T    30-37     SC,SN,C,D,H

Scan ID / Verify     *    0 1 0 0 0 0 0 T    40,41     D,(SC,SN,C,H)

Write Format         *    0 1 0 1 0 0 0 0    50        C,D,H,(SC,SN)

Seek                 *    0 1 1 1  (Rate)    70-7F     C,D,(H)

Exec Diagnostics          1 0 0 1 0 0 0 0    90        D

Set Drive Parameters      1 0 0 1 0 0 0 1    91        SC,(C),D,H

Read Multiple        +    1 1 0 0 0 1 0 0    C4        SC,SN,C,D,H

Write Multiple       +    1 1 0 0 0 1 0 1    C5        SC,SN,C,D,H

Set Multiple         +    1 1 0 0 0 1 1 0    C6        SC,D

Power Commands       +    1 1 1 0 0 x x x    E0-E6     SC,D

Read Sector Buffer        1 1 1 0 0 1 0 0    E4        D

Write Sector Buffer       1 1 1 0 1 0 0 0    E8        D

Identify Drive            1 1 1 0 1 1 0 0    EC        D

Cache On/Off         +    1 1 1 0 1 1 1 1    EF        D,WP

Power Save           +    1 1 1 1 1 x x x    F8-FD     ?

Parameters in parentheses are needed with some drives and ignored

by others (depending on the manufacturer and age). Any required

parameters must be valid before a command is started.

Although most of the commands are manufacturer-dependent, this

usually does not raise problems. For normal operation of the

drive, only the WD1010's and few of the really common commands

are needed. Now let's have a look at the options.

In the Restore and Seek commands, there is a four-bit rate field.

This was originally intended to specify the step rate for head

movements, with a zero value meaning 35 us per step and all other

values representing counts of 0.5 ms per step (so that the range

was from 0.5 to 7.5 ms). The hard disk controller had a memory

for each drive's step rate, so the same value would be used for

implied seeks later. But very soon, even with later ST-506

controller boards, this step-rate field became obsolete (due to

handshake mechanisms between controller and drive). With today's

IDE drives, the four lower bits of those commands are generally

ignored.

The Read and Write commands originally had an option flag (M) for

multi-sector (m/s) transfers. Today this flag is nonexistent. For

doing m/s transfers, the sector-count register is simply set to

the desired number of sectors to be processed.

But today's drives have another flag which originally wasn't

there: the "Long" flag (L). When it is set, the ECC (error-

correction) data is transferred after the sector's data field. I

assume this was meant for error correction when there are too

many errors for the drive to correct automatically. However,

there is a very odd characteristic: the ECC data is transferred

in bytes through the data register. This is the only case where

the data register doesn't transfer word data! By the way, the

number of ECC bytes differs from drive to drive, but can be read

out using the Identify Drive command.

All generations of hard disk controllers and IDE drives support

the last option flag for Read/Write commands, the Retry Flag (T).

Normally, the drive retries to read/write a sector after non-

fatal errors. When this option flag is set, automatic retries are

disabled. The Scan ID command also supports this option.

The Commands

I will now explain the commands and their parameters in detail.

To do this, besides drawing on my own experience in IDE

interfacing, I collected detailed information and specifications

from three different, independent sources. However, there still

might be some slightly different drives or controllers out there.

Please inform me if you encounter problems or differences with

your disk.

One general feature of both the WD1010 controller and modern IDE

drives is implied seeks. That means you don't have to explicitly

move the read/write (r/w) heads to the desired cylinder before

starting to read from or write to the disk. When the command is

issued and the actual cylinder number doesn't match that of the

cylinder registers, an implied seek is performed, transparent to

the user. This applies to every command where it may be needed

(Read, Write, Verify, Format).

Recalibrate (1xh):

This command moves the r/w heads of the selected drive from

anywhere to cylinder 0. The controller waits for the drive to

complete the seek before the task file is updated and the busy

flag is reset. Upon successful completion of the command, the

error register and the cylinder registers are set to zero, while

SC, SN, and SDH remain unchanged.

Read Sectors (2xh):

This is probably the mostly used command. It will read from 1 to

256 sectors of disk data as specified in the SC register (with an

SC value of 0 meaning 256 sectors to be read). The starting

sector is defined by SN, C, and H in the task file.

When the task file contains invalid parameters, an error occurs.

Otherwise, the r/w heads are moved to the requested cylinder if

they are not already there (implied seek). Then the data of the

starting sector is read into the sector buffer, and the DRQ (data

request) bit in the status register is then set. This informs the

host that the sector data can be read from the sector buffer.

When this is completed, DRQ is reset and the drive is ready

again.

For reading multiple sectors (SC>1), when the sector buffer is

completely read and DRQ is reset, the busy flag is set again

immediately, and the next sector's data is read into the sector

buffer. When DRQ is set again, the next sector's data can be read

from the buffer. This is repeated until the SC register value

reaches zero (this register is decremented with every

successfully read sector).

In any case, after successful completion of this command, the SC

register contains a zero value, and the other registers in the

task file are updated to contain the cylinder, head, and sector

number of the last-read sector. If an error occurs, the task file

will contain the parameters of the sector at which the error was

detected.

For the Read Sectors command, two options are possible. The "M"

option is valid only with the original WD1010 controller (thus

with old AT's). When M was not set (0), the SC content was

ignored, and exactly one sector was read. When M was set (1), the

SC value was taken as the count of sectors to read. Today things

are different. With modern IDE drives, if the M bit is set, an

error occurs. So this bit must always be cleared!

The other option ("L") is valid only with modern IDE drives. "L"

stands for long and means that the additional ECC data, which the

drive automatically puts after each sector, is transferred after

the net data of each sector. When this option is set, the drive

also does not check these ECC bytes, so it won't detect or

correct errors. This provides a way to read a sector's

(remaining) data even if it is nonrecoverably damaged. When using

this option, remember that the ECC data is transferred as bytes

through the word-wide data register!

Write Sectors (3xh):

The Write Sectors command is very similar to the Read Sectors

command. Of course the data flow direction is different... This

command will write up to 256 sectors of data to the disk. All

parameters and options are similar to those of the Read Sectors

command. After writing the command to the command register, the

drive sets the DRQ flag, informing the host that the data can be

written into the sector buffer. When all data has been

transferred, DRQ is reset, and the drive starts writing the data

buffer contents to the disk. The busy flag is set as long as the

drive is physically writing to disk. The SC register is

decremented, and, if not zero thereafter, DRQ is set again for

the next sector. When using this command with the "L" option, the

drive will use the ECC bytes delivered by the host and not

generate any by itself. For the "M" option, the details described

above apply.

Scan ID / Verify Sectors (4xh):

This is a very strange command. As far as I know, it is the only

one that is totally incompatible between the old AT's hard disk

controller and today's IDE drives. It would appear that this

command was never used by common system implementation or

application software...

For the WD1010 controller, this is the Scan ID command. It takes

no parameters at all (except for the drive and head which

originally had to be contained in a register external to the

WD1010). When the command is started, the controller searches for

the next ID field and reads the contents into the task file. This

way the actual drive, head, cylinder, and sector size could be

examined. The sector number was also transferred into the task

file, so the sector numbering order could be figured out by

repeating this command fast enough.

For the IDE drives, this is a completely different command:

Verify Sectors. It is similar to the Read Sectors command except

that no data is transferred to the host, and the "L" option is

not supported. Thus, it needs all parameters in the task file. Up

to 256 sectors of data will be read into the sector buffer, and

their ECC bytes will be verified. The DRQ flag will never be set.

The completion status of the command can be read from the status

register.

It is interesting that both types of controller/drive support the

retry option - so this is the only compatibility of this command.

Format Track (5xh):

Originally, this command was used to physically format an entire

track of the hard disk, exactly as it's done when formatting

floppy disks. The Format Track command is started similarly to

the Write Sectors command: first the task file must be set up,

then the command written to the command register. After that, the

drive responds by setting the DRQ flag. The host must then write

data into the sector buffer until the DRQ flag is reset. After

that, the command is executed.

For the format command, the sector buffer must contain special

data. As with the index field array when formatting a floppy

disk, it must contain valid sector ID's for every physical sector

of the track that will be formatted, beginning at the start of

the buffer. Each sector ID in the buffer consists of two bytes.

The unused remainder of the buffer is ignored by the format

command, but must also be written for the DRQ signal to

disappear.

The first byte of each sector ID is a flag byte. The WD1010 knew

only two different values for this descriptor:

00h = good sector,

80h = bad sector.

Today's IDE drives offer two more descriptor values:

40h = assign sector to alternate location,

20h = unassign alternate location for this sector.

We'll look further at these values below.

The second byte of each ID is the sector-number byte. It contains

the number by which the related sector is referenced later during

normal r/w operation. The ID fields in the sector buffer are

assigned to the physical sectors (created through formatting) in

the order they are stored in the buffer. So it is possible to

define an interleave factor by appropriate physical sector

numbering. Here is an example:

Addr. 00 =  00 01  00 11  00 02  00 12

08 =  00 03  00 13  00 04  00 14

10 =  80 05  00 15  00 06  00 16

etc.

Here we see the first 12 (of 32) ID words. The starting sector

has number 1 (as usual). The interleave factor is two, since each

sector appears two sectors after its logical predecessor. You can

also see that sector number 5 (the 9th sector physically) is

marked bad.

Due to surface errors on the hard disk, there are some positions

where the media won't store magnetic information reliably enough

(if at all). The defect list for a particular drive then shows

the cylinder, head, and "BFI" (byte from index) value of the

defect. People then had to calculate the bad-sector position and

number from each of those BFI values. However, it is not commonly

known that the relationship between the BFI value and the sector

number depends not only on the sector size but also on the

interleave factor and the starting sector number...

Again, things changed as the years went by... I already mentioned

when introducing the features of modern IDE hard disks, that

those drives don't have defect lists any more, due to the usage

of internal spare sectors. For compatibility reasons, these

drives still accept the Format Track command. However, most

drives only simulate its execution -- internally they don't

really format any track. Modern drives are "hard-sectored" by the

manufacturer, with the sector size unchangeable by the user. But

by virtually formatting a track, one can assign new sector

numbers (for example, starting with 0 instead of 1). However, the

sector numbering order is often ignored. Because IDE drives

commonly have built-in cache memories, the definition of an

interleave factor would make no sense. So, the drive always uses

the fixed sector ordering which gives maximum performance in

combination with the cache.

To make things still more complicated, the Format Track command

of IDE drives allows for the assignment of data sectors to the

spare sectors and for the release of those assignments (look at

the descriptor bytes above). All IDE drives have some spare

sectors to which the data of defective sectors is automatically

mapped. Normally, there is one spare sector per track, resulting

in about 2-3% spare capacity. This is more than enough. When a

sector appears too unreliable during normal operation, the drive

simply marks that sector as bad internally and moves the data to

the nearest free spare sector. As long as not all spare sectors

are assigned, the user won't notice anything.

However, these assignments can also be done explicitly by use of

the Format Track command. But it is strongly recommended not to

do that! First, one will normally get no defect list for an

individual IDE drive containing the BFI positions. Second, even

if a sector which was assigned to one of the spares is marked

good again, the related spare sector can not be used again! So

with every unassignment of a spare sector, you loose that

irretrievably.

So we come to this result: with standard (i.e., ST-506) drives

and external controller (i.e., WD1010) it makes sense to format

the drive in order to freshen the surface magnetism, to get a

defined state (sector numbering and order), and to mark defect

sectors as bad (so that the operating system can behave

accordingly). With IDE drives, it's best to leave them just as

they are coming from the factory!

Seek (7xh):

This command is used to move the r/w heads to a particular

cylinder explicitly. For normal operation of the drive, it is

usually not necessary, since all r/w commands perform implied

seeks. However, this command can easily be used for benchmarks to

determine the drive's seek times. With the WD1010 controller, the

four lower bits of the command byte contain the step rate

(described above). IDE drives simply ignore these four bits.

Execute Diagnostics (90h):

This command is common to all IDE drives but not available with

the WD1010 controller. When issued, the drive performs an

internal self-test. If the drive is a master drive, and a slave

drive is connected to it, the master also waits a limited time

for the slave to complete its self-test. During all this time, it

is busy (the according flag in the status register is set). After

finishing the test procedure, its results are placed in the error

register. In this special case, the content of the error register

has to be considered as a single byte value, not as several bit

flags. There are the following error codes:

01h   no error detected,

03h   sector buffer error.

(These codes are supported by Conner drives. Maybe other

manufacturers use more or different codes.)

If the slave drive diagnostics failed, the MSB of the error

register is set, leading to values of 8xh. However, even with

single drive configurations this bit sometimes is accidently set.

It may be ignored then.

Set Drive Parameters (91h):

An IDE-only command again. After power-up or reset, the drive can

immediately be used in its default mode. However, the drive's

logical parameters can be changed by setting them with this

command. This way, the drive can be set up to different modes in

order to emulate the parameters of another common drive.

The task file registers which are used with this command, and the

way in which they are used, may differ. Some drives are really

flexible and allow any parameters that result in no more than the

drive's real capacity. Other drives (for example, my Conner CP-

3044) support only two or three modes with fixed parameters. So

for their selection, only part of the task file's registers are

needed. Most, if not all, drives will accept this command with

valid parameters in the SC, C, and H registers (even if not all

the parameters are required), defining the number of sectors per

track, cylinders, and heads.

Because of the differences, it is advisable to first collect

detailed information about the supported emulation modes of a

particular drive, before defining its operating parameters.

Normally, it's best to operate a drive in its native mode (so the

logical parameters equal the physical ones). However, there's

another strange detail: there are drives which don't support the

native mode! My Conner drive again serves as example: the drive

has 1053x2x40 sectors (cylinders by heads by sectors) physically,

but supports only a pseudo-native mode with 526x4x40 sectors, and

an emulation mode with 981x5x17 sectors (which is for

compatibility with older 40 MB drives). Additionally, depending

on the internal software version, the drive defaults to the

emulation mode or to the pseudo-native mode.

As a result, it is recommended that the operating parameters

always be defined after power-up or reset. And to define them,

you must have detailed information about the drive you want to

use. There is a "Product Manual" for every drive type, describing

all those details. Unfortunately, these manuals are hard to get.

Most dealers are not willing to give them to their customers (and

some even don't have them in stock). The other way is to try out

some parameters, starting with the information delivered by the

Identify Drive command.

The break - a sample program

I realize that I've already filled quite a few pages again. So

I'll make a break here and continue the command descriptions in

Part IV of the "Connecting IDE Drives" article series. Instead,

I'll show you a short program which reads the ID information of

an IDE drive within a PC/AT. This sample program was written with

Turbo Pascal 5.5 but may easily be used with any version above

4.0.

You can try out this program on your AT (if you have one with an

IDE drive) and play with it until receiving the next issue of TCJ

with Part IV of the article. That part will finish the command

descriptions and will also contain some more programming examples

and shortform tables as a programmer's overview of the IDE

interface definition.

Abbreviation list:

BFI       Byte From Index (position of surface defect)

DRQ       commonly used for Data Request (bit flag or signal line)

IDE       Integrated Drive Electronics (hard disk interface type)

I/O       Input/Output

PC/AT     Personal Computer/Advanced Technology (computer class)

r/w       read/write

ST-506    older hard disk interface standard, used between

separate controllers and MFM/RLL drives

Connecting IDE Drives, Part III

by

Tilmann Reh

In part I we covered the basics of the IDE interface in terms

of history, concept, hardware, and register structure. In part

II I started describing the various commands and parameters of

IDE drives. This time I will finish that command description

and offer some sample driver routines.

I must apologize!

Sorry for the badly formatted Pascal listing printed with part

III in the previous issue of TCJ. Bill had to delete all the

empty lines in order to compress it to a single page. Now I

know that this doesn't make a program more readable or easier

to understand, even if it's written in Pascal. We will try to

do this better in the future.

Commands Continued...

We already covered most of the manufacturer-independent

commands in the previous part. However, there are three

commands not explained yet. Let's get started with the command

which was already used in the sample program printed with the

previous part -- so you'll now know what you really did there

(in case you ran that program).

Identify Drive (ECh):

This command reads some detailed parameter information from the

IDE drive. Again, it's invalid for the older (external)

controllers. It is started by writing the command code into the

command register, and then it executes like a Read Sectors

command. The DRQ Flag will be set, declaring that data can be

read. After having read a complete "sector" (256 words, 512

bytes) of data, the DRQ flag will be reset and the drive will

be ready again. The data consists of the following fields:

Word Adr. Byte Adr.   Type    Content

-------------------------------------------------------------

0         0      word    Configuration/ID word

1         2      word    Number of fixed cylinders

2         4      word    No. of removable cylinders

3         6      word    No. of heads

4         8      word    No. of unformatted bytes per

physical track

5        10      word    No. of unformatted bytes per

sector

6        12      word    No. of physical sectors per Track

7        14      word    No. of bytes in the inter-sector

gaps

8        16      word    No. of bytes in the sync fields

9        18      word    0

10-19     20-39   20 char    Serial number

20        40      word    Controller type

21        42      word    Controller buffer size (in sectors)

22        44      word    No. of ECC bytes on "long"

commands

23-26     46-53    8 char    Controller firmware revision

27-46     54-93   40 char    Model number

47        94      word    No. of sectors/interrupt

(0 = no support)

48        96      word    Double word transfer flag

(1 = capable)

49        98      word    Write protected

50-255   100-511       -      reserved (read as zero values)

Some of these fields have special meanings. The

configuration/ID word consists of 16 single-bit flags. However,

I don't know for sure if their meaning is really manufacturer-

independent. The "controller type" word is encoded as a number

representing a particular type.

Configuration/ID word bit flags:

15    Non-magnetic drive

14    Format speed tolerance gap required

13    Track offset option available

12    Data strobe offset option available

11    Rotational frequency tolerance > 0.5%

10    Data transfer rate > 10 MB/s

9    Data transfer rate > 5 MB/s, <= 10 MB/s

8    Data transfer rate <= 5 MB/s

7    Removable disk

6    Non-removable disk

5    Spindle motor can be switched off

4    Head switching time > 15 us

3    Not MFM encoded

2    Soft sectored

1    Hard sectored

0    reserved

Controller type word values:

0    Not specified

1    Single ported, single sector buffer

2    Dual ported, multiple sector buffer

3    = 2, with look-ahead read capabilities

The string-type data fields (character arrays) contain plain

text information about the serial number, controller model, and

firmware revision of the drive. Each word holds two characters,

which must be displayed with the high-byte character first in

order to get readable results.

As far as I know, most IDE drives follow the data field

description above. However, there still are many things which

are manufacturer-dependent. Fortunately, these details are not

critical. To give you some examples: The controller model field

of Conner drives contains plain text with the complete drive

description like

"Conner Peripherals 40 MB - CP3044".

Seagate's IDE drives offer only a short cryptic ID string,

which sometimes doesn't even contain the drive type.

A very interesting difference, even between drives of the same

manufacturer, shows up with the "Number of

cylinders/heads/sectors" fields. Some drives show their

physical values there, independent of the active emulation mode

(for example, my CP-3044 does so). Other drives always show the

parameters of the active emulation, or those of the default

emulation mode. Surprising especially with my drive is that the

physical parameters can't be used for drive operation! As a

result, the data delivered by this command must be considered

carefully. However, it's normally possible to extract useful

information by reading the drive's ID information for several

different active emulation modes.

Read/Write Sector Buffer (E4h/E8h):

These are the last two common IDE commands. With these commands

it's possible to read or write the drive's sector buffer

directly. I haven't found any use for these yet, but probably

there is (at least was) one. In my opinion, these commands are

useless for normal operation.

Block Mode Commands (Read/Write/Set Multiple, C4..C6h):

By the use of these commands, one can access disk data in

larger blocks than the physical sector size. Several sectors

are grouped together and handled as a block of data. However,

many drives don't support this mode. I don't have detailed

information regarding the parameters. If a particular drive

supports the block mode, the details will surely be printed in

its user manual.

Power Commands (E0..E6h, except E4h):

The power commands are not supported by every IDE drive.

However, if they are, they are normally compatible. The power

commands are commonly used within portable computers (laptops,

notebooks, handhelds, or whatever the names are). They allow

for automatic or manual changing between normally four

operation modes:

Read/Write Mode     (4.2 W)   complete drive circuitry

operating

Idle Mode           (2.0 W)   motor running, r/w circuitry

turned off while no command is

active

Standby Mode        (0.5 W)   motor stopped, r/w circuitry

turned off, interface active

Sleep Mode          (n/a)     everything stopped, exit only

with reset

The power requirements mentioned in this table are those of my

Conner 42-MB drive. While no r/w operation is in progress, the

drive normally is in idle mode (also when being reset).

Read/write mode is always automatically entered when a r/w

command is issued; after completion of that command, the drive

enters idle mode again.

When the drive is put into standby mode (manually or

automatically, see below), the drive (motor, r/w circuit) is

shut down while the host interface remains active. So when a

command is issued which requires motor or r/w operation, the

appropriate circuitry is automatically switched on again.

Once the sleep mode is entered, there is no way out except for

reset by means of hardware or software. This is because even

the drive's local processor and interface controller are

stopped, so there is no way to communicate with the drive.

(However, the task file can still be read.)

As mentioned above, there are six power commands:

Set Standby Mode (E0h), Set Idle Mode (E1h):

The drive will enter the desired mode immediately. There are no

parameters required. If the drive already is in that mode, the

command will have no effect.

Set Standby (E2h) or Idle (E3h) Mode with Auto-Power-Down:

These commands take a parameter in the sector count register.

If that parameter is non-zero, the Auto-Power-Down (APD)

feature is enabled (with a zero value, APD is disabled). When

one of these commands is issued, the drive immediately enters

the desired mode. If APD is enabled, the drive will

automatically enter standby mode after being in idle mode

without activities for a given period of time. This delay can

be specified by means of the parameter for these two commands:

the SC register must contain the delay time in counts of 5

seconds. The minimum delay of 60 seconds will be set if the SC

register contents is smaller than 12. With a maximum value of

220, the maximum delay is about 18 minutes. These limits again

apply to my particular drive; other drives may have other

specifications.

Read Power Mode (E5h):

This command reads the actual mode. If the motor is spinning

(meaning that the drive is in idle mode), the value FFh will be

returned in the SC register. Else (when in standby mode or just

spinning up) a zero value will be placed in the SC register.

Set Sleep Mode (E6h):

This command puts the drive into sleep mode immediately. Every

internal activity is terminated and all circuitry switched off.

There are some more power-related commands, having the command

codes F8..FDh (except FCh). Their general meaning is similar to

the power commands described above (E0..E5h), except that the

time delays are specified more exactly (in counts of 0.1

seconds). However, I have not yet seen a drive which supported

these commands, and I don't have detailed information about

them.

Cache On/Off (EFh):

This is the last command which I will explain here. It is used

for enabling or disabling the automatic read-ahead feature (read

cache) of the drive. The write precompensation register (WP) is

(mis-) used as a parameter register for this command (today, this

is the only use of the WP register). If the WP register contains

AAh, the feature is enabled; with 55h, it is disabled. Every

other value will result in an aborted command error. After reset,

the drive defaults to read-ahead feature enabled.

Whew -- this was a lot of stuff! (I hope it was not too hard.)

However, now you should know about IDE commands in detail (if

you didn't fall asleep while reading). Before we start

practical work, here, for the programmers, are the short-form

tables that I promised.

Table 1: Task File Registers (as printed in part II)

/CS0 /CS1 A2 A1 A0  Addr.   Read Function       Write Function

0    1   0  0  0   1F0    Data Register       Data Register

0    1   0  0  1   1F1    Error Register      (Write Precomp Reg.)

0    1   0  1  0   1F2    Sector Count        Sector Count

0    1   0  1  1   1F3    Sector Number       Sector Number

0    1   1  0  0   1F4    Cylinder Low        Cylinder Low

0    1   1  0  1   1F5    Cylinder High       Cylinder High

0    1   1  1  0   1F6    SDH Register        SDH Register

0    1   1  1  1   1F7    Status Register     Command Register

1    0   1  1  0   3F6    Alternate Status    Digital Output

1    0   1  1  1   3F7    Drive Address       Not Used

Table 2: Error Register

Bit  Flag  Meaning

7    BBK  Bad block mark detected

6    UNC  Uncorrectable data error

5      -  -

4   IDNF  Sector ID not found

3      -  -

2   ABRT  Command aborted (status error or invalid command)

1    TK0  Track 0 not found during recalibration

0      -  -

Table 3: SDH Register

Bit  Flag  Meaning

7    EXT  Extension Bit. Always 1.

6-5  SIZE  Sector Size. Always 01 (512 byte sectors).

4    DRV  Drive bit. Master/single drive = 0, slave = 1.

3-0  HEAD  Head field. Binary head number 0..15.

Table 4: Status Register, Alternate Status Register

Bit  Flag  Meaning

7    BSY  Drive busy. Task file cannot be accessed.

6   DRDY  Drive ready (up to speed and ready for command).

5    DWF  Drive write fault.

4    DSC  Drive seek complete (actuator on track).

3    DRQ  Data request (ready for data transfer).

2   CORR  Corrected data (bit is set when data has been

recovered by use of ECC).

1    IDX  Index. Active once per disk revolution.

0    ERR  Error. See other bits and error register.

Table 5: Digital Output Register

Bit  Flag  Meaning

2   SRST  Software reset (active when set to 1).

1   /IEN  Interrupt enable (active when set to 0).

Table 6: Drive Address Register

Bit  Flag  Meaning

7      -  not driven (for PC floppy compatibility)

6   /WTG  Write gate (active when 0)

5-2  /HSx  Head select 3..0, one's complement of active head

1   /DS1  Drive 1 selected (active when 0)

0   /DS0  Drive 0 selected (active when 0)

Table 7: Commonly needed Commands with Parameters

Code Command                       Parameters

1x  Recalibrate                   D

20  Read Sectors with retry       SC,SN,C,D,H

30  Write Sectors with retry      SC,SN,C,D,H

40  Verify Sectors with retry     SC,SN,C,D,H

50  Format Track                  C,D,H

7x  Seek                          C,D

90  Exec Diagnostics              D

91  Set Drive Parameters          SC,(C),D,H

Ex  Power Commands, see below

E4  Read Sector Buffer            D

E8  Write Sector Buffer           D

EC  Identify Drive                D

EF  Cache On/Off                  D,WP

Power Commands:

E0  Standby Mode                  -

E1  Idle Mode                     -

E2  Standby Mode with APD         SC

E3  Idle Mode with APD            SC

E5  Read Power Mode               (SC)

E6  Sleep Mode                    -

Table 8: Error Conditions

When  an error occurs, the error flag in the status register (ERR)  is

always  set.  For  the different groups  of  commands,  the  following

status/error flags are valid then:

Recalibrate                     ABRT,TK0,DRDY,DWF,DSC

Read, Verify                    BBK,UNC,IDNF,ABRT,DRDY,DWF,DSC,CORR

Read Long, Write, Write Long    BBK,IDNF,ABRT,DRDY,DWF,DSC

Format, Seek                    IDNF,ABRT,DRDY,DWF,DSC

Diagnostics, Initialize, R/W Buffer,

Identify, Set Cache           ABRT

Invalid command                 ABRT

Table 9: Interrupt Conditions

The  drive  generates an interrupt (if enabled)  under  the  following

conditions:

Recalibrate                   after successfully reaching track 0

Read                          each time DRQ is set

Write                         when DRQ is set, from second sector on

(only when multiple sectors are written)

Verify                        after completion for all sectors

Format Track                  after completion

Seek, Initialize, Power

Commands (except Sleep)    after command is issued/initiated

Set Sleep Mode                when drive is in sleep mode

Read Buffer, Identify         when data is ready for reading

Now let's come to the example routines for accessing an IDE

drive. These examples are given as Turbo-Pascal (3.0) source

(based on my IDE test program). They apply to the use of my IDE

interface board (described in TCJ #56), so there always are 512

data bytes transferred instead of 256 data words.

In all examples, named constants are used for accessing the IDE

registers at their particular I/O addresses. These named

constants must be declared elsewhere. Their names are derived

from the related IDE register names and IDE commands.

The examples are programmed in a very modular fashion so that

they are easy to understand. For implementation in a system

BIOS, for example, most of the subroutines will contain so

little code that the complete read/write routines will normally

be coded inline. In addition, a real implementation, unlike

these examples, will have time-out functions in most loops. If

someone is interested in the IDE driver of my CPU280 system

BIOS, please contact me (however note, it's Z280 assembly

language and commented in German).

1. General access: Wait for drive ready / wait for data request

In Pascal, two small procedures serve this purpose. In assembly

language, I use two macros instead, because the subroutine

calling overhead would be too much.

procedure Wait_Ready;

begin

repeat until port[IDE_CmdStat]<=128;

end;

procedure Wait_DRQ;

begin

repeat until port[IDE_CmdStat] and 8<>0;

end;

2. General access: Command issue

procedure IDE_Command(Cmd:byte);

begin

Wait_Ready;

port[IDE_CmdStat]:=Cmd;

end;

3. General access: Reading/Writing the sector buffer

In the Pascal implementation, the two functions return a

Boolean value which is true if there were no errors during r/w

of the buffer.

Both routines require the drive to be ready for data transfer!

function Read_SecBuf(var Buf:BufType):boolean;

var i : integer;

begin

Wait_DRQ;

i:=port[IDE_Data];  (* specific to my IDE interface board *)

for i:=0 to 511 do Buf[i]:=port[IDE_Data];

Read_SecBuf:=port[IDE_CmdStat] and $89=0;

end;

function Write_SecBuf(var Buf:BufType):boolean;

var i : integer;

begin

Wait_DRQ;

for i:=0 to 511 do port[IDE_Data]:=Buf[i];

Wait_Ready;

Write_SecBuf:=port[IDE_CmdStat] and $89=0;

end;

4. General access: First access, initialization

procedure HD_Init(Cyls,Heads,Secs:integer);

begin

port[Dig_Out]:=6;

delay(10);            (* Drive Software Reset *)

port[Dig_Out]:=2;

Wait_Ready;

port[IDE_SecCnt]:=Secs;

port[IDE_CylLow]:=lo(Cyls);

port[IDE_CylHigh]:=hi(Cyls);

port[IDE_SDH]:=pred(Heads)+$A0;

IDE_Command(Cmd_Initialize);

end;

5. Data access: Single sector read

function HD_ReadSector(Cyl,Head,Sec:integer; var Buf:BufType):boolean;

begin

Wait_Ready;

port[IDE_SecCnt]:=1;

port[IDE_SecNum]:=Sec;

port[IDE_CylLow]:=lo(Cyl);

port[IDE_CylHigh]:=hi(Cyl);

port[IDE_SDH]:=$A0+Head;

IDE_Command(Cmd_ReadSector);

HD_ReadSector:=Read_SecBuf(Buf);

end;

6. Data access: Single sector write

function HD_WriteSector(Cyl,Head,Sec:integer; var Buf:BufType);

begin

Wait_Ready;

port[IDE_SecCnt]:=1;

port[IDE_SecNum]:=Sec;

port[IDE_CylLow]:=lo(Cyl);

port[IDE_CylHigh]:=hi(Cyl);

port[IDE_SDH]:=$A0+Head;

IDE_Command(Cmd_WriteSector);

HD_WriteSector:=Write_SecBuf(Buf);

end;