PARK  Version 0.4

MFM/RLL Hard Disk Head Parker

Freeware from Kirk Lawrence

PARK is a convenient way to "park" the heads of your hard disks, if they

require software head parking to protect the data area.

PARK.CMD can park most hard disks.  The program gets the disk parameters by

calling function 8 of interrupt 13h, then moves the heads directly to the

highest available cylinder number and leaves them there.

If two hard disks are installed in the system, PARK will park both of them.

If only one hard disk is installed, that one hard disk will be parked.

After the heads are parked, an on-screen message tells you what action has

been taken, and prompts you to turn off the power to the computer...or, to

press a key if you wish to return to CP/M-86.

NOTE: Only certain MFM and RLL hard drives require software head parking to

protect the data area of the disk.  Many MFM and RLL drives, and virtually all

IDE and SCSI drives, are self-parking; they automatically ratchet to a non-data

area of the disk when the power is turned off.  It's not necessary to use PARK

with a self-parking hard disk.  If in doubt, consult your hard disk's instruc-

tion manual.

PARK.CMD is free.  As is always the case with "freeware," there are no guaran-

tees whatsoever.  If you use this program, you do so entirely at your own risk

and peril.  Your use of PARK.CMD shall be prima facie evidence under law that

you understand and accept these terms and conditions.