MCI MAIL FILE TRANSFER PROGRAMS                        by Jack Decker      As most NORTHERN BYTES readers know, I have beenadvocating the use of MCI Mail as a cost-effective method oftransferring files between computer users.  However, there isone problem with MCI Mail - it cannot be easily used totransfer certain types of files.  These programs remove thatlimitation.     The problem stems from the fact that MCI Mail is basicallya text transfer medium.  Plain vanilla ASCII text files areeasily transferred, but there is no automatic error checking.BASIC programs can be transferred if saved in ASCII format, butonce again there is the lack of error checking and in addition,lines cannot be greater than 132 characters long for reliabletransfer.     MCI Mail cannot handle ASCII characters outside the rangeof normal text - that is, under ASCII 32 ("control" characters)or greater than ASCII 127 ("graphics" or "special" characters).But, virtually any file that is not one of the two types listedabove will contain such characters.  What do you do if you wishto transfer, for example, a short machine language object orsource code file to someone else?     The programs below will take ANY regular TRS-80 ModelI/III type files and convert them to a format that can betransmitted over MCI Mail.  Even Model 4 (TRSDOS 6) formatfiles can be sent using these programs, but the programsthemselves will only run in the Model III mode on a Model 4 or4P.  You can send ANY type of file, be it text, compressedBASIC, ASCII BASIC (with any line length), word processing,Editor-Assembler source, machine language, or whatever.  TWOchecksums are maintained to inform you of any errors in thetransfer.     The programs work by converting a file containing bytes inthe range 0-255 ASCII to a file containing bytes in the range32-126 ASCII.  Text bytes in the range 32 through 123 ASCII arenot converted, but are sent as-is.  Other values are convertedto a two-byte format.  As an example, bytes in the range 0-32ASCII have a value of 32 added to them, and then are sent witha "prefix byte" of 124.  The chart below may make this a bitclearer: ORIGINAL            PREFIX BYTE         VALUE ADDED TO/ASCII VALUE         VALUE               SUBTRACTED FROM BYTE 0   -  31           124                 + 3232  - 123           none                none124 - 175           124                 - 60176 - 255           125                 - 144      ASCII 126 is used as an "end-of-file" marker, and ASCII127 is not used since it sometimes has a special meaning tocertain terminal programs.     The individual bytes of each file are transmitted as shownabove.  After 64 characters on a line have been transmitted, a"line checksum" byte is transmitted followed by a carriagereturn.  The "line checksum" will only detect gross errors(where an entire byte has been garbled, for example) because itis obtained by adding each transmitted byte on the line to thepreceding byte, then at the end of the line it is ANDed with 63to mask off the upper two bits and then a value of 32 is addedto make it a printable character.  Note that the upper two bitsare not checked at all!  Thus, if only two consecutive bitswere garbled and they happened to be the upper two bits of abyte, the error would not be detected by the "line checksum"(but would still be detected by the main checksum).  The mainpurpose of the line checksum is to help point out the line inwhich an error occurs, so that the recipient of the file has anoppertunity to attempt manual correction (possible only in sometypes of files).     Note that the above formula used to derive the "linechecksum" can yield a character in the range 32-95 ASCII, sothere is no chance of it being an "unprintable" character.     When the end of the file is reached, an "end-of-text"(ASCII 126) byte is transmitted, followed by the "linechecksum" for the line (which may be a "short" line of lessthan the normal 65 characters).  A carriage return is thensent.  The master checksum is then sent on a line by itself, inplain ASCII format (in other words, using the numerals 0-9).The master checksum may be in the range -32768 to 32767(keeping it within integer limits), and is the sum of all filebytes (not including "line checksums" or carriage returns).Should the checksum exceed the 32767 limit, a value of 65536 issubtracted and the count continues.  Those of you into assemblylanguage will recognize why these values have been used.  I'msort of hoping that someone will convert these programs tomachine language someday (I might, if I can ever find thetime), so I set up the master checksum so that it could beaccumulated in a register pair or two-byte memory location.Note, however, that the checksum is printed on the final linein decimal rather than hexadecimal format, with a leading minussign if the number is negative.     The program FILTOMCI/BAS converts a file to the formatshown above.  You must give it an input filename (the file youwant to convert) and an output filename (the file you willupload to MCI Mail).  It will then make the conversionaccording to the specifications given above.  If you'reconverting a long file, this may take awhile (BASIC is nothingif not slow!).     MCITOFIL/BAS does the reverse, taking the received fileand converting back to a mirror image of the original file.One caution must be observed by the person receiving such afile - before you download it, use the ACCOUNT command to setyour terminal line length to a value greater than 65 (either 80or 132 is suggested).  If you don't do this you'll get extracarriage returns and MCITOFIL/BAS won't be able to convert theprogram.  Also, you PRINT rather than READ your received mailto avoid getting prompt strings mixed into the program.  Ifyou've already read a file and realize you didn't do it right,remember that you have up to 24 hours to go back and re-readany mail on your "desk".  Just use ACCOUNT to set the linelength, then have MCI Mail PRINT the contents of you DESK (oryour INBOX, if you've just started reading your mail for thefirst time).  Open your terminal program's text buffer and savethe received mail to disk (MODEM80 from TAS works quite wellfor this purpose).     When you're ready to use MCITOFIL/BAS, you have the optionto skip lines at the start of file, which you should do if yousaved the header information to disk, or if the person sendingthe file sent some text for you to read prior to sending theprogram.  Just keep pressing the <S> key until you see thefirst line of the transmitted file, then press <ENTER> andMCITOFIL/BAS will do the rest.     Note that these programs could also be used with otherelectronic mail services (such as Western Union's EASYLINK),although I haven't tried it with any of them (by the way, didyou know that MCI MAIL and EASYLINK users can communicate witheach other by using the TELEX facilities of each service?  It'sprobably pretty expensive to do it this way, but it can bedone!).  Similarly, these programs could probably be convertedfor use on other versions of BASIC without too much difficulty.The one portion of MCITOFIL/BAS that might confuse even some ofyou "experts" is in lines 30 and 90.  What this does is to setup a string storage area (referenced by BASIC as V$) on theupper right-hand corner of the video display, so that a stringcontaining the line count can be "stored" (and thus displayed)there.  I have to give credit to Alan Abrahamson, editor ofVoice of the '80 (newsletter of the Fairfield County ComputerUsers Group) for giving me the idea for this one.  He used thistechnique in a BASIC program to move data from one place toanother on the video display.  Alan explains:     "The technique of using the dummy variables to block movethe DATA using "LSET" [I used RSET to pad the left side of thestring with spaces] is about ten times the speed of a similarPRINT statement.  The LSET [and RSET!] command appears to do aZ80 block move (LDIR,LDDR) between the VARPTR's (pointers) ofthe variables involved.  This technique can be used in anyscreen data manipulation routine to move data from point A topoint B.  Remember that the LSET routine physically moves thedata from one place to another, it is NOT just a pointer swap.     "The key ingredient in using this method is to establishthe dummy or real variables prior to using LSET [or RSET].  Itis not necessary to OPEN any files in using LSET, but youcannot use LSET on a variable that has not yet beeninitialized..." [I added the comments in brackets.]     The above two paragraphs were included for the benefit ofthe advanced programmers in the crowd.  If they didn't make anysense to you, then all you need to know is that line 30 (thepart after the CLS) and line 90 will probably have to bedeleted if you attempt to convert this program for any othersystem.  The purpose of those lines in this program was to letme print a running line count at a fixed location on the videodisplay, without changing the current cursor location (so thatif any "checksum error" type messages were being printed, theywouldn't keep overprinting the same line).     Two warnings about these programs:  One, they were throwntogether in some haste and do not contain any great amount oferror trapping.  There's no telling what they'll do if you usethem incorrectly.  Two, note the version number (1.0).  We canalways hope they are bug free, but that's not the way to bet.Besides that, I'm really hoping that others will see fit toimprove upon these programs and then share the results with therest of us.  In any case, watch future editions of NORTHERNBYTES for corrections and/or enhancements.     And one final note:  There is a program called HEX/CMDthat is included

in the MODEM80 package that does somethingsimilar to what I'm doing here, but the only problem with it isthat it converts ALL bytes within a file to two hexadecimalcharacters.  When you're paying by the character, that may bejust a bit too much code expansion, especially when the fileyou're converting is mostly text but with an occasional controlcode or bit of graphics or a machine language segment thrownin.  The truth is that unless you deliberately go out of yourway to set up a situation where this is not the case, ANYconverted file will be shorter, in many cases MUCH shorter,when converted using the programs below as opposed toconversion using HEX/CMD.  However, HEX/CMD is in machinelanguage and does the job much faster, so if you're onlysending a short file anyway, it may pay to use that programinstead.