PRINTGRF/BAS     Another Public Domain Program from the Keyboard of :-     BRIGGS SOFTWARE - 14 ALLAN BERRY AVENUE - NAPIER - NZPRINTGRF/BAS is a Basic program that will create a M/L filefrom Graphics Data Statements.  This file will considerablydecrease the time taken to print the Design.If you are designing a Logo or Letterhead to be run from aBasic Program, the normal procedure is to design the Logo onGraph Paper.  The resultant picture is then converted to Datastatements by dividing your design into rows of 7 or 8 lines.(This depends on the Printer but I will assume an EPSONcompatable from here).  Each of these rows would of coursecorrespond to one pass of the Printhead.  To be able to printthe design each vertical set of 8 "squares" in the row isconverted to a number by adding together the Decimal valueswherever the Lines of your design cross any "square" ofGraph-paper.e.g.  If we assume this is one small section of a Picture drawnon Graph-paper, and we use the HASH Symbol to illustrate eachsquare of your graph paper that is part of the design :- Row 8 :-   #   #           #       #     Decimal Value = 128 Row 7 :-   #       #               #     Decimal Value =  64 Row 6 :-   #       #                     Decimal Value =  32 Row 5 :-   #       #           #   #     Decimal Value =  16 Row 4 :-   #   #   #       #   #   #     Decimal Value =   8 Row 3 :-   #   #       #                 Decimal Value =   4 Row 2 :-   #   #       #       #         Decimal Value =   2 Row 1 :-   #   #       #   #   #   #     Decimal Value =   1           === === === === === === ===Value  :-  255 143 120   7 137  27 217    etc, etc, etcThese Values are then put in DATA Statements and output to theprinter.  This is done with a suitable loop which reads theData and pokes it into the Printer Memory address (or Port).eg :-100 DATA 27,64,27,65,8,10,27,65,9,10,27,65,8,10,27,65,8,10120 DATA 27,65,9,10,27,76,n1,n2,x,x,x,x,x,x,x,x,x,x,x,x,etc130 DATA 27,65,8,10,27,76,n1,n2,x,x,x,x,x,x,x,x,x,x,x,x,etc140 DATA 27,65,9,10,27,76,n1,n2,x,x,x,x,x,x,x,x,x,x,x,x,etc150 DATA 27,65,8,10160 REM  (if used on a TRS-80 Mod I)170 IF (PEEK(14312) AND 240) <> 48 THEN 170 ELSE READ P:    POKE 14312,P: GOTO 170160 REM  (if used on TRS-80 Mod III)170 IF (INP(248) AND 240) <> 48 THEN 170 ELSE READ P:    OUT 248,P GOTO 170160 REM  (If used on SYSTEM 80/PMC 80)170 IF (INP(253) AND 240) <> 48 THEN 170 ELSE READ P:    OUT 253,P:  GOTO 170NOTE :- A M/L File generated on a TRS-80 MODEL I (or SYSTEM 80)will not run on a MODEL III and Visa Versa.This routine, which is very slow, can be made to run very muchfaster by making a M/L file of the Data lines.  To do this editthe Data lines to remove any unwanted codes and insert BlockMarkers (400) at the end of each Line of Dot AddressableGraphics and 300 at the end of the File.  This may soundcomplicated but is really quite easy if you study your PrinterHandbook and gain an understanding of the control codes.The row of 27,65,(8 or 9 etc),10 are codes to alter the PaperFeed and a Line Feed.  Where there is a line of them, most maybe deleted.  In most cases the 400 Block Marker will be usedinstead of the Line feed code (10).The 27,76,n1,n2,x,x,x,x,x,x is the DOT ADDRESSABLE graphicscodes as described above.  These are the "Meat in the Sandwich"and should not be altered.When edited the above Data lines would look like this - -120 DATA 27,76,n1,n2,x,x,x,x,x,x,x,x,x,x,x,x,400130 DATA 27,65,8,10,27,76,n1,n2,x,x,x,x,x,x,x,x,x,x,x,x,400140 DATA 27,65,9,10,27,76,n1,n2,x,x,x,x,x,x,x,x,x,x,x,x,400150 DATA 300PRINTGRAF/BAS has been written to use line numbers under 100 soit is important to start you Data Statements from above thisLine Number.  Now enter 'MERGE"PRINTGRF/BAS', and when theMerge is complete, type 'RUN'.  The conversion takes time butit gets there.  When the Data is loaded the number of Bufferswill be shown.  (In the above case it would show 3)  Theprogram will then prompt you to enter a Filename and Drivenumber and DUMP the File to Disk.To use, write a simple Basic program - -10 POKE &H40B1,&HFE: POKE &H40B2,&HDF: CLEAR 100020 REM Protect the Memory above E000 Hex30 CMD"LOAD,filename/ext"40 DEFUSR1=&HE000: REM Set up the user call50 A=USR1(1):LPRINT" Test of the First Pass";60 REM The 1 in the User call = The first Buffer or Pass70 A=USR1(2):LPRINT"Test of the Second Pass";80 REM The 2 = the second Buffer90 A=USR1(3):LPRINT"Test of the Third Pass";100 REM The 3rd and final Pass120 The rest of the Program herePOINTS TO REMEMBER - - Write down the number of passes (orBuffers). In the above example there were 3.  Values of 0, ormore than the maximum number of Buffers will be ignored.  Ifprinting along side a row of Dot addressable Graphics, (as inthe above example) let the Graphics control the line feeds,they vary depending on the characters.  A Line Feed or CHR$(10)can be used after each USR Call if it is found necessary.PRINTGRF/BAS creates a M/L file starting at E000 Hex.  It isnot required to POKE &H40B1 & POKE &H40B2 to protect Mem, itcan of course instead be set when first entering BASIC, BUT itMUST BE PROTECTED above E000 Hex by one or other of the abovemethods.