Lesson 1

How to Use the ALT

What is ALT?

ALT stands for “Assembly-Language Tutor?’ We abbreviated this to ALT. A LI is an assembly-language program that will teach you assembly language. It contains:

•	An editor, similar but not identical to the BASIC program editor. This portion of ALT will help you construct text lines that represent the “language” portion of assembly-language programs. A typical line might be:

100 START	LD	A,23	;load A reg with 23

This particular line is eventually decoded as an instruction to the computer that says “At a location called START we’ll put an instruction to load a value of 23 into the A register.” No need to worry about what that instruction does now; just observe that you can talk to the computer via assembly language by plain text, such as line 100 above.

•	An assembler. This portion of ALT takes a sequence of assembly-language lines like the one above and converts them into numbers that the microprocessor can recognize. This conversion process is called “assembly?’ The line above, for example, would be converted to the decimal numbers of

3E 17

which, when input to the microprocessor in your Model I or Ill would cause a value of 23 to be put into a type of memory location called A.

•	A “debug” package. This portion of ALT will let you “eXecute?’ or run, your assembly-language programs. It will let you look at certain memory areas in which results are stored, change the areas at will, and generally let you control things.

•	An interpreter. This portion of A LI is the “controller?’ Assembly language is so fast that it’s hard to see what is happening. The interpreter interprets each assembly-language instruction and slows it down to anywhere from dozens of instructions per second to one instruction every few seconds.

At the same time, the interpreter displays the current instruction on the screen so that you can follow exactly where you are in the program and what is happening inside the microprocessor and memory.

Another important facet of the interpreter is that it doesn’t allow you to lose control by putting in incorrect assembly-language instructions. You can’t “blow-up” your programs, as the interpreter will tell you that you’ve made an error.

All the parts of the ALT are in one package. Once you load it, you don’t have to switch between different parts, but have complete control of the assembly-language program.

There’s another part of A LI also. You’ll find a number of cassettes with your system. On the cassettes are a series of 26 lessons. These lessons contain complete assembly-language programs that correspond to each lesson in the text.

You can proceed at your own pace with the lessons; we would recommend, however, that you try to complete each lesson at one sitting.

How to Load ALT

The first step of every lesson is to load the ALT program. The ALT program is on cassette.

You must have at least 16K of “RAM” (random-access memory) and Level II BASIC (Model I) or Model III BASIC (Model III) to be z~ble to use ALT.

•	Turn on your system. For Model III systems, answer the

1 How to Use the ALT

Cass?

prompt with L to select 500-baud tapes

Type an ENTER for

Memory Size?

Load the cassette tape marked ALT into the cassette player as you would any other tape to be loaded.

Rewind the cassette

Type SYSTEM and ENTER

You should see a

?

displayed on the screen.

Type ALT and ENTER

A LI should now begin loading. You should see asterisks flashing at the upper right-hand corner of the screen during the load.

After the load is done, you should see another

displayed on the screen

Type / followed by ENTER

ALT should now start executing, and you should see a “menu” of items. Now sharply hit ENTER, and you’ll see the display of Figure LESS I-I

Load ALT now so that you get the display shown above. After the load, go on to the next discussion.

Loading Lesson Files

At the beginning of each lesson, you will be asked to load the Lesson File for the lesson. These Lesson Files all have the same type of name

LESSXX

where “XX” is the number of the lesson. The file for Lesson 20, for example, is called

LESS2O

To load a Lesson File from cassette:

•	Load ALT as in the above description

V

How to Use the ALT 1

•	Put the Lesson Tape in the cassette recorder

•	Rewind the tape

•	Enter L followed by space, followed by LESSXX, followed by ENTER (“XX” is a number of I through 26, corresponding to the Lesson chapter number)

•	You should see the message

READY TAPE

displayed in the “message area” of the screen, as shown in Figure LESS 1-2

1 How to Use the ALT

What the Display Represents

There are 5 areas of the display, as shown in Figure LESS 1-4.

Figure LESS1-4. Display Areas

The first “band” on the screen shows the Z-80 microprocessor “registers?’ The registers are high-speed memory locations that are located not in RAM memory, but inside the microprocessor itself. The microprocessor is the small “chip?’ or integrated circuit, that is the heart of all the “logic” of your system.

We’ll be working continually with these registers in assembly language. We’ll describe them in detail in the next lesson.

The register names are shown on the top line. On the next line, the register “contents” are shown, directly below the corresponding register names. You’ll see the contents line change as the program executes or as you modify the contents of the registers under ALT.

The next “band?’ after the asterisks, isa text area of 6 lines. The first line is fixed and is a heading for the 8 “fields”of STATUS, LOCN, CONTENTS, LINE, LABEL, OPCODE, OPERAND, and COMMENT.

The right portion of the text area shows the assembly-language text lines. This is the text that is in the Lesson File or that you have entered for an assembly-language program.

Just as in BASIC, each line of text has a “line number” associated with it. You should see the text for Lesson I displayed on the screen at this point.

The left portion of the text area normally displays the “output” of the assembler portion of ALT. The lines in this area correspond to the lines of text. They are the “machine-language:’ or microprocessor code, equivalent of the text lines. At this point, you should not see anything in the left portion of the text area.

The next “band” after the asterisks is a memory “trace” area. This area displays the contents of memory locations that you select.

To see how it works, type

ZT 8000 followed by ENTER

You should see the display shown in Figure LESS 1-5, which represents the 8000 hexadecimal memory area of RAM (the values may be different). (If you have a 16K RAM system use “ZT 7000”to see sample data.)

4

How to Use the ALT 1

1 How to Use the ALT

You should see the last line of Lesson 1, line 440. The * character is a special character that stands for the last line of the text.

To get back to the first line of text, type

P#

You should see the beginning of the text. The 4 character is a special character that stands for the beginning line of the text.

To display the text from any line, type

PLLL

where LLL is the line number. To display from line 20~ on, for example, type

P200

You should see the text from 200 on displayed.

A special form of the P command lets us “scroll through” the text. Entering

P

alone, displays the “current text.” Entering one P after another displays the next 5 lines of text at a time. Try it now.

To get back to the start, end, or any given line, just use the P4, P*, or PLLL form of the command.

Deleting Lines

You can delete lines from the text file by using the D command. To delete line 100, for example, you’d type

Dl 00

Get back to the beginning of the text by typing P#. Do a delete of line 100 by Dl00 now.

You should see line 100 disappear and the remaining lines “scroll up’ Another form of the command is

DNNN:MMM

This form deletes lines NNN through MMM. Try to delete lines 120 through 140 now.

Did you use

D120: 140?

If so, you saw the lines deleted and a scroll to the next lines of text.

Inserting Lines

The third Edit Mode command lets you insert text. There are two conditions for this insert command.

If you do not have text in the “buffer” you can use the I command to create new text. You’d want to do this if you were creating a new assembly-language program, for example.

To see how this works, delete all the text in the buffer by

D#:.

6

How to Use the ALT 1

You should see the text disappear from the screen entirely.

Now type

1100. 10

You’ll see a “100” under the LINE heading appear. You can now type anything you’d like (at least in this lesson). Go ahead, type until you get to the end of the line.

If you typed text until the end of the line, you saw a new line number appear line 110. ALT automatically terminates a line if more than 34 characters are entered.

The 100 in the I command says “start at line 1002’ The 10 says “increment by 102’ You could keep adding text up to the limits of the text buffer. You can use any start number and increment you’d like (Ill and 13, for example), but the “normal” ones often used are 100 and JO.

To end any line just type ENTER.

Try using the Insert mode for a while. When you want to get back to the next Command, press BREAK. BREAK stops the insert mode and returns you to the ALT Command Mode for the next command.

BREAK wilt return to the ALT Command Mode under most conditions.

Type Q (Quit) while in Command Mode to return to TRSDOS.

To Sum It All Up

At the end of each Lesson, we have a little section entitled “To Sum It All Up” It will review what you’ve learned in the lesson.

In this first lesson we’ve covered:

•	ALT is a program that consists of an editor, assembler, interpreter, and debugging package

•	ALT allows you to execute assembly-language programs under your control and has preventative measures in it to keep the program under control

•	The display for ALT has 5 areas that display the contents of registers in the Z-80 microprocessor, the assembly-language text and machine-language for the program, a selected memory “trace” area, the current ALT mode and conditions, and the current command

•	The P, D, and I Edit Mode commands let you display, delete, and insert lines of text for the program

•	The BREAK key will usually return you to the Command Mode

For Further Study

Appendix I ALT Commands

7