Instrument Landing------------------Using the VOR receiver as a guide a pilot can navigate accurately from onecity to another without any view of the earth below.  VOR radials aresuitable for point-to-point navigation, but when a pilot arrives at hisdestination he needs another system of guidance to get to the airportrunway itself.  In this case the pilot must revert to a radio aid, theInstrument Landing System (ILS), a facility designed to make blind landingspossible.  A trained pilot flying an aircraft equipped with an ILS receivercan locate an airport and safely land on a runway that may not be visibleuntil a minute or so before the actual touchdown.An ILS installation consists of a group of radio transmitters arranged inthe vicinity of the airport where ILS landings are to take place.  Thesetransmitters radiate highly directional radio beams that converge at thefoot of the runway, forming a cone-shaped pattern like the rays of asearchlight.  The pilot first maneuvers the plane into this invisible cone,then uses the ILS receiver to follow the radio waves down until the air-craft is just a few hundred feet above the ground.  At this low altitudethe runway should be visible, so the actual landing can be completed inthe usual way.The airborne instruments used to locate and follow the cone of radio wavesare a marker lamp, an ILS indicator, and a radar altimeter.  On the JETSETsimulator panel these three components are identified as the MARKER, ILS,and RADAR ALT respectively.  The panel MARKER lamp flashes on when theaircraft flies over a point called the "outer marker" telling the pilotthat the plane has just entered the ILS cone.  The crosshairs (horizontaland vertical needles) of the panel ILS meter will now begin to deflect,and the pilot must maneuver the plane to keep the needles centered in orderto follow the path of the ILS radio cone.  As the aircraft descends alongthis narrow path, the radio altimeter (RADAR ALT) gives a continuousdisplay of the exact elevation from the ground (in feet).The radio alti-meter is much more sensitive than the conventional altimeter, so it isalways used for precision landings.During the time the aircraft has entered the ILS cone and is headingtoward the runway, when the pilot is making the final approach, the planeflies in a direction known as the "localizer" direction of the ILS radiobeams. The angle that the radio cone makes with the ground is called the"glidescope" angle, and the descending plane is said to be flying withinthe ILS "glidepath". The two moving needles of the ILS indicator correspondto the localizer and glideslope axes during the final approach.  The pilotchases the vertical needle (which moves left and right) to remain alignedwith the localizer direction.  The horizontal needle (which deflects upand down) must be chased using the elevator controls to keep the planewithin the glidepath.Once the descending aircraft reaches the ILS "middle marker", the panelMARKER lamp will flash again, alerting the pilot that the plane is just afraction of a mile from the runway.  This critical location is called the"decision height" of the final approach because the pilot must now decidewhether he can safely complete the landing.  If the runway appears in viewdirectly ahead, the pilot can make a visual landing.  If, however, theplane is not properly lined up with the runway (because the ILS needleswere not kept centered), the pilot must abort the landing attempt at onceby climbing out of the glidepath.  This situation is known as a "missedapproach".  When a pilot misses the approach, he flies a safe distanceaway from airport traffic and then returns to the OM point for another try.Every ILS equipped airport uses an arrangement which places the VOR stationaway from the airport in such a way that the plane will cross the ILS conenear the outer marker.  The exact ILS arrangement (localizer direction andglidescope angle) for any given airport is published in a manual of approachdiagrams (one for each airport), which the pilot studies well in advance ofhis instrument landing.Obviously, an instrument landing is a tricky procedure that airline pilotsmust practice in large-scale simulators to perfect.  The routines that sim-ulate landing are an important part of the JETSET program; they closelyfollow the sequences that develop when a plane flies into the ILS pattern.You may have to make several attempts at a simulated landing before youcan consider yourself qualified to handle a jetliner under bad weatherconditions.Flying Lesson #5 - Practicing ILS---------------------------------Preparing for an instrument landing, even aboard the JETSET simulator,begins when the plane is still many miles away from the airport.  Becauseall ILS landing procedures follow a standard pattern, the John F. Kennedy(JFK) International Airport, conveniently located with respect to Phila-delphia, can serve as a practice landing site.  A simulated flight fromPhiladelphia to JFK lasts about 20 minutes from takeoff until the jet rollsto a stop on the runway.Every airline flight must be conducted in accordance with a flight plan, adocument that specifies the routes the pilot will fly until he arrives atthe destination.  An actual flight takes place at standard altitude levelsand under close supervision of air traffic controllers, but the flight planprepared for the practice run to JFK International tells the JETSET pilotexactly how to proceed.  See Table 4.Using the Philadelphia-JFK flight plan as a guide, execute the takeoffprocedure and climb to 5000 feet while maintaining a compass course of 075degrees. During the climb, tune your VOR to the JFK ground station (115.9MHz) and input the radial value of 058 degrees.Level off at an altitude of approximately 6000 feet.  Use the "<" key forthe left rudder to alter the compass course to approximately 000 degrees.HOld this course until the VOR needle nears its center position.  Now steerto 058 degrees and begin chasing the VOR needle.The jet will head directly for JFK as long as you keep the VOR needlecentered-the 058 degree radial is used because it's the "initial approach"radial defined for JFK airport. It will lead to an intercept with therunway outer marker (OM), a prerequisite for the instrument landing.As soon as the DME indicator reads 38, you must prepare for landing.  Tobegin a descent, adjust, adjust the elevators for a pitch of -10 degrees(press the key twice) and level off at an altitude of about 1900 feet.Start the "initial approach trim" procedure for the jetliner when the DMEdistance is 20 NM. First reduce your airspeed to 300 knots (S key), lowerthe landing gear (W key), and lower the wing flaps (L key).  The airspeedwill automatically drop back to 120 knots as soon as the flaps are lowered,as required for a proper landing.  Complete the trim procedure by adjustingaltitude until the ALTITUDE indicator reads between 1700 and 1900 feet.You must execute this procedure quickly so that the aircraft is in proper"profile" or flight configuration as it approaches the OM along the initialapproach radial.  You will reach the OM when the DME reads exactly 12 NM, sothe jet should be in its trim profile and steered to keep the VOR needlecentered (to within two graphic dots) as the OM point nears.If you've done these steps carefully, the panel MARKER lamp will flash whenwhen the DME indicator reads 12 NM.  This is a signal that the aircraft hasjust intercepted the ILS radio cone and must be promptly steered to align withthe localizer direction (028 degrees) at JFK airport.Press the left rudder (< key) quickly when the MARKER lamp flashes.  It'simperative that you swing the jet to a compass course of 028 degrees beforeit flies out of the narrow area of the radio cone (this would occur about15 seconds after the MARKER lamp turns on).  A compass reading of 028degrees (give or take one degree) before the MARKER lamp goes off willensure that you completed the turn in time for the jetliner to enter theILS radio cone.  Both the ILS indicator and the RADAR ALT meter should beactivated.  If not, the turn took too long to complete and you need morepractice in making a fast turn.  For another attempt, you can stop thesimulation program and begin again or raise the flaps and wheels and circleback to pick up the initial approach radial for another attempt.The rapid updating of the ILS indicator means the jet is now beginning itscrucial final approach.  You have very little margin for error.  The programwill automatically change the sensitivity of the elevator and rudder keys;each press of the elevator key varies the pitch by one degree and the coursechanges by one degree each time a rudder key is pressed.  Quickly press the"D" key three times to pitch the nose down 3 degrees and turn your attentionto the ILS display.You must use the rudder keys to chase the vertical needle of the ILSindicator as the jet loses altitude (as shown by the RADAR ALT reading).If ;the ILS horizontal needle moves from center, chase by using theelevator keys.  Crosswinds blowing across the airport will tend to deflectthe jet (and the vertical ILS needle), so you must make every effort tokeep the two ILS needles where they belong-exactly on center.The RADAR ALT indicator, a meter that activates when the final approachbegins, shows the elevation of the descending jet (feet above groundlevel).  At an elevation of about 600 feet, JETSET will display theapproaching runway on the lower-right portion of the screen to simulatethat the ground is now visible. The arrow appearing at the foot of thegraphic screen shows the exact alignment of the jet in relation to theapproach end of the airport runway.  You mut now use this visual referenceinstead of the ILS

indicator to quickly correct any course errors.  Forexample, if the arrow extends too far to the left, beyond the runway base,apply some right rudder to realign the jet's path.After a few more seconds the MARKER lamp shold flash again to announcethat the plane has just reached the middle marker point along the approachpath, the decision-height location.  Now a quick decision is vital.  Ifthe arrow of the runway graphic extends too far left or right, beyond therunway base, the jet is not properly lined up for a safe landing and youmust press the "M" key immediately to signal a missed approach to thecomputer.  JETSET will comply by announcing that the pilot's decision wascorrect for the landing situation.If however, the runway arrow shows that the jetliner is safely aligned fora landing, you must bring it down as follows:     1. At an elevation of 100 feet (RADAR ALT reading), press the "S" key        once.  This command will "chop the throttle" (abruptly reduce the        engine thrust to idle).     2. At 50 feet, press the "C" key once to "flare up" the nose of the        jet. This maneuver automatically tilts the aircraft upward slightly        to a positive pitch, causing a controlled stall.  The jet will        now sink gently down to ground level as it loses aerodynamic lift.     3. At 0 feet the jet has landed and is rolling along the runway.        Quickly press the "Q" key to apply reverse thrust to the engines.        Reverse thrust decelerates the ircraft gradually until the AIRSPEED        readout reaches zero.Your JETSET flight concludes with a display of the landing informationthat  tells you how well you handled the jet.   This information  specifieswhere ground contact occurred and where the jet finally rolled to  a  halt.If  you made a mistake at the middle marker,  the landing report will printout the consequences.This is only a small part of the capabilities of the JETSET simulator.There are about 15 to 20 additional airports built in.  It is conceivablethat you could fly all over the United States.  Remember though, thissimulator flies in real-time.  If it takes 6 hours to fly from New Yorkto San Fransico in a real aircraft, it will take the same 6 hours flyingthe simulator. Table 1. Listed below are the keyboard keys, functions, and definitions:        KEY   FUNCTION            DEFINITION         ---   --------            ----------          F    THRUST INCREASE*    INCREASES POWER TO JET ENGINES          S    THRUST DECREASE*    DECREASES POWER TO JET ENGINES          Q    THRUST REVERSE      REVERSES ENGINE THRUST DURING LANDING          D    PITCH DOWN*         LOWERS NOSE OF AIRCRAFT BY 5 DEGREES          U    PITCH UP*           LIFTS NOSE OF AIRCRAFT BY 5 DEGREES          \    PITCH CANCEL        SETS NOSE TO LEVEL FLIGHT          <    RUDDER LEFT*        INCREASES RUDDER LEFT BY ONE INCREMENT          >    RUDDER RIGHT*       INCREASES RUDDER RIGHT BY ONE INCREMENT          /    RUDDER CANCEL       RETURNS RUDDER TO CENTER POSITION          L    FLAPS               RAISES AND LOWERS WING FLAPS          W    WHEELS              RAISES AND LOWERS LANDING GEAR          B    BREAKS              RELEASES WHEEL BRAKES FOR TAKEOFF          M    MISSED APPROACH     SIGNALS AN ABORTED LANDING ATTEMPT          V    VOR FREQUENCY TUNE  INPUTS A FREQUENCY TO VOR RECEIVER          R    VOR RADIAL SELECT   SELECTS A RADIAL VALUE FOR NAVIGATING          A    VOR AUTO SELECT     AUTOMATICALLY ROTATES RADIAL SELECTOR DIALNOTES:   1.   The CAPS LOCK key must be engaged throughout the simulation.          2.   An asterisk (*) identifies keys that may be typed additional               times to increase their control function.Table 2.  Instrument Panel Legend--------  ---------- ----- ------Instrument  Units         Function----------  -----          --------FUEL        pounds,%       fuel aboard (in puounds and percentage full)VHF         MHz            communications channelTHRUST                     position of engine thrust leversPITCH                      attitude of aircraftDEG         degrees        angle of pitch, measured from horizontalCOMPASS     degreds        compass heading of aircraft (direction of nose)AIRSPEED    knots          aircraft velocity through the airVERT SPEED  feet/minute    rate of climb or descentALTITUDE    feet           altitude above the groundCLOCK       hr.min.sec     time of day (local)VLF OMEGA   degrees,min    aircraft position (latitude and longitude)RUD                        rudder angleFLAPS                      flaps positionWHEELS                     landing gear positionBRAKE                      position of wheel brakesVOR         MHz            frequency to which VOR receiver is tunedRANGE                      status of VOR receiverRADIAL      degrees        value of selected radial (needle moves along                              window directly above radial)DME         nautical miles distance to VOR ground stationRADAR ALT   feet           aircraft elevation during final approachMARKER                     turns on when flying directly over the ILS outer                              and middle marker beaconsILS                        pair of needles that deflect according to aircraft                              position in ILS radio coneSTALL                      flashes as aircraft is stalled during final approachTable 3. Locations and frequencies of simulated VOR ground stationsLOCATION        FREQUENCY    LATITUDE         LONGITUDE--------         ---------    --------         ---------Philipsburg, PA  ll5.5 MHz    40 deg 55 min N  77 deg 59 min WJFK, NY          115.9        40     38        73     46Boston, MA       112.7        42     22        70     59Buffalo, NY      116.4        42     56        78     39Flint, MI        116.9        42     58        83     44Green Bay, WI    117.0        44     33        88     12Joliet, IL       112.3        41     33        88     19Cleveland, OH    113.6        41     22        82     10TAKEOFF PROCEDURE------- ---------A.   Lower flaps (L key).B.   Release breaks (B key).C.   Apply full throttle (F key).D.   Steer  along the 075-degree runway using the left/right rudder keys     (< and >).  Coordinate steering with the COMPASS reading and the position     of the arrow located at the base of the runway graphic.E.   As soon as the AIRSPEED indicates 150 knots,  press the U key once to     gently lift the jet off the runway.F.   After  the  horizon line drops below the screen,  press the W key  to     raise the landing gear.G.   Retract the flaps (L key).H.   Throttle back the engines (S key).I.   Sit back and relax for a minute or so as the jet gains altitude.PRACTICE FLIGHT TO BUFFALO-------- ------ -- -------A.   Execute the takeoff form Philadelphia as described above.B.   Level off at 10,000 feet.C.   Steer approximately north.D.   Adjust airspeed to 600 knots.E.   Tune to the frequency of the Buffalo VOR station (115.5 MHz).F.   Input the value of 278-degrees radial into the receiver.G.   When  the  VOR needle moves to center,  alter course  to  295-degrees     (COMPASS).H.   Now steer to keep the VOR needle centered.  This indicator, not the     compass, will provide exact guidance for the remainder of the flight.I.   Use the DME indicator to keep track of the distance remaining, in nautical     miles, to Buffalo.  To estimate the remaining flying time (in minutes),     simply divide the DME reading by 10.J.   When the DME readout reaches zero, the jet has arrived.INSTRUMENT LANDING---------- -------A.   Execute the takeoff procedures.B.   Continue  to  climb  to an altitude of 3000 feet on a  course  of  075     degrees.C.   At  3000  feet,  alter course  to 000 degrees and  continue  climbing.     Adjust  thrust  for airspeed of 580 knots.   Tune VOR  to  Philipsburg     station (115.5 MHz), and set radial to 278 degrees.D.   Steer  along 278-degree radial when intercepted.   Level off at 40,000     feet and proceed to Philipsburg at 600 knots.E.   At DME=20 NM,  retune VOR to Buffalo (116.4 MHz) and set radial to 346     degrees.F.   Upon  intercepting the 346-degree radial,  alter course to follow  the     radial to Buffalo.G    At  DME=73 NM,  begin decent to 1900 feet  (descend  at  approximately     11,000 FPM).H.   Level off at 1900 feet.  Remain aligned with the radial.I.   Begin initial approach trim when DME=20 NM as follows:     1.   Reduce airspeed to 300 knots (S key).     2.   Drop landing gear (W key).     3.   Lower the flaps (L key).     4.   Adjust altitude to between 1700 and 1900 feet (elevator keys).     5.   Keep the VOR needle centered (rudder keys) to stay on the initial          approach radial.J.   Be  alert for the flash of the MARKER lamp  (which  occurs  when  the     DME=12).   At  this  signal the jet must be maneuvered for  the  final     approach:     1.   Quickly swing the nose until the compass shows 042 degrees.     2.   Use  rudder and elevator keys to keep the ILS  indicator  needles          centered as the jet descends along the glidepath.     3.   As  soon as the runway graphic appears on  the  screen,  use  the          graphic arrow as a guide to apply rudder corrections.K.   When the MARKER lamp flashes again to announce arrival at the decision-     height point, check the runway alignment using the graphic displayed on     the screen.   If necessary, press the M (Missed Approach) key to abort     the landing attempt.  Otherwise, if the plane is lined up safely, take     all cues from the RADAR ALT from here on in:     1.   At 100 feet, idle the engines (S key).     2.   At  50 feet, flare up the nose (\ key).     3.   At   0 feet, the jet is on the runway.   Slow it down by

applying          reverse thrust to the engines (Q key).FLIGHT PLAN - PHILADELPHIA, PA TO BUFFALO, NY------ ----   ------------- -- -- -------- --1.  After takeoff, continue climbing to 3000 feet on course 075 degrees.2.  At 3000 feet alter course to 000 degrees and continue climbing.  Adjust    thrust for airspeed 580 knots, tune VOR to Philipsburg station (115.5 MHz),    and set radial to 278 degrees.3.  Steer along 278 degree radial. When intercepted, level off at 40,000 feet    and proceed to Philipsburg at 600 knots.4.  At DME = 20 NM, retune VOR to Buffalo (116.4 MHz) and set radial to 346    degrees.5.  Upon intercepting 346-degree radial, alter course to follow radial to    Buffalo.6.  At DME = 73 NM, begin descent to 1900 feet (descend at approximately    11,000 feet per minute).7.  Level off at 1900 feet.  Remain aligned with radial.8.  Begin initial approach trim when DME = 20 NM.9.  Execute ILS final approach procedures when MARKER lamp flashes.  Localizer    direction is 042 degrees.FLIGHT PLAN - PHILADELPHIA, PA TO JFK INTERNATIONAL, NY------ ----   ------------- -- -- --- -------------- --1.  After takeoff, continue climbing to 6000 feet on course 075 degrees.    While climbing, tune VOR to JFK station (115.9 MHz) and set radial    to 058 degrees.2.  Level off at 6000 feet.  Steer left to intercept radial, align with it,    and proceed toward Long Island, NY at 400 knots.3.  At DME = 38 NM, begin descent to 1900 feet (descend at approximately    7410 feet per minute).4.  Level off at 1900 feet.  Remain aligned with radial.5.  Begin initial approach trim when DME = 20 NM.6.  Execute ILS final approach procedures when MARKER lamp flashes. Localizer    direction is 028 degrees..This Flight Simulator program was converted to run on the Mod 4 by the effortsof Tim Downey.The FLISIM4/BAS program has been saved in ASCII to obtain the maximumcompression when archived.  Because of this, expect a much slower loading timethan normal.  Have fun!!!George Brann, SysOpLast Chance BBSSan Antonio, Tx512-822-7519300/12008/N/1#