WARNING: This program uses instructions that are specific to the HD64180.It will NOT run on a standard Model 4. You must have an XLR8er installed.This program has been tested on a Gate Array Model 4P with XLR8er, uLabsgraphic board and a 20 Meg Hard Drive.                            XLR8er Graphics                            ===============   The XLR8er board has many features that can be used to improve the Model 4.One of these features is Direct Memory Access (DMA). The HD64180 actually hastwo DMAC's but we'll only be using one of them with this program. XLRHR/CMDdisplays /HR type files using DMAC1 to transfer data from memory to thegraphics board.Use the following command from LS-DOS Ready:XLRHR filename/HRA file extention is optional. A default extention of /HR will be used if noextention is given on the command line.Program execution is as follows:   1> Fetch picture filename from command line.   2> Clear graphics screen and display Logo.   3> Transfer disk file to memory starting at x'4000'   4> Program DMAC1 to transfer picture to graphics bd. 80 bytes at a time.   5> Picture display complete. Wait for keypress then exit to Dos.   If you have any problem with this program please send a complete reportgiving your hardware configuration (ex. RS or uLab Hi-Res board).Mike HarrowCIS ID 72507,2435GEnie  M.HARROW2File Update 04/23/90NOTE: IF THE PROGRAM FAILS TO LOAD THE HR PICTURE-------------------------------------------------If the program displays the XLR8er Graphics logo but fails to display the/HR picture you've specified on the Command line please read on.The program is executing properly but DMAC1 of your XLR8er board needs to beenabled via hardware. XLRHR/CMD uses OTIR to display the graphics logo thensets up DMAC1 for a transfer from memory to I/O (graphics board). The DMA won'tstart the transfer until DREQ1 is pulled low - at pin 54 of the HD64180 chip.Apparently some XLR8er boards already have this pin low. Unfortunately yoursisn't one of them. The result - only the graphics logo will be displayed givingyou the impression the program is at fault and doesn't work.I've modified my XLR8er board to permanently enable DMAC1 so that programslike XLRHR/CMD will work. If you decide to modify your XLR8er please becareful. The mod requires a resistor (150 ohms) be soldered from pin 54of the HD64180 to ground. Pin 1 of the HD64180 is a convenient ground point.This mod will allow XLRHR/CMD to work and Frank Slinkman's GRAM/CMD (seeTMQ IV.iii).If you aren't the type of person to disassemble your computer and soldercomponents then DON'T even attempt this modification. This type of job isbest left to an experienced Technician or Electronics Hobbiest.