SIMPLE SPEED MOD                      ==================Here's a simple modification for your Model 4. It requires noadditional hardware and it will bring your computer to its true4Mhz speed. I should add that I have used my machine for about5 years after making this simple change with no ill effects.One thing you must consider: This modification has worked on mymachine, but I cannot guarantee the results on yours. If mymemory serves me right, credit for this tip should be given toLarry Paine.Whith that out of the way, open your machine; if you need me totell you how to do it, you better get someone else to do it.Remove the shield that covers the computer's main board in theread of the machine.Locate the Z-80 chip (CPU). Pull it out carefully. Bend backpin 27. Reinsert the chip making sure that pin 27 does not makecontact with anything.Replace the shield and the machine's cover. That's it!What the modification does is to remove the wait states in theZ-80, which are not really needed in your Model 4. To test thedifference it makes, run a program such as CLKSPD/CMD beforeand after the modification.Luis M. Garcia-BarrioSysop of 8/N/1 #4215-848-5728