** ZMP Documentation **

11 Oct 88

1. Introduction.

ZMP is  a  communications/file  transfer  program  for  CP/M  which

performs  Xmodem,  Xmodem-1k  (often  erroneously  called  Ymodem), true

Ymodem and Zmodem file transfer protocols.  Although tested with Z80DOS,

ZRDOS  and  CP/M  2.2, there seems to be no reason why it shouldn't work

with CP/M 3 as well.  The only requirements are a Z80  processor  (sorry

about that!), a computer running CP/M in one of its various guises, with

at least 45k of TPA (but the more the better!), and a modem.

When you try to pack this many features into one program,  you  end

up with a pretty large file.  The big problem occurs when file transfers

are attempted: unless you have at least 4-8k of buffer size,  you  might

as  well  use  xmodem  protocol.  The  approach  taken  in ZMP is to use

overlays for various functions, accepting the time taken to  load  these

from  disk.  Thus performance will vary depending on your disk setup: if

you have a hard disk and a fast processor you will likely not notice the

difference.  If, on the other hand, you are running a Commodore 128 with

CP/M on a 1571 drive, there is no physical reason why  ZMP  won't  work,

but  you  might  consider investing in a book to read while the overlays

load.  (A suitable book to read might be a computer catalogue!).

The curious amongst you  may  notice  that  the  beginning  of  the

ZMPX.COM  file has the magic 'Z3ENV' string, but don't let this fool you

into thinking that you don't need to add terminal  characteristics  into

the  overlay  if  you  have  a  ZCPR3 system.  It has proved possible to

persuade this  particular  C  compiler  to  access  ZCPR3's  environment

descriptor, but  not for ZMP.  Yet. Perhaps later.  In the meantime, the

startup code is there for it.

In order to produce a program  which  would  work  with  most  CP/M

systems,  the  Zmodem  protocol  performed by ZMP is fairly simple.  The

transmit section uses 'Full Streaming with Reverse Interrupt', as  Chuck

Forsberg  calls  it  in  his  description  of  the Zmodem protocol.  The

receive section uses 'Segmented Streaming'.  This  means  that, if  your

system  can  do  serial  I/O and disk I/O at the same time, ZMP does not

take advantage of this.  Since, however, I can't write and listen at the

same  time,  and  neither  can  my  computer,  and  neither can the vast

majority of CP/M computers, it seemed the best approach to take.

2. Customisation.

ZMP  must  be  customised  to  suit  your  system.   This  involves

overlaying  the  un-installed copy of ZMP.COM (contained in this library

as ZMPX.COM) with a user-written installation  overlay.  Some  hints  on

writing  this  are  given  below:  there is a blank overlay file in this

library, or you may be able to obtain one for  your  computer  from  the

same place you got this library.  Note that there is one variable in the

overlay file which will need to be changed to suit  future  versions  of ZMP:  the  'userdef'  equate.  This occurs since ZMP is written in C and

it's not easy  to  maintain  the  user-defined  functions  at  the  same

address.  The  current  value of  'USERDEF'  is contained in the ZMP.FOR

file in this library.

Once the installation overlay is written, assemble it with M80  (or

SLR  or  whatever),  use  RELHEX to create a .HEX file, and use MLOAD to

overlay it over the ZMPX.COM file to produce your very own ZMP.COM.

3. Operation.

The following files must be on the  same  disk  and  user  area  as

ZMP.COM:

ZMCONFIG.OVR -- the configuration overlay

ZMINIT.OVR   -- the initialisation overlay

ZMTERM.OVR   -- the terminal overlay

ZMXFER.OVR   -- the file transfer overlay

ZMP.HLP      -- the help file (recommended)

Start  the program with ZMP.  The screen should clear, then a title

message is printed, then ZMP enters terminal mode. You may type escape-H

for help at this point.  When you first run the program, the first thing

you need to do is to enter the configuration overlay (type escape-C) and

set all the defaults and others to suit your system as required.  If you

don't know whether to change something or not, it's probably  better  to

leave  it alone.  When you exit the configuration program, answer 'Y' to

the 'Make changes permanent' question.   ZMP.FON  and  ZMP.CFG  will  be

produced on your disk.

Operation  of the program is controlled by escape sequences entered

in terminal mode.  Escape-H gives you a list of options. Most  of  these

are self-explanatory, but they will be summarized here.

B - Send Break to modem.

If your overlay has been set up to send a  break  command

to  your  UART, this command will perform this function. (Some

remote systems also may need  a  break  sent  to  them  during

Zmodem transfers).

C - Configure system.

This  function  is  designed to set system defaults, save

phone numbers etc.  Changing baud rates etc., for a particular

call  are  better  handled  with  the escape-L option.  If you

answer Y or y to the 'make changes permanent?'  question,  the

new  configuration  will  be  saved in a ZMP.CFG file, and the

phone numbers in a ZMP.FON file for later use. The  .CFG  file

is read, if it exists, when ZMP is first started.

D - Get disk Directory.

Gets  a  directory  of  the  current drive and user area.

Change to another with the escape-F command, described below.

F - Disk/File operations.

This command is used to  change  the  current  drive/user      area, to reset a disk in the current drive, and to view, erase

or rename files.  A menu is presented: operation is obvious.

H - Get Help.

Prints the ZMP.HLP file.

I - Initiate phone call.

Reads the ZMP.FON file, if any, and prints it.  Then  you

are asked which number you want.  Enter the identifying letter

of the number you wish to call, or you can also enter  numbers

not  in  the list.  Multiple numbers can be called by entering

them separated with commas.  Dialing  can be aborted  with the

escape key.  Note that the baud rate used for the call is that

in the .FON file for that number, or the current one if a  new

number is entered.

K - Display Keyboard macros.

The  configuration  option allows up to ten macro keys to

be defined, and these are recalled by typing  escape  followed

by  the  numbers  0  to  9.  This function  prints the current

assignments.

L - Change  Line  settings.

This  function  allows  temporary  changes of baud rate,

stop bits, and data bits.  There is also an option to  operate

terminal  mode  in  full  duplex (locally typed characters are

sent but not displayed on the screen),  half  duplex  (locally

typed  characters  are sent and also displayed on the screen),

or echo mode (as for half duplex, but received characters  are

echoed  to  the remote system as well as being displayed).  Do

not run two computers in echo mode unless you're  bored.  Like

IMP, ZMP uses location 003C hex to store a value corresponding

to the present baud rate.  Then, if you leave  ZMP  and  later

re-enter,  it  checks  location  003C.  If it contains a legal

value, then that baud rate is set for you, else  it  sets  the

default baud rate as selected in the .CFG file.

M - Toggle capture mode in Memory.

Received characters will be saved in a buffer  and  saved

in  a  file  named 'ZMP.LOG' when the buffer fills or when the

command is entered again.  A control-S/control-Q  sequence  is

sent to the remote computer while the buffer is being saved in

an attempt to stop it sending more data.

P - Toggle Printer.

This is similar  to  the  'M'  command,  except  incoming

characters  are  sent  to the printer.  This functions best if

your system performs the BIOS 'List Status' function correctly.

Q - Quit the program.

Obvious.  You will be asked if this is  really  what  you

want.  Any entry other than N or n will exit to CP/M.

R - Receive a file.

You  will  be  asked which protocol you wish to use.  The      default is ZMODEM.  The  <X>modem  option  will  allow  either

128-byte  blocks  (standard  XMODEM) or 1k blocks (XMODEM-1k),

since this is decided by the transmit end.  Note  that  if  an

attempt is made to receive a file which has the same name as a

file on the current drive/user area, the current one  will  be

renamed  to  .BAK  and  the  new  one  will  then  be received

normally. Note that the byte count on the screen is  not  kept

up-  to-date  on  Zmodem  receive.   This  is because the data

arrives non-stop and there is simply no  time  available  with

non-  interrupt  driven  computers  to  update the screen.  An

update is performed if errors occur,  and  when  the  computer

pauses to write to the disk.

S - Send a file.

Operation  is similar to the receive function. Additional

options  available  are  ASCII  send  and  the  capability  of

distinguishing between normal Xmodem and Xmodem-1k.  In Ymodem

and Zmodem modes, wildcard filenames  and  multiple  filenames

are  allowed.   Multiple filenames should be entered separated

by spaces.  In all cases, files on different drives/user areas

may  be  specified by supplying a zcpr3-style du: prefix (e.g.

C7:NEATPROG.WOW).  Byte  count  information  is  displayed  in

Zmodem mode on transmit. This causes noticeable breaks between

packets, but it  is  felt  that  this  is  outweighed  by  the

usefulness  of  the information.  ZMP's Zmodem mode is capable

of CRC-32 opera- tion, although CRC-16 mode  is  used  if  the

receiver  is  incap-  able  of  CRC-32.   Some  other terminal

programs,  however,  do  strange  things  when  faced  with  a

receiving  program  which  claims  it  can  do CRC-32 (we have

encountered one for the Amiga which  exhibits  this  problem).

If  this  happens,  the  configuration  function  (esc-C  from

terminal mode) has an option to disable CRC-32.

X - Hangup.

This function causes the modem  to  disconnect  from  the

phone line.

Y - Print screen.

Allows  the  current screen to be dumped to printer. Note

that this must be supported in the overlay: most terminals are

incapable of this function.  The standard overlay prints 'This

function not supported.'.  If you can make  it  work  on  your

system, good luck!

Z - Clear screen.

Allows the screen to be cleared.  Useful if it fills with

rubbish.

4. Hints for overlay writers.

Most  of  the  code  is  quite straight forward and will present no

problem. The section most likely to cause trouble is that part concerned

with  setting  the  required  baud  rate  and  (especially) the code for

setting the correct parity, number of stop bit(s) and data bits.

We suggest that you first get the main part of the  code  going  by

putting  a  return  in the init: section at the point where it says "put

your code here" This will let you check the rest of the  code.   If  the

opening  screen  looks 'neat and tidy' then you know that 1) the code is

in the right place, and 2) your cursor addressing is working. Check  the

I/O  by talking to your modem and then by exchanging a file with another

zmodem user/board after you set the I/O chip  up  with  another  program

such as IMP.

Now  get the baud-setting working and then finally the bit- bashing

required to set the parity, etc.

5. Acknowledgements.

ZMP was developed from Hal Maney's  Heath-specific  HMODEM  II.  We

would like to thank Hal for writing HMODEM: CP/M users have been without

ZMODEM capability for far too long.  As requested in  the  source  file,

acknowledgement  is  given to him therein.  Appreciations also go to the

authors of the Hi-Tech  'C'  compiler, which  proved  to  be  capable of

producing fast and compact code for Z80 machines.

6. Finally...

The  files  contained  in  this  library  are  placed in the public

domain.  Just don't sell it, claim you wrote it, or do anything  similar

that might annoy us.  We haven't distributed the source files partly due

to the size of them, partly due to the fact that  compilation  is  messy

(several  modifications  were  needed to "standard" library functions!),

and partly due to the fact that  there's  still  work  to  do  on  them.

Perhaps later.

We  are  currently  working on a version of ZMP for use with remote

systems.  Called (predictably) RZMP, an elementary  version  is  up  and

running  on  Z-Node  62  in  a test mode.  Some cleaning up is necessary

before general release, but be assured that it's on its way!

Comments, suggestions and bug reports are welcome. Send the to:

Z-Node 62

Perth, Western Australia

(061+) 09-450-0200

-- Ron Murray

-- Lindsay Allen

11 October 1988

. ................................... .                                 . .    Downloaded in 1989 from:     . .                                 . .      ********************       . .      *     8/N/1 #1     *       . .      *   904/377-1200   *       . .      * Gainesville, FL  *       . .      * Guy Omer - SYSOP *       . .      ********************       . .                                 . . Software support for the TRS-80 . ...................................