INTERRPT/CMDA MODULE BASED ON NEWDOS80/V2 [MODEL I VERSION] TO INTERRUPT A    RUNNING PROGRAM AND SAVE A COMPLETE CORE-IMAGE TO DISK.   THE INTERRUPTED PROGRAM MAY BE RESTARTED SOME LATER TIME                      by Joachim Kelterbaum         (Frankenstr. 305, 4300 Essen 1, West Germany)      If you are the type of user who has programs that takehours to run, read on.  Probably you have experienced a suddenreboot after 2 hours or so of running your favoriteapplication.  The whole time spent was just wasted, because youhave to start over again - hoping not to get another one ofthose nasty reboots.     Well, how does the following sound?     Start your application and save the actual contents of RAMincluding registers, stack etc. to disk (about every hour).Then just resume execution. If you have an unwanted reboot orsilent death or whatever, start from where you last saved RAMto disk.  This can save some time and aggravation in the longrun.     To activate the module just enter INTERRPT.  If yourapplication polls the keyboard from time to time via the devicecontrol block (BASIC does this all the time to detect SHIFT-@)you may just press SHIFT-CLEAR.  From this very moment thewhole machine status is saved to disk onto a file calledMEMORY/CIM:0 (it has to be created before the first applicationand must allow for 39 granules of space).  After the status hasbeen saved, a reboot is done.  Now you can either resumeexecution by entering INTERRPT,* or run anything else beforeyou do so.  That's all there is to it.     You will find a commented source listing of the module inthe file INTERRPT/ASM.  As it is (at least I hope so) wellcommented, I'll only deal with the major aspects of operationhere:     If you enter the program via INTERRPT (not followed by"*"), first the original keyboard driver address is pluggedinto the instruction SCALL (line 1750).  Then the moduleSCALL...END is relocated directly below HIMEM and its new entryaddress is stored to RELAD and to the DCB address.  HIMEM isupdated accordingly.  Then the first sector of the fileINTERRPT/CMD is written back to disk.  This permanently updatesRELAD for a future restart.     The instructions RLCT (line 830) to DONE (line 1090)update a few absolute addresses in the relocated module usingRELTBL as a relocation table.  This has to be done because notall of the Z80 code is 'truly relocatable'.     After this a sign on message is displayed ((FLAG) = 0) anda jump to DOS is done.  Now the new keyboard intercept routineis active (lines 1750 to 1820).  If and only if SHIFT-CLEAR ispressed, a jump to SHFCLR is done.  Then all registers aresaved to stack and the stackpointer is stored to STACK = 4023H(2 unused bytes in the video DCB).  After this RAM from 3C00Hto FFFFH except for 4D00H-51FFH (DOS overlay-area) is saved tothe file MEMORY/CIM:0.     Note that there is no explicit file buffer specified inthe open file command.  Instead, IX is loaded with the FCBaddress, and IX+3, IX+4 (that is the address of the associatedfile buffer) is changed.  By this means you save additional 256bytes for a buffer.  The buffer itself sort of 'floats' throughRAM.  When all RAM has been saved (ENDIT, line 2220) the fileis closed, a message (MS) is displayed, a wait for a fewseconds is done and a reboot is forced by the HALT instruction.     If you enter the routine by INTERRPT,* a jump to STAR(line 1250) is done.  First the stackpointer is moved into3FF0H (video RAM) so it won't be in the way while reloading theoriginal RAM contents.  This trick only works if you have allbits available in the video RAM (i.e. lowercase must beinstalled) -as far as I have been told, it does not work withthe PMC80 for other hardware reasons-.     After this, the module (SCALL-END) is relocated to theaddress (RELAD) which has been written to disk before.  Thekeyboard driver address is plugged again and a jump to RLCT isdone, where some addresses in the relocated module are fixed upagain.  As (FLAG) = 1 now, the program continues at STARNT(line 1520).  Then the entry address of RESTOR (line 2400) inthe relocated module is calculated and a jump to that addressis done.  Now the RAM is read back from the file MEMORY/CIMagain.  It is stored beginning at 4000H (rel. record # 4 ff.),because the video RAM still acts as temporary stack.  The DOSoverlay-area is skipped again.  When FFFFH is reached, the FCBis repositioned to record # 0, the old stack is restored, andthe original screen contents are read back (4 records = 1024bytes).     Then the FCB is positioned to EOF and the file is closed(this is necessary because the original contents of the FCB(line 3120) are needed for subsequent "shift-clear's".     Finally, all registers are restored.  Now, everything isfixed up except for the overlay-area.  We now simply plug 0into address 4317H to make DOS believe, it had loaded overlay #0 (which would be BOOT/SYS).  When the next overlay requestwill follow DOS will take care of that by itself, so we won'thave to bother with it.     Finally, we jump to SCALL which was the address where weinterrupted our running program (this fixes the programcounter).     Please, do read the comments at the beginning of thelisting before using this program!_!##�M�B#>N�F(#>Y�kN���#,##~��8�(�>�kN��>#2 O##O��#_!�B#�p(#}�1oV+^���N>/�kN�!#O��c@�##~�A+0

##~�0 ##�###~#�kN#�#����3#���̈́N##͢Nz�{�>/�	D�ͧN~�A�#0�##�ͩN~�H# ��H��#��#�N�#####~�0�

8

�@��#�#��

�bkO���)�)��@(#)###�)�Y###����##�###!#O�N#F#�>/<	8��B��0 #

(###�N

�kN#�{�0�kN��#��00000H�`##B##�####��p`xhXHP�@#������(##q#���yiY�a9Q)#	��A��z�����������������:mC�g>7(b�sN2#P��L�##P�6D�!#B~� H#~�� B#n>#�͒L:#C͔L���#"#P��O:#B�

g��##!�O��O�!#P��O��"#P��O"#P�<D�##��>,ÁN##P�6D�# ����O:#B�P�P �:#B� �##B##��O!#P###�####�O �#��BOOTDIR     SYS�`##B##�####��#############################################################################################################################################################################################################################��P###################################################################################################################################################################################################################################################M 79, 80, 32, 66#��##�  73, 84, 44, 32, 50, 32, 70, 79, 82, 32#��!#�  50, 32, 83, 84, 79, 80, 83, 32, 61, 32#߃##�   0, 66, 32, 61, 32, 66, 85, 70, 70, 69#�%#�  82, 32, 79, 78, 45, 79, 70, 70, 32, 84#;�'#�  79, 71, 71, 76, 69, 13, 67, 32, 61, 32#i�)#�  67, 76, 69, 65, 82, 32, 66, 85, 70, 70#��+#�  69, 82, 32, 40, 83, 72, 79, 85, 76, 68#ń-#�  32, 66, 69, 32, 85, 83, 69, 68, 32, 66#�/#�  69, 70, 79, 82, 69, 32, 69, 65, 67, 72#!�1#�  32, 68, 79, 87, 78, 76, 79, 65, 68, 41#O�3#�  13, 69, 32, 61, 32, 69, 78, 6