XFERPC2/BAS written by Bernard R. Clark [71455,1311]

October 1983

This program will permit the transfer of a BASIC program

stored in ASCII form and conforming to the PC-2 dialect

from a Model 4 to a PC-2 (or Sharp PC-1500) via the

RS-232 interface and a null modem adapter. This program

will perform the transfer based on the transfer protocol

as stated on pg. 13 of the PC-2 RS-232C owner's manual.

The settings for the RS-232C interfaces can be anything

you wish to use just so long as both interface settings

match.

How it works:

=== == =====

Line 100 defines the variable types. Any variable begin-

ning with "A" will be treated as a string variable and

those with "B" will be treated as integers. Lines 120-130

will cause the program to prompt the operator for the

file to be downloaded. Lines 140-220 prompt the operator

through the command sequence necessary to get the RS-232C

interface of the PC-2 operational. The subroutine at line

390 will allow the system to wait for the operator.

All that is needed is for the operator to press <ENTER>.

At this point, the PC-2 is standing by waiting for program

lines. Lines 230-240 configure the Model 4's RS-232C

interface for the transfer. The file is opened for

sequential input at line 250 followed by line 260 which

alerts the operator of the start of transmission. Line 270

links the video display to the communications line. Anything

that is sent to the display is likewise sent to the RS-232C.

Lines 280-340 represent a loop which can be escaped once

end of file is reached. This loop inputs a line from the

file and sends it to the display (and coincidentally to the

the RS-232C). The program then enters the subroutine at line

400 which samples the seconds portion of TIME$. Lines 310-

330 consist of another loop nested within that checks for

an elapse time of 2 seconds before proceeding on. Once the

file has been completely transferred the link between is

broken between the display and the communications line at

line 350. This is subsequently followed by the closing of

the disk file and furthermore by the message "Transmission

Complete".

PLEASE NOTE: Certain reserve words may be construed if

care is not taken in the alteration of a PC-2 program

upon transmission. For example:

10 IF H>G PRINT "loss: ": GOTO 20

becomes

10 IF H>GPRINT "loss: ": GOTO 20

20 IF D=A READ F$

becomes

20 IF D=AREAD F$

The above examples would result in a runtime error and

would never get picked up during the transfer. Also be

certain that you have enough memory to hold the program.

It's easy enough to run out of memory with an oversized

program.

Notes on uploading:

===== == =========

If you are uploading a program  from the PC-2 to the Model 4,

you have two options available:

1. transfer via CSAVEa, using the same command sequence

as outlined in this program for the download but not

using CLOADa. This will result in a transfer of each

program line as ASCII text at 2 second intervals.

This method is really only necessary for transfer

from one PC-2 to another.

2. transfer via LLIST. This method will allow a transfer

of program lines without the 2 second delay per line.

To implement this method, use the following command

sequence:

SETCOM 1200,8,E,1

SETDEV PO

OUTSTAT 0

LLIST

Please note that you do NOT need a printer/cassette

interface to perform this type of transfer. If you do

have one attached, nothing will be printed as this

has the same effect as ROUTE *PR *CL on the Model 4.

In either case, the text comes over as the same. In addition,

you will need a smart terminal program with buffer capturing

capability and preferably with the means to alter the RS-232C

settings. One such program is Videotex Plus. The different

transfer rates (i.e. 2400 baud in method #1; 1200 baud in method

#2) are based on actual use with a Model 4 running Videotex Plus.

For some reason, Videotex loses characters running at 2400 baud

using method #2 and therefore that is why the "slower" baud rate

is used. Inspite of this, the transfer is still relatively faster

than method #1.